| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0410 |
| --- | --- |

| Title: | Simple NAL Unit Header for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jung Won Kang, Hahyun Lee, Jin Soo Choi, 218, Gajeong-no, Yuseong, Daejeon, Korea Truong Cong Thang Tsuruga, Ikki-Machi, Aizu-Wakamatsu, Japan 965-8580 | Tel: Email: | +82-42-860-5137 {jungwon, hanilee, jschoi} @ etri.re.kr thang@u-aizu.ac.jp |
| Source: | ETRI, University of Aizu |  |  |

_____________________________

Abstract

NAL unit header is crucial for a network node to process packet by packet. This contribution proposes some simple options for NAL unit header that can support any extensions of HEVC. The main objective of the proposed options is to have self-contained packet header so as to facilitate simple adaptation at the packet level.

# Introduction

The NAL unit header, including contained information of scalability, is crucial for any node in a content delivery path to modify the bitstream. Especially, the nodes inside a network should be able to process packet by packet efficiently without having to store much status information of the related bitstreams.

In this contribution, we propose some options/methods to convey the scalability information. The goals of the following methods are 1) to provide flexible mechanisms to describe the scalability information of different kinds of bitstream and 2) to enable efficient adaptation at the packet level. Especially, for the second goal, NAL unit header should have self-contained packet to facilitate the fast processing of each NAL unit at any network node.

# Proposed Options for simple NAL header

## Option using Flags

In this option, a number of flags are used to signal the presence/absence of the fields (or elements) corresponding to the basic scalability dimensions. The flags and corresponding fields are shown in the following syntax table. Among the reserved bits of this syntax, some bits could be used as flags to extend the header (e.g. if a given flag is 1, some new bytes would be appended to the existing NAL unit header).

| nal_unit( NumBytesInNALunit ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_ref_flag | u(1) |
| nal_unit_type | u(6) |
| NumBytesInRBSP = 0 |  |
| nalUnitHeaderBytes = 1 |  |
| scalability_info_flag | u(1) |
| if( scalability_info_flag = = 1 ) { |  |
| priority_id_flag | u(1) |
| dependency_id _flag | u(1) |
| temporal_id _flag | u(1) |
| quality_id _flag | u(1) |
| view_id _flag | u(1) |
| reserved_one_2bits | u(2) |
| if( priority_id_flag ) |  |
| priority_id | u(6) |
| if( dependency_id_flag ) |  |
| dependency_id | u(3) |
| if( temporal_id_flag ) |  |
| temporal_id | u(3) |
| if( quality_id_flag ) |  |
| quality_id | u(4) |
| if( view_id_flag ) { |  |
| long_view_id_flag | u(1) |
| view_id_part1 | u(2) |
| if( long_view_id_flag ) |  |
| view_id_part2 | u(8) |
| } |  |
| reserved_bits | u(v) |
| ...... |  |
| } |  |

The semantics of the above syntax elements is defined as follows:

- scalability_info_flag: if equal to 1, indicates that the scalability information is available in this NAL unit.

- priority_id_flag: if equal to 1, indicates that the parameter priority_id is available in this NAL unit.

- dependency_id_flag: if equal to 1, indicates that the parameter priority_id is available in this NAL unit.

- temporal_id_flag: if equal to 1, indicates that the parameter priority_id is available in this NAL unit.

- quality_id_flag: if equal to 1, indicates that the parameter priority_id is available in this NAL unit.

- view_id_flag: if equal to 1, indicates that the parameter priority_id is available in this NAL unit.

- priority_id: this parameter is the same as the priority_id of SVC specification.

- dependency_id: this parameter is the same as the dependency_id of SVC specification.

- temporal_id: this parameter is the same as the temporal_id of SVC specification.

- quality_id: this parameter is the same as the quality_id of SVC specification.

- long_view_id_flag: if equal to 1, indicates that the parameter view_id_part2 is available in this NAL unit.

- view_id_part1: this 2bit-long parameter indicates the view identifiers of a bitstream

- view_id_part2: when long_view_id_flag is equal to 1, this 8bit-long parameter is merged with view_id_part1 to have a 10bit-long parameter to indicate a large number of views of a bitstream.

The reserved_bits element after the view_id_part1/view_id_part2 is to store some other reserved bits. The length N (N < 8) of this element is defined such that the length of NAL unit header from the forbidden_zero_bit to this element (inclusive) is an integer of bytes.

In the above, the separation of view_id_part1 and view_id_part2 is to support bitstreams that contain small number of views (e.g. 3D video) without having to use a long parameter.

The advantage of this method is its simplicity, while the main disadvantage is the use of many flags and fixed field lengths for the elements. However, this amount of bits could be considered trivial in comparison to the total size of each NAL unit.

## Option using Scalability Identifier

Instead of using flags, an identifier (scalability_identifier) in the NAL unit header is used in this option. Here, this identifier is similar to a payload type; each value of the identifier indicates a specific combination (or profile) of scalability dimensions. Specifically, given an identifier value, certain fields (together with their lengths) shall be present in the NAL unit header to describe the scalability dimensions of that combination/profile. Moreover, one special value of this element could be used to signal that the presence/length information of the fields is provided in a special message/packet of the bitstream.

The elements for this option are shown in the following syntax table.

| nal_unit( NumBytesInNALunit ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_ref_flag | u(1) |
| nal_unit_type | u(6) |
| NumBytesInRBSP = 0 |  |
| nalUnitHeaderBytes = 1 |  |
| scalability_info_flag | u(1) |
| if( scalability_info_flag = = 1 ) { |  |
| scalability_identifier | u(5) |
| reserved_one_2bits | u(2) |
| priority_id | u(v) |
| dependency_id | u(v) |
| temporal_id | u(v) |
| quality_id | u(v) |
| view_id | u(v) |
| other_ids | u(v) |
| } |  |
| reserved_bits | u(v) |
| ...... |  |
| } |  |

The semantics of these elements is defined as follows:

- scalability_info_flag: if equal to 1, indicates that the scalability information is available in this NAL unit.

- scalability_identifier: indicates a scalability profile combinations of scalability dimensions. Each scalability_identifier value corresponds to a specific combination which defines certain fields (together with their lengths) for scalability information of that profile

- id’s for scalability: following the scalability_identifier are the fields or id’s (such asnamely priority_id, dependency_id, temporal_id, quality_id, view_id, etc.). The presence and length of each field is defined by the specific combination identified by a scalability_identifier.

- other_ids: the place holder for any future fields or id’s of a scalability profile.

Semantics of id’s for scalability and reserved_bits is the same as before. However, the number of bits used for each element is now defined by a combination/profile identified by the scalability_identifier. The use of scalability_identifier element allows more flexibility than the previous method. However, the drawback is that the processing node may have to maintain the knowledge of different combinations/profiles of scalability.

Some typical scalability profiles can be defined as follows.

Profile 1: scalability_identifier equal to 1 indicates a scalability profile where only the following elements shall be present as follows:

- The first 6 bits after reserved_one_2bits are used for priority_id, whose semantics is the same as the priority_id of SVC specification.

- The next 3 bits are used for dependency_id, whose semantics is the same as the dependency_id of SVC specification.

- The next 3 bits are used for temporal_id, whose semantics is the same as the temporal_id of SVC specification.

- The next 4 bits are used for quality_id, whose semantics is the same as the quality_id of SVC specification.

- No bits are used for reserved_bits element

Profile 2: scalability_identifier equal to 2 indicates a scalability profile where only the following elements shall be present as follows:

- The first 10 bits after reserved_one_2bits are used for view_id, whose semantics is the same as the view_id of MVC specification.

- The next 6 bits are used for reserved_bits element

Profile 3: scalability_identifier equal to 3 indicates a scalability profile where only the following elements shall be present as follows:

- The first 6 bits after reserved_one_2bits are used for priority_id, whose semantics is the same as the priority_id of SVC specification.

- The next 2 bits are used for reserved_bits element

# Patent rights declaration(s)

ETRI and University of Aizu may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).