| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, 1-10 February, 2012 | Document: JCTVC-H0411 |
| --- | --- |

| Title: | Significant Coefficient Flag Context Models for Modular Processing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nguyen Nguyen, Tianying Ji, Dake He and Gaëlle Martin-Cocher 185 Columbia St. W. Waterloo, ON, Canada N2L 5Z5 | Tel: Email: | +1-519-8887465 nnguyen@rim.com tiji@rim.com dhe@rim.com gmartincocher@rim.com |
| Source: | Research In Motion Ltd. |  |  |

_____________________________

# Abstract

A modified neighbourhood for context derivation of significant coefficient flags in transform units (TUs) of size greater than 8x8 is proposed.  To encode (decode) the significant coefficient flags in a 4x4 coefficient group, the existing neighbourhood requires a 6x6 memory footprint.  The proposed neighbourhood reduces this requirement to a 5x5 memory footprint to better facilitate modular processing.  On average, under common test conditions against HM-5.0 anchor, the proposal yields a BD-rate that is within 0.1%.

# Problem

The HEVC current working draft [2] specifies that in TUs of size greater than 8x8, the context of a significance flag in position x, where x is not the DC position, is determined by the significance flags of its neighbours a, b, c, d and e.  This is illustrated in Figure 1.

|  | x | a | d |  |
| --- | --- | --- | --- | --- |
|  | c | b |  |  |
|  | e |  |  |  |

Figure 1

If any of these neighbours fall outside the boundary of the TU, their significance is assumed to be 0 for the calculation of the context of x.  Certain neighbours may also be excluded depending on the position of x to allow for parallel processing, as done in JCTVC-G323 [5].

In implementations that process large TUs as a set of smaller 4x4 coefficient groups (i.e., sub-blocks) using a modular architecture, the neighbourhood-based context model poses a problem: positions at the right and lower edges of a coefficient group require significance flags outside the current coefficient group to be accessed.  That is, to process a 4x4 coefficient group, in addition to the 16 positions in the current coefficient group, 17 positions in neighbouring coefficient groups must be accessed, loaded into local memory and unloaded from local memory.   This amounts to an overhead of 17/16 > 1.

Figure 2

Figure 2 illustrates the significant coefficient flags required to process a given coefficient group, where the positions highlighted in green are in the current coefficient group and positions highlighted in yellow are in the neighbouring right and lower coefficient groups.  To avoid complexities associated with this irregular shape, it would most likely be processed as a 6x6 block in practice, making the overhead even greater, 20/16. To facilitate modular processing, it is desired to substantially reduce the overhead in local memory footprint.

# Proposal

## Modified Neighborhood Definition

Ideally, each coefficient group could be processed independently of any other coefficient group.  One approach is to remove the outlying neighbours for positions at the right and lower edges of the coefficient group.  However, this would result in significant RD performance loss.  A compromise is proposed.  This proposal modifies the neighbourhood for positions at the right and edges of a coefficient group subject to the constraint that at most one column and one row outside the given coefficient group may be accessed.   This allows large TUs to be processed modularly as overlapping 5x5 tiles as shown below in Figure 3, and reduces the overhead from 17/16 or 20/16 to 9/16.

Figure 3

In the following text, transform coefficients (and their associated significance flags) are identified by their position within a coefficient group.  Reverse sub-block scan order, as described in JCTVC-G323 [5], is used to index the positions.

| 15 | 13 | 10 | 6 |
| --- | --- | --- | --- |
| 14 | 11 | 7 | 3 |
| 12 | 8 | 4 | 1 |
| 9 | 5 | 2 | 0 |

The following describes the proposed neighbourhood definition.

- For position 0 in a coefficient group, the neighbourhood is

|  |  |  |  | e |
| --- | --- | --- | --- | --- |
|  |  |  | x | a |
|  |  | d | c | b |

- For position 1 in a coefficient group, the neighbourhood is

|  |  |  |  | e |
| --- | --- | --- | --- | --- |
|  |  |  | x | a |
|  |  |  |  | b |
|  |  |  | d |  |

- For position 3 in a coefficient group, the neighbourhood is

|  |  |  |  | e |
| --- | --- | --- | --- | --- |
|  |  |  | x | a |
|  |  |  | c | b |
|  |  |  | d |  |

- For position 6 in a coefficient group, the neighbourhood is

|  |  |  | x | a |
| --- | --- | --- | --- | --- |
|  |  |  | c | b |
|  |  |  | d | e |

- For positions 2 and 5 in a coefficient group, the neighbourhood is

|  | x | a | d |  |
| --- | --- | --- | --- | --- |
| e | c | b |  |  |

- For position 9 in a coefficient group, the neighbourhood is

| x | a | d |  |  |
| --- | --- | --- | --- | --- |
| c | b | e |  |  |

- For all other positions in a coefficient group, the neighbourhood remains unchanged from the HEVC current working draft [2].

Note that the proposed neighbourhood satisfies the condition

D >= 2

where D is the difference in scan order between x and any of the neighbours used to derive the context for its significance flag.  This criterion allows for parallel processing since context derivation does not depend on the previously processed position (i.e., the previous position in reverse scan order).  Furthermore, note that the proposed neighbourhood does not necessarily preclude other neighbourhood definitions.  The essence of the proposal is to enable modular processing by restricting the neighbourhood to be outside the current coefficient group by at most one column to the right and one row below.

# Results

In this section, we report experimental results of the proposal (integrated into HM-5.0) against HM-5.0 anchor under common test conditions [4].

Table 3.1 below shows that the BD-rates of the proposal against HM-5.0 anchor (a negative value indicates gain; and a positive value indicates loss).  Note that the encoding time results of the proposal and the HM-5.0 anchor are obtained on the same computing platform; and the decoding time results of the proposal and the HM-5.0 anchor are obtained on the same computer.

Table 3.1: BD-rate vs. HM-5.0

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class B | 0.1% | 0.2% | 0.2% | 0.0% | 0.2% | 0.1% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class E | 0.1% | 0.3% | 0.3% | 0.1% | 0.3% | 0.3% |
| Overall | 0.1% | 0.2% | 0.1% | 0.0% | 0.1% | 0.1% |
|  | 0.1% | 0.2% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class F | 0.0% | 0.0% | 0.2% | 0.0% | 0.2% | -0.1% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.1% | -0.2% | 0.0% | 0.2% | -0.2% | 0.1% | 0.2% | 0.2% |
| Class B | 0.0% | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% | 0.1% | 0.1% | 0.0% |
| Class C | 0.0% | -0.1% | 0.1% | 0.0% | 0.2% | 0.0% |  |  |  |
| Class D | 0.0% | -0.3% | 0.0% | 0.0% | 0.5% | -0.4% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | -0.1% | 0.1% | 0.1% | 0.1% |
|  | 0.0% | -0.1% | 0.1% | 0.0% | 0.2% | -0.1% | 0.1% | 0.2% | 0.1% |
| Class F | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.4% | 0.0% | -0.1% | 0.3% |
| Class C | 0.0% | -0.1% | 0.2% | 0.0% | 0.2% | 0.0% |
| Class D | 0.0% | -0.1% | 0.4% | 0.0% | 0.7% | 0.2% |
| Class E | 0.0% | 0.6% | -0.5% | 0.1% | -0.5% | 0.0% |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | 0.1% |
| Class F | -0.1% | -0.1% | -0.4% | 0.2% | 0.5% | -0.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

Note that decoding timing results will be updated shortly.

# Conclusion

Significant coefficient flag coding as specified by the HEVC current working draft [2] does not lend itself to modular implementations because of the large overhead: 17/16 or 20/16.  That is, to encode (decode) 16 significant coefficient flags, a total of 33 or 36 significant coefficient flags must be accessed, loaded into memory and unloaded from memory.  By comparison, this proposal decreases the overhead to 9/16, allowing for modular processing, while having minimal impact on coding efficiency.

# Change to the HEVC text spec.

## Proposed changes to “9.3.3.1.1.4 Derivation process of ctxIdxInc for the syntax element significant_coeff_flag”

[Note: preceding text is omitted because it is not affected]

- Otherwise (xC  +  yC is greater than 0), sigCtx is derived using previously decoded bins of the syntax element significant_coeff_flag as follows.

- The variable sigCtx is initialized as follows.

sigCtx  =  0		(9-1)

- The variable bottomRow is set to true if yC % 4 = 3 and false otherwise.

- The variable rightCol is set to true if xC % 4 = 3 and false otherwise.

- When xC is less than ( 1 << log2TrafoWidth ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC ]	(9-2)

- When xC is less than ( 1 << log2TrafoWidth  ) − 1, and yC is less than ( 1 << log2TrafoHeight ) – 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]	(9-3)

- When xC is less than ( 1 << log2TrafoHeight  ) – 2 and rightCol is false, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 2 ][ yC ]	(9-4)

- When all of the following conditions are true,

  - yC is less than ( 1 << log2TrafoHeight ) − 1,

  - xC % 4 is not equal to 0 or yC % 4 is not equal to 0,

  - xC % 4 is not equal to 3 or yC % 4 is not equal to 2,

the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 1 ]

- When yC is less than ( 1 << log2TrafoHeight ) – 2, and sigCtx is less than 4, and bottomRow is false, the following applies.

# sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]

- When rightCol is true, yC % 4 > 0, xC is less than ( 1 << log2TrafoSize ) – 1, and sigCtx is less than 4, the following applies.

# sigCtx  =  sigCtx  +  significant_coeff_flag[ xC  + 1][ yC - 1 ]

- When rightCol is true, yC % 4 = 0, xC is less than ( 1 << log2TrafoSize ) – 1, and sigCtx is less than 4, the following applies.

# sigCtx  =  sigCtx  +  significant_coeff_flag[ xC  + 1][ yC + 2 ]

- When bottomRow is true, xC % 4 > 0, yC is less than ( 1 << log2TrafoSize ) − 1, and sigCtx is less than 4, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC - 1 ][ yC + 1 ]

- When bottomRow is true, xC % 4 = 0, yC is less than ( 1 << log2TrafoSize ) – 1, and sigCtx is less than 43, the following applies.

# sigCtx  =  sigCtx  +  significant_coeff_flag[ xC  + 2][ yC + 1 ]

[Note: succeeding text is omitted because it is not affected]

# References

[1] D. Marpe, H. Schwarz, and T. Wiegand, “Context-based adaptive binary arithmetic coding in the h.264/AVC video compression standard,” IEEE Transactions on Circuits and Systems for Video Technology, 13(7):620–636, Jul. 2003.

[2] B. Bross, W-J Han, J-R Ohm, G. J. Sullivan, and T. Wiegand, “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCTVC-G1103.

[3] T. Nguyen, T. Winken, D. Marpe et al., “Reduced-complexity entropy coding of transform coefficient levels using a combination of VLC and PIPE”, JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 4th Meeting, Daegu, January 2011.

[4] F. Bossen, “Common test conditions and software reference configurations,” JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 6th Meeting, Torino, July 2011.

[5] J. Sole, R. Joshi, and M. Karczewicz, “Non-CE11: Diagonal sub-block scan for HE residual coding,” JCTVC-G323.

# Patent rights declaration(s)

Research In Motion Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).