| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0415_r2 WG11 Number: m23291 |
| --- | --- |

| Title: | Non-CE8: ALF BA feature computation only accessing 3x3 area |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wang (PoLin) Lai Felix C. A. Fernandes Ehsan Maani Ali Tabatabai Masaru Ikeda | Tel: Email: | wlai@sta.samsung.com +1 972 761 7456 ffernandes@sta.samsung.com +1 972 761 7427 ehsan.maani@am.sony.com ali.tabatabai@am.sony.com +1-408-352-4710 Masaru.Ikeda@jp.sony.com +81-3-5435-3308 |
| Source: | Samsung Electronics Co., Ltd. Sony Electronics Inc., Sony Corporation. |  |  |

_____________________________

# Abstract

This contribution presents changes in ALF block-based adaptation mode (BA-mode), in which for a 4×4 block, the directional features are computed only using the upper-left 3×3 pixel area. Within this area, 3 horizontal and 3 vertical feature computations are performed (3H and 3V), reduced from computing 4H and 4V in HM5.0. The modification facilitates: 1. The computation of BA features and the immediate subsequent filtering operation to be performed with the same accessing area, even when the smaller sized snowflake(star)-5×5 filter is used. 2. No additional line memory since all computation is within the current 4×4 block.

The BD-rates results as compared to anchor HM5.0 are (in HE configuration): AI: 0.1%, RA: 0.1%, RA_10: 0.2%, LD: 0.1%, and LDB: 0.2%. The run-time measurements do not have noticeable changes.

# Introduction

In High-Efficiency Video Coding test model version 5.0 (HM5.0), ALF block-based adaptation mode (BA-mode) classifies a 4×4 block by computing directional features only at center 4 pixels [1]. The merit is keeping the same number of operations with features computed only at subset of pixels [2], while also achieving reduced access area from 5×5 [2] to 4×4.

It was reported [3] at the 7th JCTVC meeting that, for a 4×4 block, when snowflake(star)-5×5 filter is used, BA feature computation needs to access pixels outside the area required for filtering the first pixel. The left side of Figure 1 demonstrates the access areas for BA feature computation and for filtering the first pixel with snowflake(star)-5×5 filter, in current HM5.0. As can be seen, additional pixels need to be accessed outside the green-dashed area. A method was presented to restrain the BA computation within the green-dashed area [3], with irregular feature pattern and 0.1% BD-rate losses for AI, RA, LD, LDP (HE 10 bits) configurations.

This contribution follows the same merit to modify the BA feature computation, such that pixels used in computing directional features are all within the required area for filtering the first pixel. The feature computation also does not access any pixel outside the current 4×4. Furthermore, the computation of directional feature is also reduced in this proposal.

# BA computation only accessing 3x3 area

In this proposal, feature computation for ALF BA-mode in a 4×4 block is modified as illustrated in the figure below:

- In the upper-left 3×3 pixel area, 3 horizontal and 3 vertical feature computations are performed (3H and 3V), which is reduced from computing 4H and 4V in HM5.0.

- Pixels used in BA feature computation are all within the required area for filtering the first pixel even when the smaller sized snowflake(star)-5×5 filter is used.

- No additional line memory is required since all pixels involved in BA feature computation are within the current 4×4 block.

# Experimental results

The BD-rates results as compared to anchor HM5.0 are (in HE configuration):

JCTVC-H0415 ---- AI: 0.1%, RA: 0.1%, RA_10: 0.2%, LD: 0.1%, and LDB: 0.2%.

As reference, the results of JCTVC-H413 [4], which computes 4H and 4V using upper-left 4x4 area, are:

JCTVC-H0413 ---- AI: 0.0%, RA: 0.1%, RA_10: 0.2%, LD: 0.1%, and LDB: 0.1%.

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | 0.2% | 0.0% | 0.0% |
| Class B | 0.1% | 0.0% | 0.0% |
| Class C | 0.1% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.2% | 0.0% | 0.0% |
| Overall | 0.1% | 0.0% | 0.0% |
|  | 0.1% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A (8bit) | 0.4% | 0.1% | -0.1% |
| Class B | 0.1% | 0.0% | 0.0% |
| Class C | 0.1% | 0.0% | 0.0% |
| Class D | 0.1% | -0.1% | -0.2% |
| Class E |  |  |  |
| Overall | 0.1% | 0.0% | -0.1% |
|  | 0.1% | 0.0% | -0.1% |
| Class F | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.1% | 0.2% | 0.5% |
| Class C | 0.1% | 0.0% | 0.1% |
| Class D | 0.1% | 0.1% | 0.0% |
| Class E | 0.1% | -0.2% | -0.9% |
| Overall | 0.1% | 0.0% | 0.0% |
|  | 0.1% | 0.1% | -0.1% |
| Class F | -0.1% | 0.1% | -0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.2% | 0.1% | -0.1% |
| Class C | 0.2% | 0.1% | -0.1% |
| Class D | 0.1% | -0.2% | -0.3% |
| Class E | 0.3% | 0.0% | 0.0% |
| Overall | 0.2% | 0.0% | -0.1% |
|  | 0.2% | 0.0% | -0.2% |
| Class F | 0.0% | 0.3% | 0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

|  | Random Access HE-10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.3% | 0.1% | 0.0% |
| Class B | 0.2% | 0.0% | 0.1% |
| Class C |  |  |  |
| Class D |  |  |  |
| Class E |  |  |  |
| Overall | 0.2% | 0.0% | 0.0% |
|  | 0.2% | 0.0% | 0.0% |
| Class F |  |  |  |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |

# Changes in Working Draft Text

- 8.6.3.3	Derivation process for filter index array for luma samples

- ......

- ......

- Otherwise (alf_region_flag is equal to 0), the following ordered steps apply.

- The variables varTempH[ x ][ y ], varTempV[ x ][ y ] and varTemp1[ x ][ y ] with x, y = 0, 2, 4,...(nS) is derived as

varTempH[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x-1, yC+y ] – s’’[ xC+x+1, yC+y ] |	(8-1)
	varTempV[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x, yC+y-1 ] – s’’[ xC+x, yC+y+1 ] |	(8-2)

- The variables varTempH1[ x, y ], varTempV1[ x, y ], and varTemp3[ x, y ] with x, y = 0..( (nS) – 1 )>>2 are derived as

varTempH1[ x ][ y ] = ij varTempH[ (x << 2 ) + i ][ (y << 2) + j ] with (i, j) = (0,1),(1,1),(2,1)	(8-3)
	varTempV1[ x ][ y ] = ij varTempV[ (x << 2 ) + i ][ (y << 2) + j ] with (i, j) =(1,0),(1,1),(1,2) 	(8-4)
	varTemp3[ x ][ y ] = ( varTempH1[ x ][ y ] + varTempV1[ x ][ y ] ) >> 2	(8-5)

# Patent rights declarations

Samsung may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “CE8 Subtest a, Tool 2: Directional feature calculation on subset of pixels,” JCTVC-G609, Geneva, Switzerland, Nov. 2011

- “CE8 Subtest 1: Block-based filter adaptation with features on subset of pixels,” JCTVC-F301, Torino, Italy, Jul. 2011

- “Modification of ALF classification,” JCTVC-G351, Geneva, Switzerland, Nov. 2011

- “CE8.b.1: ALF BA feature computation at upper-left 4 pixels in a 4×4 block,” JCTVC-H0413, San José, CA, USA, Feb. 2012