| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0416_r1 WG11 Number: m23292 |
| --- | --- |

| Title: | CE8.b.2 Crosscheck of JCTVC-H0068 ALF with single filter type |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Wang (PoLin) Lai Felix C. A. Fernandes | Tel: Email: | wlai@sta.samsung.com +1 972 761 7456 ffernandes@sta.samsung.com +1 972 761 7427 |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This report presents Samsung’s cross-check results for CE8.b.2 on ALF with single filter type (JCTVC-H0068). Five filter types with complete Excel results in JCTVC-H0068 were cross-checked. Rate-PSNR numbers match exactly to the proponent’s results. As for software study: (1) The design of the checked 10-coefficient shapes and 11-coefficient shape all conform to the no-ALF line buffer principle in the previously adopted JCTVC-G212. (2) The cross-checker recognized changes that could be made to the k-index mapping tables in two of the shapes, such that the design of k-index mapping can be consistent with HM5.0, and the cross-checker provide such changes and the corresponding results (no significant differences in coding results are reported with such changes).

# Cross-check results

Five filter types with complete Excel results in JCTVC-H0068 CE8.b.2 on ALF with single filter type [1] are cross-checked. The corresponding shapes are depicted below (figures from [1]). The simulations were run in the 64-bit Linux cluster environment.

Summary of the results are shown in the tables below and more detailed results can be found in the accompanying Excel sheet. Yellow numbers indicates losses greater than 0.5%, Class-wise or average over all classes.

|  | filter taps |  | AI-HE | RA-HE | RA-HE10 | LB-HE | LP-HE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| HM-5.0 star-5x5 only | 9 | Class A | 0.0% | 0.1% | 0.3% |  |  |
|  |  | Class B | 0.2% | 0.2% | 0.2% | 0.3% | 0.3% |
|  |  | Class C | 0.2% | 0.4% |  | 0.6% | 0.3% |
|  |  | Class D | 0.3% | 0.9% |  | 0.7% | 0.1% |
|  |  | Class E | 0.1% |  |  | 0.5% | 0.6% |
|  |  | Average | 0.2% | 0.4% | 0.3% | 0.5% | 0.3% |
|  |  | Enc Time | 95% | 100% | 99% | 99% | 100% |
|  |  | Dec Time | 99% | 101% | 101% | 100% | 103% |

|  | filter taps |  | AI-HE | RA-HE | RA-HE10 | LB-HE | LP-HE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| HM-5.0 cross-9x9 only | 9 | Class A | 0.1% | 0.4% | 0.4% |  |  |
|  |  | Class B | 0.1% | 0.6% | 0.6% | 0.5% | 0.9% |
|  |  | Class C | 0.0% | 0.2% |  | 0.2% | 0.5% |
|  |  | Class D | 0.0% | 0.1% |  | 0.1% | 0.1% |
|  |  | Class E | 0.0% |  |  | 0.7% | 0.9% |
|  |  | Average | 0.1% | 0.3% | 0.5% | 0.4% | 0.6% |
|  |  | Enc Time | 97% | 100% | 100% | 100% | 99% |
|  |  | Dec Time | 100% | 102% | 102% | 101% | 100% |

|  | filter taps |  | AI-HE | RA-HE | RA-HE10 | LB-HE | LP-HE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 9x7 cross, center 3x3 square | 10 | Class A | 0.0% | 0.1% | 0.1% |  |  |
|  |  | Class B | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
|  |  | Class C | -0.1% | 0.0% |  | 0.1% | 0.1% |
|  |  | Class D | 0.0% | 0.2% |  | 0.2% | 0.1% |
|  |  | Class E | -0.2% |  |  | 0.3% | 0.5% |
|  |  | Average | -0.1% | 0.1% | 0.1% | 0.2% | 0.2% |
|  |  | Enc Time | 103% | 101% | 101% | 101% | 101% |
|  |  | Dec Time | 101% | 103% | 103% | 102% | 103% |

|  | filter taps |  | AI-HE | RA-HE | RA-HE10 | LB-HE | LP-HE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7x5 cross, center 5x5 star (Asymmetric star-7x5) | 10 | Class A | 0.0% | 0.0% | 0.1% |  |  |
|  |  | Class B | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
|  |  | Class C | 0.0% | 0.2% |  | 0.3% | 0.1% |
|  |  | Class D | 0.2% | 0.6% |  | 0.6% | 0.1% |
|  |  | Class E | -0.1% |  |  | 0.2% | 0.3% |
|  |  | Average | 0.0% | 0.2% | 0.0% | 0.3% | 0.1% |
|  |  | Enc Time | 102% | 101% | 101% | 100% | 101% |
|  |  | Dec Time | 100% | 101% | 101% | 100% | 100% |

|  | filter taps |  | AI-HE | RA-HE | RA-HE10 | LB-HE | LP-HE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7x5 cross, center 5x5 star (Circular star-7x7) | 11 | Class A | -0.1% | -0.1% | 0.0% |  |  |
|  |  | Class B | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
|  |  | Class C | 0.0% | 0.1% |  | 0.2% | 0.1% |
|  |  | Class D | 0.1% | 0.4% |  | 0.3% | 0.0% |
|  |  | Class E | -0.2% |  |  | 0.0% | 0.1% |
|  |  | Average | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |
|  |  | Enc Time | 108% | 102% | 102% | 101% | 102% |
|  |  | Dec Time | 100% | 102% | 104% | 102% | 102% |

# Software study

- The two 10 coefficients shapes design conform to the no-ALF line buffer principle in the adopted JCTVC-G212 [2].

- For “asymmetric star-7x5” and “circular star-7x7”, since the shapes cover smaller areas, to make the design consistent with HM5.0 star-5x5 and cross-9x9, the corresponding k-index mapping for k-th order exponential Golomb code for ALF coefficients in JCTVC-H0068 software could also be changed as follows, with difference in coding results highlighted in red:

| // Asymmetric star-7x5 by H0068 Int depthIntShape1Sym[ALF_MAX_NUM_COEF+1] = { 6, 7, 6, 7, 8, 7, 6, 7, 8, 9, }; | // Asymmetric star-7x5 by cross-checker Int depthIntShape1Sym[ALF_MAX_NUM_COEF+1] = { 3, 5, 3, 5, 6, 5, 4, 5, 6, 7, }; |
| --- | --- |

|  | filter taps |  | AI-HE | RA-HE | RA-HE10 | LB-HE | LP-HE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7x5 cross, center 5x5 star (Asymmetric star-7x5) | 10 | Class A | 0.0% | 0.0% | 0.1% |  |  |
|  |  | Class B | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
|  |  | Class C | 0.0% | 0.2% |  | 0.4% | 0.1% |
|  |  | Class D | 0.2% | 0.6% |  | 0.6% | 0.0% |
|  |  | Class E | -0.1% |  |  | 0.2% | 0.3% |
|  |  | Average | 0.0% | 0.2% | 0.0% | 0.3% | 0.1% |
|  |  | Enc Time | 102% | 101% | 101% | 100% | 101% |
|  |  | Dec Time | 100% | 101% | 101% | 100% | 100% |

| // Circular star-7x7 by H0068 Int depthIntShape1Sym[ALF_MAX_NUM_COEF+1] = { 6, 6, 7, 6, 7, 8, 7, 6, 7, 8, 9, }; |  |  |  |  | // Circular star-7x7 by cross-checker Int depthIntShape1Sym[ALF_MAX_NUM_COEF+1] = { 4, 3, 5, 3, 5, 6, 5, 4, 5, 6, 7, }; |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | filter taps |  | AI-HE |  | RA-HE | RA-HE10 | LB-HE | LP-HE |  |
|  | 7x5 cross, center 5x5 star (Circular star-7x7) | 11 | Class A | -0.1% |  | -0.1% | 0.0% |  |  |  |
|  |  |  | Class B | 0.0% |  | -0.1% | 0.0% | 0.0% | -0.1% |  |
|  |  |  | Class C | 0.0% |  | 0.1% |  | 0.2% | 0.1% |  |
|  |  |  | Class D | 0.1% |  | 0.4% |  | 0.3% | 0.0% |  |
|  |  |  | Class E | -0.2% |  |  |  | 0.1% | 0.1% |  |
|  |  |  | Average | 0.0% |  | 0.1% | 0.0% | 0.2% | 0.0% |  |
|  |  |  | Enc Time | 108% |  | 102% | 102% | 101% | 102% |  |
|  |  |  | Dec Time | 100% |  | 102% | 104% | 102% | 102% |  |

# Patent rights declarations

Samsung may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “CE8.b: ALF with single filter type,” JCTVC-H0068, San José, CA, USA, Feb. 2012

- “Non-CE8.c.7: Single-source SAO and ALF virtual boundary processing with cross9x9,” JCTVC-G212, Geneva, Switzerland, Nov. 2011