| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0417_r3 WG11 Number: m23293 |
| --- | --- |

| Title: | Non-CE8: Supplementary analysis of ALF with single shape |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wang (PoLin) Lai Felix C. A. Fernandes | Tel: Email: | wlai@sta.samsung.com +1 972 761 7456 ffernandes@sta.samsung.com +1 972 761 7427 |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution presents supplementary analysis of ALF with single shape. Comparisons were made between (a) Gains between ALF on/off, in HM5 two-shape condition; and (b) Gains between ALF on/off, when there is only one shape. Red Flag cases, defined as losing more than 0.5% BD-rate gain going from (a) to (b), while that loss of 0.5% BD-rate accounts for more than 1/10 of the original gain (a) achievable by two shapes, are highlighted. Thus the Red flags represent cases where the losses are significant in both absolute and relative senses.

Looking beyond the average BD-rate losses, the results report: (1) There are several Red Flag cases when using only snowflake(star)-5x5 and when using only cross-9x9, but they have very rare overlaps, i.e. complementary. (2) For the two 10-tap shapes, “asymmetric snowflake(star)-7x5” produces Red Flag cases on 4 sequences, especially larger losses for sequence with zoom-in effect (PartyScene) and sequence with combined zoom-out + vertical-panning effect (BQSquare); while “asymmetric cross 9x7 with 3x3 square center” has Red Flag cases in BQSquare and video conferencing sequence. (3) Finally the 11-tap shape “snowflake(star)-7x7” has Red Flag cases in BQSquare.

# Introduction

In High-Efficiency Video Coding HM5.0, ALF has two shapes: Snowflake(star)-5x5 and cross-9x9, both have only nine coefficients [1][2]. In the 8th JCTVC meeting, ALF using single shape are reported [3]. This contribution provides more analysis for individual sequences under different coding structures, where cases with large degree of losses by using single shapes are highlighted.

# Analysis of single shape ALF: Red Flag cases

The goal of this analysis is to identify, on a per-sequence basis, cases where significant losses occur when changing ALF from two shapes to one. Analysis was performed by conducting the following steps to investigate how ALF efficiency is affected:

- Record BD-rate gains between ALF on/off, in HM5 two-shape condition.

- Record BD-rate gains between ALF on/off, when there is only one shape.

- Compare the gains going from two shape to one, how much drops.

Significant losses are identified as “Red Flag cases”, which meet the following two conditions:

- The change in BD-Rate gain, from two shapes to one shape, has to drop more than 0.5% BD-Rate.

- That at least 0.5% BD-Rate gain drop, has to accounts for more than 1/10 of the original gain achievable by two shapes.

By doing so, only losses that are significant in both absolute and relative senses will be marked as Red Flag cases. (**For example, for a particular sequence and say in Random Access structure, as compared to no ALF, it has 10% gain when using ALF with two shapes, while 9.2% gain when using ALF with only one shape. Then this loss is considered NOT significant, since it only account for 0.8/10 = 8%, which is less than 1/10 of the original achievable gain. Thus this will NOT result in a Red Flag case.)

Full results can be found in the accompanying Excel sheet [JCTVC-H0417-OneShapeRedFlag_r2.xls]. Here the Red Flag cases are listed, and high level summary are provided:

| Star-5x5 (9 tap) | AI |  |  |  | RA |  |  |  | LD |  |  |  | LDP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2 Shp Gain | 1 Shp Gain | Ratio |  | 2 Shp Gain | 1 Shp Gain | Ratio |  | 2 Shp Gain | 1 Shp Gain | Ratio |  | 2 Shp Gain | 1 Shp Gain | Ratio |
| Kimono | -2.5% | -2.1% | 86% |  | -2.7% | -2.2% | 81% |  | -2.6% | -2.2% | 87% |  | -6.2% | -5.7% | 92% |
| PartyScene | -0.3% | -0.1% | 43% |  | -4.1% | -2.9% | 72% |  | -2.8% | -1.4% | 51% |  | -0.7% | -0.4% | 60% |
| RaceHorses | -1.9% | -1.1% | 55% |  | -1.6% | -1.1% | 68% |  | -1.6% | -1.1% | 68% |  | -2.9% | -2.3% | 79% |
| BQSquare | -0.2% | -0.1% | 65% |  | -9.4% | -6.7% | 72% |  | -6.1% | -3.8% | 62% |  | -1.0% | -0.7% | 76% |
| RaceHorses(D) | -2.0% | -1.3% | 62% |  | -0.8% | -0.5% | 58% |  | -0.6% | -0.3% | 43% |  | -1.1% | -0.7% | 65% |
| Vidyo3 | -2.7% | -2.6% | 94% |  |  |  |  |  | -5.0% | -4.3% | 87% |  | -10.8% | -9.9% | 92% |
| BasketDrillText | -1.1% | -1.1% | 104% |  | -1.8% | -1.7% | 93% |  | -4.0% | -3.3% | 81% |  | -2.3% | -2.4% | 102% |
| Cross-9x9 (9 tap) | I |  |  |  | RA |  |  |  | LD |  |  |  | LDP |  |  |
|  | 2 Shp Gain | 1 Shp Gain |  |  | 2 Shp Gain | 1 Shp Gain | Ratio |  | 2 Shp Gain | 1 Shp Gain | Ratio |  | 2 Shp Gain | 1 Shp Gain | Ratio |
|  |  |  | Ratio |  |  |  |  |  |  |  |  |  |  |  |  |
| Traffic | -2.9% | -2.8% | 95% |  | -3.2% | -2.6% | 82% |  |  |  |  |  |  |  |  |
| BasketballDrive | -1.0% | -0.8% | 82% |  | -2.8% | -2.3% | 82% |  | -3.0% | -2.5% | 82% |  | -6.2% | -5.3% | 87% |
| BQTerrace | -1.2% | -0.8% | 71% |  | -6.5% | -4.6% | 71% |  | -5.5% | -4.1% | 74% |  | -10.8% | -7.9% | 74% |
| BasketballDrill | -1.2% | -1.2% | 96% |  | -1.7% | -1.3% | 74% |  | -4.0% | -3.5% | 87% |  | -3.5% | -2.1% | 61% |
| BQMall | -0.9% | -0.8% | 93% |  | -1.7% | -1.5% | 86% |  | -1.9% | -1.7% | 88% |  | -2.9% | -2.4% | 83% |
| Vidyo1 | -2.2% | -2.2% | 98% |  |  |  |  |  | -2.1% | -1.7% | 81% |  | -4.0% | -3.4% | 84% |
| Vidyo3 | -2.7% | -2.7% | 100% |  |  |  |  |  | -5.0% | -3.6% | 73% |  | -10.8% | -9.1% | 84% |
| BasketDrillText | -1.1% | -0.7% | 68% |  | -1.8% | -1.4% | 79% |  | -4.0% | -3.7% | 91% |  | -2.3% | -1.6% | 70% |
| SlideShow | -1.2% | -1.2% | 96% |  | -1.6% | -1.5% | 96% |  | -2.7% | -2.6% | 96% |  | -3.2% | -2.7% | 85% |
| Asymmetric cross9x7 + 3x3 square center | I |  |  |  | RA |  |  |  | LD |  |  |  | LDP |  |  |
|  | 2 Shp Gain | 1 Shp Gain | Ratio |  | 2 Shp Gain | 1 Shp Gain | Ratio |  | 2 Shp Gain | 1 Shp Gain | Ratio |  | 2 Shp | 1 Shp | Ratio |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Gain | Gain |  |
| BQSquare | -0.2% | -0.2% | 121% |  | -9.4% | -8.9% | 95% |  | -6.1% | -5.5% | 90% |  | -1.0% | -1.0% | 103% |
| Vidyo1 | -2.2% | -2.4% | 108% |  |  |  |  |  | -2.1% | -1.8% | 85% |  | -4.0% | -3.5% | 88% |
| Asymmetric star-7x5 (10 tap) | I |  |  |  | RA |  |  |  | LD |  |  |  | LDP |  |  |
|  | 2 Shp Gain | 1 Shp Gain |  |  | 2 Shp Gain | 1 Shp Gain | Ratio |  | 2 Shp Gain | 1 Shp Gain | Ratio |  | 2 Shp Gain | 1 Shp Gain | Ratio |
|  |  |  | Ratio |  |  |  |  |  |  |  |  |  |  |  |  |
| PartyScene | -0.3% | -0.2% | 72% |  | -4.1% | -3.5% | 85% |  | -2.8% | -2.0% | 69% |  | -0.7% | -0.6% | 88% |
| BQSquare | -0.2% | -0.1% | 84% |  | -9.4% | -7.5% | 80% |  | -6.1% | -4.3% | 71% |  | -1.0% | -0.8% | 86% |
| RaceHorses(D) | -2.0% | -1.4% | 70% |  | -0.8% | -0.5% | 60% |  | -0.6% | -0.3% | 41% |  | -1.1% | -0.8% | 70% |
| Vidyo1 | -2.2% | -2.3% | 103% |  |  |  |  |  | -2.1% | -1.8% | 84% |  | -4.0% | -3.5% | 89% |
| Star-7x7 (11 tap) | I |  |  |  | RA |  |  |  | LD |  |  |  | LDP |  |  |
|  | 2 Shp Gain | 1 Shp Gain | Ratio |  | 2 Shp Gain | 1 Shp Gain | Ratio |  | 2 Shp Gain | 1 Shp Gain | Ratio |  | 2 Shp Gain | 1 Shp Gain | Ratio |
| BQSquare | -0.2% | -0.2% | 94% |  | -9.4% | -8.4% | 89% |  | -6.1% | -5.3% | 86% |  | -1.0% | -1.0% | 103% |

For 9-tap shapes:

- “Star-5x5” results in 2, 4, 5, 1, 1 Red Flags in AI, RA, LD, LDP, RA_10

- “Cross-9x9” results in 0, 3, 4, 8, 4 Red Flags in AI, RA, LD, LDP, RA_10

- “Star-5x5” and “Cross-9x9” only have 1 overlapping Red Flag (In LD as highlighted above)

- All other Red Flags occur in different sequences / coding configurations, i.e. complementary.

For 10-tap shapes:

- “Asymmetric cross9x7 with 3x3 square center” results in 0, 0, 1, 1, 0 Red Flags in AI, RA, LD, LDP, RA_10; in particular for video conferencing sequence and sequence with combined zoom-out + vertical-panning effect (BQSquare).

- “Asymmetric snowflake(star)-7x5” results in 1, 2, 2, 1, 0 Red Flags in AI, RA, LD, LDP, RA_10; in particular for sequence with zoom-in effect (PartyScene), sequence with combined zoom-out + vertical-panning effect (BQSquare), and video conferencing sequence.

For 11-tap:

- “Symmetric snowflake(star)-7x7” results in 0, 1, 1, 0, 0 Red Flags in AI, RA, LD, LDP, RA_10; which occur on BQSquare.

# Conclusions from the Red Flag analysis

- When constrained with only 9 taps, “Star-5x5” and “Cross-9x9” provide complementary coding efficiency for using ALF.

- The two 10-tap shapes are still sensitive to certain type of effects exhibited in the video content.

- Suggest to keep the current two 9-tap shapes, “Star-5x5” and “Cross-9x9”, for ALF.

# Patent rights declarations

Samsung may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- W. Lai, F. C. A. Fernandes, H. Guermazi, F. Kossentini, and M. Horowitz, “CE8 Subtest 4: ALF using vertical-size 5 filters with up to 9 coefficients,” JCTVC-F303, Torino, Italy 2011

- C.-Y. Chen, C.-M. Fu, C.-Y. Tsai, Y.-W. Huang, S. Lei, S. Esenlik, M. Narroschke, T. Wedi, I. S. Chong, and M. Karczewicz, “Non-CE8.c.7: Single-source SAO and ALF virtual boundary processing with cross9x9,” JCTVC-G212, Geneva, Switzerland, Nov. 2011

- “CE8.b.2: ALF with single filter type,” JCTVC-H0068, San José, CA, USA, Feb. 2012