| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0418_r1 WG11 Number: m23294 |
| --- | --- |

| Title: | Non-CE8: Unification of k-index mapping for ALF luma filter coefficients of the two shapes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wang (PoLin) Lai Felix C. A. Fernandes | Tel: Email: | wlai@sta.samsung.com +1 972 761 7456 ffernandes@sta.samsung.com +1 972 761 7427 |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution presents unification of k-index mapping for ALF luma filter coefficients of the two shapes. The k-index mapping maps ALF luma coefficients to different order of exponential Golomb code. In HM5.0, the two k-index mapping tables for two filter shapes snowflake(star)-5x5 and cross-9x9 have different index ranges, leading to additional processing steps in entropy coding and decoding for ALF luma coefficients. The unification of k-index mapping tables using the same index range removes the associated processing steps, with exactly the same coding results.

# Introduction

In High-Efficiency Video Coding test model version 5.0 (HM5.0), there are two filter shapes for ALF: Snowflake(star)-5x5 and cross-9x9, both have nine coefficients [1][2]. The ALF luma coefficients are coded with syntax transmitted k-table indices which corresponds to the k-th order exponential Golomb code. Furthermore, the mappings for filter coefficients sharing the same index are defined as:

| // Snowflake (star) 5x5 Int depthIntShape0Sym[10] = { 1, 3, 1, 3, 4, 3, 3, 4, 5 }; | // Cross 9x9 Int depthIntShape1Sym[10] = { 5, 6, 7, 8, 5, 6, 7, 8, 9, }; |
| --- | --- |

As can be seen, the two k-index mapping tables utilize different ranges of indices, while the index differences with respect to the center coefficient for the two mappings share the same pattern. The different index ranges leading to additional processing steps in entropy coding and decoding for ALF luma coefficients. In the “Adaptive loop filter parameter syntax table”:

Adaptive loop filter parameter syntax table

| For Snowflake(star)-5x5 AlfMinKPos is 0 and AlfMaxKPos is 5; while for cross-9x9 AlfMinKPos is 4 and AlfMaxKPos is 9. The corresponding values are derived from the ALF filter shape syntax and the corresponding k-index mapping table. for (i=AlfMinKPos; i < AlfMaxKPos ; i++) |  |  |
| --- | --- | --- |
| alf_golomb_index_bit[i] | 2 | u(1) |

In the software, this decoding part is implemented as:

Void TDecEntropy::readFilterCodingParams(ALFParam* pAlfParam)

// Determine minScanVal

int minScanVal = (pAlfParam->filter_shape == ALF_STAR5x5) ? 0: MIN_SCAN_POS_CROSS;

// Determine maxScanVal

maxScanVal = 0;

pDepthInt = pDepthIntTabShapes[pAlfParam->filter_shape];

for(ind = 0; ind < pAlfParam->num_coeff; ind++)

maxScanVal = max(maxScanVal, pDepthInt[ind]);

In this contribution, unification of the two k-index mapping table is presented using the same index range, such that the associated processing steps can be eliminated, with exactly the same coding results.

# Unification of k-index mapping for the two ALF shapes in HM5.0

The unification is achieved simply by making the k-index mapping of the cross-9x9 filter having the same range as that of snowflake (star) 5x5. Thus, for the same coefficient position (with respect to the center position), the mapping indices are the same for the two shapes.

| // Snowflake (star) 5x5 Int depthIntShape0Sym[10] = { 1, 3, 1, 3, 4, 3, 3, 4, 5 }; | // Cross 9x9 Int depthIntShape1Sym[10] = { 1, 2, 3, 4, 1, 2, 3, 4, 5, }; |
| --- | --- |

With unified k-index mapping ranges, there is no need to derive AlfMinKPos and AlfMaxKPos. The corresponding decoding process and be simplified as:

Adaptive loop filter parameter syntax table

| Coding results is exactly the same as by HM5.0. for (i=0; i < 5 ; i++) |  |  |
| --- | --- | --- |
| alf_golomb_index_bit[i] | 2 | u(1) |

Furthermore, the unification provides a decoder implementation possibility that the two shapes can be conceptually viewed as virtual superset shape, thanks to the same symmetric properties and now the same k-index mapping, while different subset of coefficient positions are decoded and used:

| 1, 2, 1, 3, 1, 3, 4, 3, 1, 2, 3, 4, 5, 4, 3, 2, 1, 3, 4, 3, 1, 3, 1, 2, 1, Virtual superset shape | 1, 2, 1, 3, 1, 3, 4, 3, 1, 2, 3, 4, 5, 4, 3, 2, 1, 3, 4, 3, 1, 3, 1, 2, 1, Snowflake (star) 5x5 |
| --- | --- |
|  | 1, 2, 1, 3, 1, 3, 4, 3, 1, 2, 3, 4, 5, 4, 3, 2, 1, 3, 4, 3, 1, 3, 1, 2, 1, Cross-9x9 |

# Patent rights declarations

Samsung may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- W. Lai, F. C. A. Fernandes, H. Guermazi, F. Kossentini, and M. Horowitz, “CE8 Subtest 4: ALF using vertical-size 5 filters with up to 9 coefficients,” JCTVC-F303, Torino, Italy 2011

- C.-Y. Chen, C.-M. Fu, C.-Y. Tsai, Y.-W. Huang, S. Lei, S. Esenlik, M. Narroschke, T. Wedi, I. S. Chong, and M. Karczewicz, “Non-CE8.c.7: Single-source SAO and ALF virtual boundary processing with cross9x9,” JCTVC-G212, Geneva, Switzerland, Nov. 2011