| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0419_r1 WG11 Number: m23295 |
| --- | --- |

| Title: | Non-CE8: Techniques in LCU-based ALF in CE8.a.2 and CE8.a.4 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Wang (PoLin) Lai Felix C. A. Fernandes | Tel: Email: | wlai@sta.samsung.com +1 972 761 7456 ffernandes@sta.samsung.com +1 972 761 7427 |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document presents high level summary of techniques in the two LCU-based ALF approaches, namely CE8.a.2 and CE8.a.4 with changes in ALF only (SAO the same as HM5.0, two-stage SAO-ALF).

# Introduction

In the 8th JCTVC meeting, two LCU-based ALF approaches are proposed:

- CE8.a.2, by Cisco [1]

- CE8.a.4 with changes in ALF only (SAO as HM5.0), by MediaTek, Qualcomm, Toshiba [2]

While they both have LCU-by-LCU ALF estimation and selection, the technical details appear to be noticeably different. This document is meant to provide high level summary of techniques in the two LCU-based ALF approaches, to facilitate better understanding for the committee.

# Technical comparisons

## Coding efficiency (AI / RA / RA_10 / LD / LDP)

CE8.a.2

- LCU-Encoding: 0.3 / 0.6 / 1.0 / 0.2 / 0.0

- (Pic-Encoding: Currently s/w does not provide such)

CE8.a.4 with Changes only on ALF

- LCU-Encoding: 0.4 / 0.6 / 1.0 / 0.6 / 0.6

- (Pic-Encoding: 0.1 / 0.2 / 0.3 / 0.1 / 0.1)

## Slice level: Filter history syntax / operation, and memory implications

CE8.a.2

- ALF parameters are coded within LCU.

- Adaptive codebook of size-8 for each shape:

  - Populated incrementally LCU by LCU when determined using new filter.

  - Updated on the fly: When full, if new filter needed, oldest filter goes out.

- 8 filters for each shape, total 16 filters  Worst case 16 filters memory requirement.

CE8.a.4 with Changes only on ALF

- ALF parameters coded in APS (single-slice cases) or in slice header (multi-slice cases)

- Repeat-row flag: One row of LCUs use same ALF parameter of above whole row.

- Run-length for multiple consecutive LCUs sharing the same ALF parameters.

- Merge-up flag: Current LCU use same ALF parameters as the above LCU.

  - Potential worst case memory: Whole line of LCU, each LCU with multiple filters

 Using “the number of filters per frame” constraint to control the memory usage:

128 per frame, and allocate this budget among LCUs with merging process at encoder. 
(For example: Budges for the current LCU = Remaining # filter / # of remaining LCU.)

## LCU-level: Decoding process and syntax

CE8.a.2

- LCU on/off for ALF, separate 2 flags for luma (Y) and luma (CbCr)

No CU-level on/off , i.e. no filter re-design

- Shape flag {snowflake-5x5 or cross-9x9}

- Copy flag

/ Ture: From the codebook of the selected shape, pick a filter using copyID

\ False: decode new filter of the selected shape

- Same filter coefficients / shape for Y and for CbCr

- Coding technique for ALF coefficients:

  - QP-dependent and pic_height-dependent.

  - CABAC for coding filter coefficients

  - Center coefficients derived using sum as 1

CE8.a.4 with Changes only on ALF

- < If Repeat-row flag = false, Run-length = 0, and Merge-up flag = false >

- LCU on/off for ALF, separate 3 flags for Y, Cb, and Cr component

No CU-level on/off , i.e. no filter re-design

(NOTE: CU on/off syntax remains, and is used for Pic-Encoding method.)

- New filter flag

/ True: decode new filter set

\ False: from stored filter set, pick one using store_filter_ID

- IF new filter set:

  - For Y: Filter shapes {snowflake-5x5 or cross-9x9}

  - For Y: Number of filters, and filter label merge syntax*

  - For Y: Filter coefficient coding method: self or from other filter

  - For Y: Decode coefficients

  - For Cb or Cr: Filter shapes {snowflake-5x5 or cross-9x9}

The number of filters is equal to one

  - For Cb or Cr: Different filter coefficients (not share with Y)

- Classify blocks using BA directional classification* computed on subset of pixels
Apply filters according to class label and merge syntax.

- Coding technique for ALF coefficients:

  - QP-dependent

  - VLC for coding filter coefficients (APS does not allow CABAC).

  - Center coefficients differential-coded from other coefficients.

Note*: For LCU-encoding, most LCU choose 1 filter, thus for decoder no BA and label merging.

# Patent rights declarations

Samsung may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “CE8.a.2: ALF with LCU-based syntax,” JCTVC-H0066, San José, CA, USA, Feb. 2012

- “CE8.a.4: One-stage/Two-stage SAO and ALF with LCU-based syntax,” JCTVC-H0274, San José, CA, USA, Feb. 2012