| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0420_r1 WG11 Number: m23296 |
| --- | --- |

| Title: | Non-CE8: Variable codebook size for LCU-based ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Zhan Ma Wang (PoLin) Lai Felix C. A. Fernandes | Tel: Email: | zhan.ma@sta.samsung.com +1 972 761 7134 wlai@sta.samsung.com +1 972 761 7456 ffernandes@sta.samsung.com +1 972 761 7427 |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution presents informational results of using variable codebook size for LCU-based ALF, in which the codebook size is set to be resolution-dependent. Results indicate increasing codebook size for high resolution does not improve coding efficiency.

# Introduction

In the 7th JCTVC meeting, LCU-based ALF syntax and encoding method was proposed [1]. In this approach, filters are estimated on a LCU-by-LCU basis in raster-scanning order, and an adaptive codebook is populated on-the-fly as new filters being used. The size of the adaptive codebook is fixed. As the encoding scanning through LCUs, if the codebook is already populated full, and a new filter is determined beneficial to the coding efficiency, a previously stored filter in the codebook will be eliminated to accommodate the new filter.

# Variable codebook size for LCU-based ALF

To facilitate larger codebook size for high resolution sequences, while also taking into account the coefficient side information for low resolution sequences, resolution-dependent codebook size can be determined as follows:

- Codebook-size = ceiling ( FrameWidth / LCU_size ) * C

In the above manner, higher resolution sequences will use larger codebook size. More specifically, for C=1, the codebook size will be exactly large enough even to hold a whole row of LCU’s filters. See Figure 1 below for illustration.

| LCU | LCU | LCU | LCU | LCU | LCU | LCU | LCU (Upper) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LCU | LCU | LCU | LCU | LCU | LCU | LCU (Left) | LCU (current) |
| LCU | LCU | LCU | LCU | LCU | LCU | LCU | LCU |

Figure. 1 Codebook-size = ceiling ( FrameWidth / LCU size ).
The codebook can hold filter from the shaded LCU’s, for the current LCU to select candidate filters.

For HEVC test sequences, with LCU size = 64 as in common test conditions, the codebook sizes set with ceiling ( FrameWidth / LCU_size ) will be:

- 1920x1080: 30, for each of the two ALF shapes

- 1280x720: 20, for each of the two ALF shapes

- 832x480: 13, for each of the two ALF shapes

- 416x240: 7, for each of the two ALF shapes

# Simulation results

Note that resolution-dependent codebook size is implemented on top of the latest release of CE8.a.2 “ALF with LCU-based syntax” [2]. Yellow highlight indicates losses as compared to CE8.a.2.

|  | All Intra HE |  |  |  | Random Access HE |  |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  | Y | U | V |  | Y | U | V |
| Class A (8bit) | 1.2% | 0.3% | 0.5% |  |  |  |  |  |  |  |  |
| Class B | 0.4% | 0.7% | 0.6% |  | 1.5% | -0.6% | -0.6% |  |  |  |  |
| Class C | 0.0% | 1.6% | 1.6% |  | 0.8% | 0.0% | -0.6% |  |  |  |  |
| Class D | 0.2% | 1.6% | 1.9% |  | 0.2% | -0.3% | 0.0% |  |  |  |  |
| Class E | 0.4% | 4.0% | 4.4% |  | 0.7% | 0.2% | 0.4% |  |  |  |  |
| Overall | 0.4% | 1.6% | 1.7% |  |  |  |  |  |  |  |  |
|  | 0.4% | 1.6% | 1.7% |  | 0.7% | -0.1% | -0.2% |  |  |  |  |
| Class F | 0.1% | 0.8% | 0.8% |  | 0.7% | -0.1% | -0.1% |  |  |  |  |
| Enc Time[%] |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B HE |  |  |  | Low delay P HE |  |  |  |  |  |  |
|  | Y | U | V |  | Y | U | V |  |  |  |  |
| Class A | 0.2% | -0.6% | -0.9% |  |  |  |  |  |  |  |  |
| Class B | 0.0% | -0.3% | -0.5% |  | 0.2% | -1.4% | -1.8% |  |  |  |  |
| Class C | 0.7% | 1.1% | 1.6% |  | -0.1% | -0.6% | -0.9% |  |  |  |  |
| Class D | -0.1% | 0.7% | -1.9% |  | 0.2% | 0.6% | 0.5% |  |  |  |  |
| Class E | 0.2% | 0.1% | -0.3% |  | -0.1% | 0.6% | -2.9% |  |  |  |  |
| Overall | 0.2% | 0.1% | -0.4% |  | 0.1% | -0.4% | -1.2% |  |  |  |  |
|  | 0.1% | 0.1% | -0.5% |  | 0.2% | 0.1% | -0.4% |  |  |  |  |
| Class F | 0.2% | -0.6% | -0.9% |  | 0.2% | -1.4% | -1.8% |  |  |  |  |
| Enc Time[%] |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |  |  |  |  |  |

# Patent rights declarations

Samsung may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- A. Fuldseth and G. Bjøntegaard, “Improved ALF with low latency and reduced complexity,” JCTVC-G499, Geneva, Switzerland, Nov. 2011

- A. Fuldseth and G. Bjøntegaard, “ALF with LCU-based syntax,” JCTVC-H0066, San José, CA, USA, Feb. 2012