| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, CA, USA, 1-10 February, 2012 | Document: JCTVC-H0421 M23297 |
| --- | --- |

| Title: | AHG7: Cross-verification of controllable memory bandwidth reduction with bi-pred to uni-pred conversion (JCTVC-H0096 from Sharp) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc; |  |  |

_____________________________

# Abstract

This contribution reports cross-check results for controllable memory bandwidth reduction with bi-pred to uni-pred conversion (JCTVC-H0096). In their proposal, two variants of bi-prediction are proposed, 1) 4x8 and 8x8 bi-prediction is disabled when SCU size is 8x8, 2) 8x8, 4x8 and 8x4 bi-prediction is disabled in simulation (intention here is only to disable when SCU size 16x16). A signaling method based on SCU size and bi-prediction disabling flag is also proposed to enable or disable bi-prediction restrictions for small block size PUs.  When the bi-prediction is disabled, bi-prediction direction is not signaled in the bitstream. Instead, bi-prediction is inferred to list 1 uni-directional prediction for those small size PUs. The merge/skip MVP list derivation process is modified accordingly when bi-prediction is disabled for small block size PUs. The proposed restrictions can effectively cuts the worst case motion compensation memory bandwidth.  Compared to HM5.0 anchor, disabling 4x8 and 8x4 bi-prediction leads to an average BD-rate increase of 0.0% in RA-HE, 0.1% in RA-LC, 0.1% in LB-HE and 0.2% in LB-LC, while disabling 4x8, 8x4 and 8x8 bi-prediction causes an average coding loss of  1.5% in RA-HE, 2.6% in RA-LC, 1.8% in LB-HE and 3.3% in LB-LC, respectively. The results match the ones reported by proponents. The source code is also checked to verify its consistency with the proposal description.

.

# Test Settings and Conditions

The simulations of this document have used HM4.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed. Two settings are tested,

- Disable 4x8 and 8x4 bi-prediction

- Disable 4x8, 8x4 and 8x8 bi-prediction.

# Experimental results

The experimental results are summarized in Table 1 and Table 2. Compared to HM5.0 anchor, the average BD-rate increase is 0.0% in RA-HE, 0.1% in RA-LC, 0.1% in LB-HE and 0.2% in LB-LC for setting 1, and 1.5% in RA-HE, 2.6% in RA-LC, 1.8% in LB-HE and 3.3%. Those results match what proponents have reported.  Please be advised in Table 1 and Table 2 runtime may not be accurate.

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | -0.1% | -0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class C | 0.1% | 0.0% | 0.1% | 0.1% | 0.2% | 0.3% |  |  |  |
| Class D | 0.1% | 0.1% | 0.2% | 0.1% | 0.0% | 0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
|  | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class F | -0.3% | -0.1% | -0.2% | -0.2% | -0.2% | -0.2% |  |  |  |
| Enc Time[%] | 89% |  |  | 86% |  |  | 83% |  |  |
| Dec Time[%] | 93% |  |  | 86% |  |  | 81% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class B | 0.0% | 0.1% | 0.2% | 0.1% | -0.1% | 0.3% |  |  |  |
| Class C | 0.1% | 0.0% | 0.2% | 0.2% | 0.4% | 0.2% |  |  |  |
| Class D | 0.1% | 0.1% | 0.5% | 0.2% | 0.4% | -0.1% |  |  |  |
| Class E | 0.1% | -0.1% | -0.4% | 0.2% | -0.2% | 0.3% |  |  |  |
| Overall | 0.1% | 0.1% | 0.2% | 0.2% | 0.1% | 0.2% |  |  |  |
|  | 0.1% | 0.0% | 0.1% | 0.2% | 0.1% | 0.2% |  |  |  |
| Class F | -0.5% | -0.4% | -0.5% | -0.5% | -0.8% | -0.6% |  |  |  |
| Enc Time[%] | 89% |  |  | 87% |  |  |  |  |  |
| Dec Time[%] | 91% |  |  | 89% |  |  |  |  |  |

Table 1.  Experimental results on 4x8 and 8x4 bi-prediction restriction

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 1.0% | 1.7% | 1.4% | 1.7% | 2.7% | 2.6% | 0.6% | 1.0% | 1.1% |
| Class B | 0.7% | 0.7% | 0.6% | 1.3% | 1.1% | 1.2% | 0.7% | 0.7% | 0.6% |
| Class C | 1.6% | 1.9% | 2.1% | 2.9% | 3.1% | 3.4% |  |  |  |
| Class D | 2.7% | 2.8% | 2.8% | 4.2% | 4.6% | 4.4% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 1.5% | 1.7% | 1.7% | 2.6% | 2.8% | 2.8% | 0.6% | 0.8% | 0.8% |
|  | 1.5% | 1.7% | 1.7% | 2.6% | 2.8% | 2.8% | 0.6% | 0.8% | 0.8% |
| Class F | 1.7% | 1.9% | 1.9% | 2.6% | 3.0% | 2.9% |  |  |  |
| Enc Time[%] | 77% |  |  | 73% |  |  | 81% |  |  |
| Dec Time[%] | 92% |  |  | 90% |  |  | 93% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class B | 0.9% | 0.5% | 0.8% | 1.7% | 1.1% | 1.4% |  |  |  |
| Class C | 2.2% | 2.0% | 2.4% | 3.8% | 4.0% | 3.8% |  |  |  |
| Class D | 3.5% | 3.0% | 3.5% | 5.8% | 6.3% | 5.6% |  |  |  |
| Class E | 0.8% | 0.3% | 0.8% | 1.9% | 1.3% | 1.9% |  |  |  |
| Overall | 1.8% | 1.5% | 1.9% | 3.3% | 3.2% | 3.1% |  |  |  |
|  | 1.8% | 1.5% | 1.9% | 3.3% | 3.2% | 3.1% |  |  |  |
| Class F | 2.8% | 2.8% | 2.7% | 4.4% | 3.8% | 4.3% |  |  |  |
| Enc Time[%] | 73% |  |  | 69% |  |  |  |  |  |
| Dec Time[%] | 86% |  |  | 87% |  |  |  |  |  |

Table 2.  Experimental results on 4x8, 8x4 and 8x8 bi-prediction restriction

A bug has been identified that sometimes bi-pred is still used for small size PUs even if it is disabled. Table 3 summarizes the results for 4x8/8x4 bi-prediction restriction after bug fix. The loss is about 0.1% higher on average when compared to the previous version. The results match ones provided by proponents.

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | -0.1% | 0.1% | 0.2% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class B | 0.1% | 0.0% | 0.0% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class C | 0.1% | 0.2% | 0.3% | 0.3% | 0.3% | 0.3% |  |  |  |
| Class D | 0.3% | 0.2% | 0.2% | 0.3% | 0.1% | 0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
|  | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| Class F | -0.3% | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% |  |  |  |
| Enc Time[%] | 84% |  |  | 91% |  |  | 93% |  |  |
| Dec Time[%] | 85% |  |  | 96% |  |  | 94% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class B | 0.1% | 0.1% | 0.3% | 0.2% | 0.0% | 0.0% |  |  |  |
| Class C | 0.2% | 0.2% | 0.1% | 0.4% | 0.5% | 0.2% |  |  |  |
| Class D | 0.3% | -0.2% | 0.1% | 0.4% | 1.0% | 0.8% |  |  |  |
| Class E | 0.0% | -0.2% | -0.3% | 0.3% | -0.1% | 0.2% |  |  |  |
| Overall | 0.1% | 0.0% | 0.1% | 0.3% | 0.3% | 0.3% |  |  |  |
|  | 0.1% | 0.0% | 0.0% | 0.3% | 0.3% | 0.3% |  |  |  |
| Class F | -0.5% | -0.3% | -0.6% | -0.6% | -1.2% | -0.7% |  |  |  |
| Enc Time[%] | 90% |  |  | 89% |  |  |  |  |  |
| Dec Time[%] | 94% |  |  | 91% |  |  |  |  |  |

Table 3.  Experimental results on 4x8 and 8x4 bi-prediction restriction w/ bug fix

# Comments

# The proposed technique (Variant 1 which disables 4x8 and 8x4 bi-prediction) reduces the worst case motion compensation memory bandwidth requirements by 32% (in HM5.0 the worst case is 8x4 or 4x8 bi-prediction, reference block size for 8x8 is 660 = 15*11*2*2, now the worst case is 8x8 bi-prediction, reference block size for 8x8 is 450 = 15*15*2, 220/660 = 31.8%) without harming coding efficiency, this technique is very beneficial for memory bandwidth reduction purpose.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-F900, Torino, IT, July 2011.

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-F803, Torino, IT, July 2011

[3]  T. Ikai, “AHG7: Controllable memory bandwidth reduction with bi-pred to uni-pred conversion”, JCT-VC Document, JCTVC-H0096, San Jose, CA, USA,  Feb.  2012