| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0422 WG11 Number: m23298 |
| --- | --- |

| Title: | CE8.a.4 Crosscheck of JCTVC-H0274 Table 7, 8, and 9: ALF with LCU-based syntax (SAO the same as HM5.0, two-stage SAO-ALF) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Wang (PoLin) Lai Felix C. A. Fernandes | Tel: Email: | wlai@sta.samsung.com +1 972 761 7456 ffernandes@sta.samsung.com +1 972 761 7427 |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This report presents Samsung’s cross-check results for CEb.a.4 JCTVC-H0274 Table 7, 8, and 9: ALF with LCU-based syntax (SAO the same as HM5.0, two-stage SAO-ALF). Three ALF configurations with LCU-based syntax were cross-checked: (1) Picture-based encoding algorithm; (2) LCU-based encoding algorithm; and (3) Low-delay multi-slice per picture, using LCU-based encoding algorithm. Rate-PSNR numbers match exactly to the proponent’s results.

Syntax study: ALF syntax is put in APS for single slice per picture, and in slice header for multi-slice per picture. The syntax allows one whole row of LCU using same ALF parameters of above row, one LCU using same ALF parameter as above LCU, run-length coding for multiple consecutive LCU sharing same ALF parameters. Otherwise, the current LCU can use stored filter, or has its own filter decoded.

Software encoder study: In picture-based encoding (1), reduced filter design passes is used, as compared to HM5.0 with 14 passes. In LCU-based encoding (2)(3), no filter redesign (CU on/off control map not used). In (2)(3), a constraint on number of filters per frame is used to allocate filter budges among LCUs.

# Cross-check results

The simulations were run in the 64-bit Linux cluster environment.

Cross-check JCTVC-H0274 [1] Table 7

| - Single-slice per picture - ALF with LCU-based syntax - Picture-based encoding algorithm |  | AI-HE | RA-HE | RAHE10 | LB-HE | LP-HE |
| --- | --- | --- | --- | --- | --- | --- |
|  | Average | 0.1% | 0.2% | 0.3% | 0.1% | 0.1% |
|  | Enc Time | 88 (81) | 96 (91) | 95 (92) | 97 (90) | 97 (91) |
|  | Dec Time | 100 (102) | 100 (97) | 94 (88) | 101 (99) | 101 (99) |

Cross-check JCTVC-H0274 Table 8

| - Single-slice per picture - ALF with LCU-based syntax - LCU-based encoding algorithm |  | AI-HE | RA-HE | RAHE10 | LB-HE | LP-HE |
| --- | --- | --- | --- | --- | --- | --- |
|  | Average | 0.4% | 0.6% | 1.0% | 0.6% | 0.6% |
|  | Enc Time | 78% (73) | 95% (91) | 93% (91) | 96% (94) | 94% (92) |
|  | Dec Time | 92% (96) | 96% (95) | 91% (87) | 91% (93) | 90% (93) |

Cross-check JCTVC-H0274 Table 9

| - Multi-slice per picture - ALF with LCU-based syntax - LCU -based encoding algorithm |  | AI-HE | RA-HE | RAHE10 | LB-HE | LP-HE |
| --- | --- | --- | --- | --- | --- | --- |
|  | Average |  |  |  | -1.8% | 2.9% |
|  | Enc Time |  |  |  | 103 (///) | 105 (///) |
|  | Dec Time |  |  |  | 110 (///) | 111 (///) |

# Syntax study

- Syntax is in APS for single slice per picture, and in slice header for multi-slice per picture.

- The syntax allows one whole row of LCU using same ALF parameters of above row, one LCU using same ALF parameter as above LCU, run-length coding for multiple consecutive LCU sharing same ALF parameters. Otherwise, the current LCU can use stored filter, or has its own filter decoded.

# Software encoder study

- In picture-based encoding, reduced filter design passes is used, as compared to HM5.0 with 14 passes.

- In LCU-based encoding, no filter redesign (CU on/off control map not used).

- In LCU-based encoding, a constraint on number of filters per frame is used to allocate filter budges among LCUs.

# Patent rights declarations

Samsung may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “CE8.a.4: One-stage/Two-stage SAO and ALF with LCU-based syntaxJCTVC-H0274, San José, CA, USA, Feb. 2012