| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0423 |
| --- | --- |

| Title: | Indication of the temporal structure of coded video sequences |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Srikanth Manchenahally Gopalakrishna Nokia Research Center Visiokatu 1, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

The contribution returns to the topic of describing the temporal structure of coded video sequences, which was proposed earlier in documents JCTVC-D200 and JCTVC-E279. A primary use case for knowing the temporal structure a priori is to assist transraters and bitstream extractors in pruning the coded video sequence.

By referring to the reference picture set index, the proposal is asserted to provide a more accurate description of the temporal structure than what was proposed in JCTVC-D200 and JCTVC-E279. Furthermore, by specifying the temporal structure in the sequence parameter set, it is reportedly possible to omit certain syntax elements from the slice header and hence reduce the bit count of the slice header.

The proposal can be considered to consist of the following ingredients:

- The short term reference picture set syntax structure is moved from PPS to SPS.

- A SOP description syntax structure is added. The syntax structure describes a structure of pictures (SOP) in decoding order.

- A coded video sequence can be described as part of the SPS with reference to described SOPs.

- When the SOP description is used, the slice header includes a SOP index referring to a particular SOP description and a picture index within the SOP structure. Certain syntax elements of the slice header are excluded provided that the SOP description is used.

It is asserted that up to about 3% bitrate savings may be possible at a selected low bitrate and high frame rate relating to the test case “dyadic and nested temporal scalability for high frame rate” in the common test conditions specified in JCTVC-G1036, while typical bitrate savings are expected to be one magnitude smaller.

# Motivation

## Motivation for Signaling of GOP/Sequence Structure (JCTVC-D200)

JCTVC-D200 proposed a coding structure SEI message among other things. The motivation for proposing the SEI message was given in the introduction of Section 4 of JCTVC-D200 as copied into the following paragraphs.

Although the current HEVC design and the H.264/AVC standard allow for flexible patterns of coded pictures, many encoders, including the JM and HM reference software, generate sequences of coded pictures following a fixed pattern.   Even when a fixed pattern is in used, gateways and decoders are not aware of it.

Knowing the coding structure a priori can help in dropping frames for transraters and bitstream extractors.  Additionally, transraters can optimally choose frames to transcode and minimize drift.  DVRs can also utilize the coding structure information for operations like fast forwarding.  Finally, knowing the structure can enable decoders to process certain frames in parallel.  For example, for the hierarchical P picture coding structure shown below, pictures P1 and P2 can be processed in parallel, then P3 and P4, and so on.

An encoder generates a sequence of coded pictures that consists of one or more GOPs (Group of Pictures).  Here, a GOP is defined as a sequence of coded pictures that start with an I/IDR picture and ends with a picture immediately preceding the next I/IDR picture.  Many encoders generate one or more GOPs of the same structure of coded pictures.  Furthermore, each GOP generated by these encoders typically consists of a smaller fixed pattern that repeats.  The coding structure SEI message describes this smaller, repeating, fixed pattern of a temporal picture coding structure that is used by the encoder.  For example, in the figure shown below, the 3 B picture structure repeats between I and P pictures; this SEI message can be used to describe the repeating structure.  Hereinafter, the first lowest temporal layer picture followed by the repeating set of pictures (in coding order) are referred to as an SOP (Structure of Pictures).  A GOP (Group of Pictures) begins with an I frame, and includes one or more SOPs.  In the example below, there are 4 pictures in the SOP and 3 SOPs in the GOP: the first SOP consists of the first I picture followed by the 3 immediately following B pictures, the second SOP consists of the first P picture followed by the next 3 B pictures, and so on.

## Motivation for This Proposal

JCTVC-D200 proposed an SEI message for coding/temporal structure signaling. The proposed message syntax was later updated in JCTVC-E279r3 and copied below:

| temporal_structure( payloadSize ) { | Descriptor |
| --- | --- |
| num_active_temporal_layers_minus1 | u(3) |
| coding_structure_flag | u(1) |
| if( coding_structure_flag ) { |  |
| num_pictures_in_sop_minus1 | ue(v) |
| num_sops_in_gop | ue(v) |
| for( i = 0; i < num_pictures_in_sop_minus1 + 1; i++ ) { |  |
| primary_pic_type[ i ] | u(2) |
| ref_flag[ i ] | u(1) |
| temporal_num[ i ] | u(3) |
| display_num[ i ] | ue(v) |
| } |  |
| } |  |
| coding_statistics_flag | u(1) |
| if( coding_statistics_flag ) { |  |
| average_frame_rate_flag | u(1) |
| average_bit_rate_flag | u(1) |
| if( average_frame_rate_flag ) |  |
| average_frame_rate | u(16) |
| if( average_bit_rate_flag ) |  |
| for( i = 0; i < NumTemporalLayers; i++ ) |  |
| average_bit_rate[ i ] | u(16) |
| } |  |
| } |  |

We found that the design of the temporal structure SEI message could be improved from the following two aspects:

- The temporal structure SEI message does not reveal the prediction references. For example, it is not known, which pictures refer to other pictures having the same temporal_id and hence cannot act as temporal_id switch points. Thanks to the introduction of reference picture sets, the information on prediction references could be easily added to the temporal structure description by including RPS index as one item for each described picture.

- Some items in the slice header would become redundant if the temporal structure were normatively provided. In other words, there is a possibility to reduce the bit count used for slice headers by deducing some of the syntax element values.

# Overview of the Proposal

The proposal consists of the following ingredients:

- It is proposed to move the short term reference picture set syntax structure from PPS to SPS. It is believed many times the temporal structure of the bitstream and RPS are determined without adaptation to content, and hence moving the RPS structures from PPS to SPS do not essentially limit flexibility. In any case, the option to specify RPS in slice headers remains.

- A SOP description syntax structure is added. The syntax structure describes a structure of pictures in decoding order. For each described picture the following information is given:

  - nal_ref_flag

  - temporal_id

  - slice type (if all slices of the picture use the same type)

  - POC difference relative to the previous picture in decoding order, and

  - an index to short-term RPS.

- A coded video sequence can be described as part of the SPS by providing a list of pairs, where each pair contains a SOP index referring to a particular SOP description and a repetition count. The repetition count for the last SOP of the coded video sequence may be infinite.

- In the slice header, the following syntax elements are included provided that the SOP description is used:

  - SOP index referring to a particular SOP description

  - Picture index within the SOP structure for pictures having the same nal_ref_flag and temporal_id as the current picture

The following syntax elements are excluded from the slice header provided that the SOP description is used:

  - slice_type (if all slices of the picture use the same type)

  - pic_order_cnt_lsb

  - all reference picture set related syntax elements

It is noted that this proposal would make the coding structure part of the temporal structure SEI proposed in JCTVC-D200 and JCTVC- JCTVC-E279r3 unnecessary, while the part of the SEI message giving the statistical information of the frame rates and bitrates per temporal_id values still has its own merits.

# Bitrate Reduction

The bitrate reduction of the proposal is analyzed with reference to the common conditions specified in JCTVC-G1036. For non-regular SOP patterns, the proposal cannot be used and hence no bitrate gains for them are achieved. In order to provide a rough estimate for achievable bitrate savings in regular SOP patterns, two cases of the common conditions were analyzed for two assumptions: 1) low bitrate (100 kbps), 1 slice per picture, frame rate as specified in the common conditions, 2) 1500 bytes/slice. In the next version of this document, a more comprehensive analysis using the common conditions may be provided.

| Test case | Bit count in current slice header | Bit count in proposal | Bitrate saving |
| --- | --- | --- | --- |
| 2.1 Random access common conditions | slice_type: 3 pic_order_cnt_lsb: 5 (estimated to give sufficient error robustness) short_term_ref_pic_set_pps_flag: 1 short_term_ref_pic_set_idx: 4 Total: 13 | sop_idx: 1 (after the first SOP) pic_idx_in_sop: 2 Total 3 | 0.3% (100 kbps@30Hz, 1 slice/pic) 0.1% (1500B/slice) |
| 2.5 Dyadic and nested temporal scalability for high frame rate | slice_type: 3 pic_order_cnt_lsb: 9 (estimated to give sufficient error robustness) short_term_ref_pic_set_pps_flag: 1 short_term_ref_pic_set_idx: 6 Total: 19 | sop_idx: 1 (after the first SOP) pic_idx_in_sop: 4.3 (on average per picture) Total: 5.3 | 3.3% (100kbps@240Hz, 1 slice/pic) 0.1% (1500B/slice) |

# Proposed Changes to Specification Text

Add the following definitions:

SOP sequence: One or more SOPs consecutive in decoding order

structure of pictures: One of more coded pictures consecutive in decoding order, in which the first coded picture in decoding order has temporal_id equal to 0 and nal_ref_flag equal to 1

Add the following to abbreviations:

SOP	Structure of Pictures

Add the following syntax to the sequence parameter set:

| num_short_term_ref_pic_sets | ue(v) |
| --- | --- |
| for(idx = 0; idx < num_short_term_ref_pic_sets; idx++) |  |
| short_term_ref_pic_set( idx ) |  |
| if( num_short_term_ref_pic_sets ) { |  |
| sequence_description_flag | u(1) |
| if( sequence_description_flag ) { |  |
| num_sops_minus1 | ue(v) |
| for( sopIdx = 0; sopIdx <= num_sops_minus1; sopIdx++ ) |  |
| sop_description( sopIdx ) |  |
| num_sop_sequences_minus1 | ue(v) |
| for( i = 0; i <= num_sop_sequences_minus1; i++ ) { |  |
| sop_repetition_cnt_idc[ i ] | ue(v) |
| sop_idx[ i ] | ue(v) |
| } |  |
| } |  |
| } |  |

Add the following syntax structure:

- SOP description syntax

| sop_description( sopIdx ) { | Descriptor |
| --- | --- |
| num_pics_in_sop_minus1[ sopIdx ] | ue(v) |
| for( i = 0; i <= num_pics_in_sop_minus1; i++ ) { |  |
| nal_ref_flag[ sopIdx ][ i ] | u(1) |
| temporal_id[ sopIdx ][ i ] | u(3) |
| single_slice_type_flag[ sopIdx ][ i ] | u(1) |
| if( single_slice_type_flag[ sopIdx ][ i ] ) |  |
| slice_type[ sopIdx ][ i ] | ue(v) |
| poc_delta[ sopIdx ][ i ] | se(v) |
| st_rps_idx[ sopIdx ][ i ] | ue(v) |
| } |  |
| } |  |

Modify picture parameter set syntax by deleting the following indicated lines:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| … |  |
| } |  |

Modify slice header syntax by deleting the following strikethrough lines and adding the yellow-highlighted lines:

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| pic_parameter_set_id | ue(v) |
| if( IdrPicFlag ) { |  |
| idr_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } |  |
| else { |  |
| if( sequence_description_flag ) { |  |
| sop_idx | ue(v) |
| if( MaxPicIdx[ sop_idx ][ temporal_id ][ nal_ref_flag ] > 1 ) |  |
| pic_idx_in_sop | u(v) |
| } |  |
| if( !sequence_description_flag ¦ ¦ ( sequence_description_flag && !SingleSliceFlag[ sop_idx ][ pic_idx_in_sop ][ temporal_id ] [ nal_ref_flag ] ) ) |  |
| slice_type | ue(v) |
| if( !sequence_description_flag ) { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_pps_flag | u(1) |
| if( !short_term_ref_pic_set_pps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| delta_poc_lsb_lt_minus1[ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |

Add the following to sequence parameter set semantics:

num_short_term_ref_pic_sets specifies the number of short-term reference picture sets that are specified in the picture parameter set. The value of num_short_term_ref_pic_sets shall be in the range of 0 to 64, inclusive.

NOTE – A decoder must allocate space for a total number of num_short_term_ref_pic_sets + 1 short-term reference picture sets since a coded video sequence may contain up to one short-term reference picture set explicitly signalled in the slice headers of a current picture. An explicitly signalled short-term reference picture set will always have an index equal to num_short_term_ref_pic_sets in the list of short-term reference picture sets.

sequence_description_flag equal to 0 specifies that the coded video sequence is not described in the sequence parameter set. sequence_description_flag equal to 1 specifies that the coded video sequence is described in the sequence parameter set.

num_sops_minus1 + 1 specifies the number of SOPs described with the sop_description( ) syntax structure.

num_sop_sequences_minus1 + 1 specifies the number of SOP sequences described for the coded video sequence.

sop_repetition_cnt_idc[ i ] specifies the variable SopRepetitionCnt[ i ] as follows.

if( i  = =  num_sop_sequences_minus1 )
		SopRepetitionCnt[ i ] = sop_repetition_cnt_idc[ i ]
	else													(7-x)
		SopRepetitionCnt[ i ] = sop_repetition_cnt_idc[ i ] + 1

Variable SopRepetitionCnt[ i ] specifies the number of times the SOP with index sop_idx[ i ] is repeated in this SOP sequence. If SopRepetitionCnt[ i ] is 0, this SOP sequence is the last SOP sequence in the coded video sequence and the SOP with index sop_idx[ i ] may be repeated any number of times in this SOP sequence.

sop_idx[ i ] specifies the SOP index included in SOP sequence of index i.

Add the following SOP description semantics:

- SOP description semantics

This syntax structure has an input parameter sopIdx indicating the SOP index for the SOP described in the syntax structure.

num_pics_in_sop_minus1[ sopIdx ] + 1 specifies the number of pictures in the SOP.

nal_ref_flag[ sopIdx ][ i ] specifies the nal_ref_flag value of the i-th picture in decoding order within the SOP.

temporal_id[ sopIdx ][ i ] specifies the temporal_id value of the i-th picture in decoding order within the SOP.

poc_delta[ sopIdx ][ i ], when i is greater than 0, specifies the value of DiffPicOrderCnt( the i-th picture in decoding order within the SOP, the (i-1)th picture in decoding order within the SOP ). poc_delta[ sopIdx ][ i ], when i is equal to 0 and this SOP is not the first SOP of the coded video sequence, specifies the value of DiffPicOrderCnt( the i-th picture in decoding order within the SOP, the last picture in decoding in the previous SOP in the coded video sequence ). poc_delta[ sopIdx ][ i ], when i is equal to 0 and this SOP is the first SOP of the coded video sequence, specifies the value of DiffPicOrderCnt( the i-th picture in decoding order within the SOP, 0 ).

st_rps_idx[ sopIdx ][ i ] specifies the short-term reference picture set used by the i-th picture in decoding order within the SOP.

Variables PocInSOP[ sopIdx ][ picIdx ][ tId ][ nalRefFlag ], SingleSliceFlag[ sopIdx ][ picIdx ][ tId ][ nalRefFlag ], LastPocInSOP[ sopIdx ], and MaxPicIdx[ sopIdx ][ tId ][ nalRefFlag ], where tId and nalRefFlag are temporal_id and nal_ref_flag values appearing in the described SOP and picIdx is a picture index among pictures in the SOP having the particular temporal_id and nal_ref_flag values, are derived as follows. Initially, MaxPicIdx[ sopIdx ][ tId ][ nalRefFlag ] = 0 for all values of tId and nalRefFlag.

pocInSOP = 0
for( i = 0; i <= num_pics_in_sop_minus1; i++ ) {
	tId = temporal_id[ sopIdx ][ i ]
	nalRefFlag = nal_ref_flag[ sopIdx ][ i ]
	pocInSOP += poc_delta[ sopIdx ][ i ]
	PocInSOP[ sopIdx ][ MaxPicIdx[ sopIdx ][ tId ][ nalRefFlag ] ][ tId ][ nalRefFlag ] = pocInSOP
	SingleSliceFlag[ sopIdx ][ MaxPicIdx[ sopIdx ][ tId ][ nalRefFlag ] ][ tId ][ nalRefFlag ] = 
		single_slice_flag[ sopIdx ][ i ]
	LastPocInSOP[ sopIdx ] = pocInSOP
	MaxPicIdx[ sopIdx ][ tId ][ nalRefFlag ]++
}

NOTE – A bitstream may not contain all the pictures described in a SOP description. For example, the bitstream may have been subject to temporal_id based sub-bitstream extraction, while the sequence parameter set has not been modified to remove those pictures from SOP descriptions that have temporal_id values no longer existing in the extracted bitstream.

Remove num_short_term_ref_pic_sets semantics from picture parameter set semantics.

Add the following to the slice header semantics:

sop_idx specifies the SOP index of the SOP including the current picture.

pic_idx_in_sop specifies the picture index within the SOP identified by sop_idx among pictures in that SOP having the same temporal_id and nal_ref_flag values as the current picture. The length of the pic_idx_in_sop syntax element is Ceil( (Log2( MaxPicIdx[ sop_idx ][ temporal_id ][ nal_ref_flag ] ) ). If pic_idx_in_sop is not present, it is inferred to be 0.

Add the yellow-highlighted lines below to the semantics of picture order count:

- …

- If sequence_description_flag is equal to 0, the following applies.

- The variables prevPicOrderCntLsb and prevPicOrderCntMsb are derived as follows.

- If the current picture is an IDR picture, both prevPicOrderCntLsb and prevPicOrderCntMsb are set equal to 0.

–	Otherwise (the current picture is not an IDR picture), the following applies.

–	Let prevRefPic be the previous reference picture in decoding order that has temporal_id equal to 0. The variable prevPicOrderCntLsb is set equal to pic_order_cnt_lsb of prevRefPic, and the variable prevPicOrderCntMsb is set equal to PicOrderCntMsb of prevRefPic.

- PicOrderCntMsb of the current picture is derived as specified by the following pseudo-code:

if( ( pic_order_cnt_lsb <  prevPicOrderCntLsb )  &&
		( ( prevPicOrderCntLsb − pic_order_cnt_lsb )  >=  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb + MaxPicOrderCntLsb	(8-x)
else if( (pic_order_cnt_lsb  >  prevPicOrderCntLsb )  &&
		( (pic_order_cnt_lsb − prevPicOrderCntLsb )  >  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb − MaxPicOrderCntLsb
else
	PicOrderCntMsb = prevPicOrderCntMsb

- PicOrderCnt is derived as

PicOrderCnt = PicOrderCntMsb + pic_order_cnt_lsb		(8-x)

NOTE 1 – All IDR pictures will have PicOrderCnt equal to 0 since pic_order_lsb is inferred to be 0 for IDR pictures and prevPicOrderCntLsb and prevPicOrderCntMsb are both set equal to 0.

Otherwise (sequence_description_flag is equal to 1), the following applies.

- If the current picture is an IDR picture, PicOrderCnt is 0 and RefPicOrderCnt is 0.

- Otherwise, if temporal_id is equal to 0 and nal_ref_flag is equal to 1 and pic_idx_in_sop is equal to 0, PicOrderCnt = RefPicOrderCnt + PocInSOP[ sop_idx ][ pic_idx_in_sop ][ temporal_id ][ nal_ref_flag ] and RefPicOrderCnt += LastPocInSOP[ sop_idx ].

- Otherwise, 
PicOrderCnt = RefPicOrderCnt + PocInSOP[ sop_idx ][ pic_idx_in_sop ][ temporal_id ][ nal_ref_flag ]

- PicOrderCntLsb = ( PicOrderCnt & (MaxPicOrderCntLsb – 1 ) )

- PicOrderCntMsb = PicOrderCnt – PicOrderCntLsb

# References

JCTVC-D200	J. Boyce, D. Hong, A. Eleftheriadis, “High layer syntax to improve support for temporal scalability”.

JCTVC-E279	J. Boyce, D. Hong, S. Wenger, “Extensible High Layer Syntax for Scalability”.

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).