| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0423r2 |
| --- | --- |

| Title: | Indication of the temporal structure of coded video sequences |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Srikanth Manchenahally Gopalakrishna Nokia Research Center Visiokatu 1, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

The contribution returns to the topic of describing the temporal structure of coded video sequences, which was proposed earlier in documents JCTVC-D200 and JCTVC-E279. A primary use case for knowing the temporal structure a priori is to assist transraters and bitstream extractors in pruning the coded video sequence.

By referring to the reference picture set index, the proposal is asserted to provide a more accurate description of the temporal structure than what was proposed in JCTVC-D200 and JCTVC-E279.

Revision 2 of the document therefore proposes an SEI message where reference picture set index is included in the SOP description.

# Revision History

Revisions 0 and 1 of the proposal can be considered to consist of the following ingredients:

- A SOP description syntax structure is added. The syntax structure describes a structure of pictures (SOP) in decoding order.

- When the SOP description is used, the slice header includes a SOP index referring to a particular SOP description and a picture index within the SOP structure. Certain syntax elements of the slice header are excluded provided that the SOP description is used.

Revision 0 of the proposal also proposed the following sequence-level ingredients

- The short term reference picture set syntax structure is moved from PPS to SPS.

- A coded video sequence can be described as part of the SPS with reference to described SOPs.

# Motivation

## Motivation for Signaling of GOP/Sequence Structure (JCTVC-D200)

JCTVC-D200 proposed a coding structure SEI message among other things. The motivation for proposing the SEI message was given in the introduction of Section 4 of JCTVC-D200 as copied into the following paragraphs.

Although the current HEVC design and the H.264/AVC standard allow for flexible patterns of coded pictures, many encoders, including the JM and HM reference software, generate sequences of coded pictures following a fixed pattern.   Even when a fixed pattern is in used, gateways and decoders are not aware of it.

Knowing the coding structure a priori can help in dropping frames for transraters and bitstream extractors.  Additionally, transraters can optimally choose frames to transcode and minimize drift.  DVRs can also utilize the coding structure information for operations like fast forwarding.  Finally, knowing the structure can enable decoders to process certain frames in parallel.  For example, for the hierarchical P picture coding structure shown below, pictures P1 and P2 can be processed in parallel, then P3 and P4, and so on.

An encoder generates a sequence of coded pictures that consists of one or more GOPs (Group of Pictures).  Here, a GOP is defined as a sequence of coded pictures that start with an I/IDR picture and ends with a picture immediately preceding the next I/IDR picture.  Many encoders generate one or more GOPs of the same structure of coded pictures.  Furthermore, each GOP generated by these encoders typically consists of a smaller fixed pattern that repeats.  The coding structure SEI message describes this smaller, repeating, fixed pattern of a temporal picture coding structure that is used by the encoder.  For example, in the figure shown below, the 3 B picture structure repeats between I and P pictures; this SEI message can be used to describe the repeating structure.  Hereinafter, the first lowest temporal layer picture followed by the repeating set of pictures (in coding order) are referred to as an SOP (Structure of Pictures).  A GOP (Group of Pictures) begins with an I frame, and includes one or more SOPs.  In the example below, there are 4 pictures in the SOP and 3 SOPs in the GOP: the first SOP consists of the first I picture followed by the 3 immediately following B pictures, the second SOP consists of the first P picture followed by the next 3 B pictures, and so on.

## Motivation for This Proposal

JCTVC-D200 proposed an SEI message for coding/temporal structure signaling. The proposed message syntax was later updated in JCTVC-E279r3 and copied below:

| temporal_structure( payloadSize ) { | Descriptor |
| --- | --- |
| num_active_temporal_layers_minus1 | u(3) |
| coding_structure_flag | u(1) |
| if( coding_structure_flag ) { |  |
| num_pictures_in_sop_minus1 | ue(v) |
| num_sops_in_gop | ue(v) |
| for( i = 0; i < num_pictures_in_sop_minus1 + 1; i++ ) { |  |
| primary_pic_type[ i ] | u(2) |
| ref_flag[ i ] | u(1) |
| temporal_num[ i ] | u(3) |
| display_num[ i ] | ue(v) |
| } |  |
| } |  |
| coding_statistics_flag | u(1) |
| if( coding_statistics_flag ) { |  |
| average_frame_rate_flag | u(1) |
| average_bit_rate_flag | u(1) |
| if( average_frame_rate_flag ) |  |
| average_frame_rate | u(16) |
| if( average_bit_rate_flag ) |  |
| for( i = 0; i < NumTemporalLayers; i++ ) |  |
| average_bit_rate[ i ] | u(16) |
| } |  |
| } |  |

# Proposed Changes to Specification Text

Add the following definitions:

structure of pictures: One of more coded pictures consecutive in decoding order, in which the first coded picture in decoding order has temporal_id equal to 0 and nal_ref_flag equal to 1

Add the following to abbreviations:

SOP	Structure of Pictures

- SOP description SEI message syntax

| sop_description( payloadSize ) { | Descriptor |
| --- | --- |
| pps_id /* or sps_id, if RPS is moved to SPS */ | ue(v) |
| num_pics_in_sop_minus1 | ue(v) |
| for( i = 0; i <= num_pics_in_sop_minus1; i++ ) { |  |
| nal_ref_flag[ i ] | u(1) |
| temporal_id[ i ] | u(3) |
| st_rps_idx[ i ] | ue(v) |
| if( i > 0 ) |  |
| poc_delta[ i ] | se(v) |
| } |  |
| } |  |

- SOP description SEI message semantics

The SOP description SEI message indicates constraints that shall be used in the SOP that starts from the access unit where the SOP description SEI message resides.

The SOP description SEI message shall not be present in access units with temporal_id greater than 0.

pps_id specifies the pic_parameter_set_id value identifying a picture parameter set.

num_pics_in_sop_minus1 + 1 specifies the number of pictures in the SOP.

nal_ref_flag[ i ] specifies the nal_ref_flag value of the i-th picture in decoding order within the SOP.

temporal_id[ i ] specifies the temporal_id value of the i-th picture in decoding order within the SOP.

st_rps_idx[ i ] specifies the short-term reference picture set included in the picture parameter set identified by pps_id and used by the i-th picture in decoding order within the SOP.

poc_delta[ i ] specifies the value of DiffPicOrderCnt( the i-th picture in decoding order within the SOP, the (i-1)th picture in decoding order within the SOP ).

NOTE 1 – A bitstream may not contain all the pictures described in a SOP description. For example, the bitstream may have been subject to temporal_id based sub-bitstream extraction, while the sequence parameter set has not been modified to remove those pictures from SOP descriptions that have temporal_id values no longer existing in the extracted bitstream. The following constraints specify that pictures that are present in the bitstream must match to the information given in this SEI message.

NOTE 2 – As indicated in the constraints below, the used short-term reference picture sets must be identical in active parameter sets compared to the respective short-term reference picture sets in the picture parameter set with the same pps_id that the SEI can access at the time of its parsing.

Variable picOrderCntExp[ i ] is derived as follows.

picOrderCntExp[ 0 ] = PicOrderCntVal of the current picture
for( i = 1; i <= num_pics_in_sop_minus1; i++ ) {
	picOrderCntExp[ i ] = picOrderCntExp[ i – 1 ] + poc_delta[ i ]
}

Let ppsId[ j ], nalRefFlag[ j ], tId[ j ], stRpsIdx[ j ], and picOrderCntVal[ j ] be the values of pic_parameter_set_id, nal_ref_flag, temporal_id, st_rps_idx, and PicOrderCntVal that are in effect for the j-th picture in decoding order starting from j equal to 0 indicating the first picture of the SOP where this SEI message resides.

Let currPicParamSet be the previous pic_parameter_set_rbsp with pic_parameter_set_id equal to pps_id in decoding order. Let activePicParamSet[ j ] be the active pic_parameter_set_rbsp in the j-th picture in decoding order starting from j equal to 0 indicating the first picture of the SOP where this SEI message resides. Let currNumShortTermRefPicSets be num_short_term_ref_pic_sets and currShortTermRefPicSet[ idx ] be short_term_ref_pic_set( idx ), for all values of idx from 0 to currNumShortTermRefPicSets – 1, inclusive, in currPicParamSet. Let activeNumShortTermRefPicSets[ j ] be num_short_term_ref_pic_sets and activeShortTermRefPicSet[ j ][ idx ] be short_term_ref_pic_set( idx ), for all values of idx from 0 to activeNumShortTermRefPicSets[ j ] – 1, inclusive, in activePicParamSet[ j ].

The following shall apply:

for( i = 0, j = 0; i <= num_pics_in_sop_minus1; i++) {
	if( picOrderCntExp[ i ]  = =  picOrderCntVal[ j ] ) {
		nalRefFlag[ j ] shall be equal to nal_ref_flag[ i ]
		tId[ j ] shall be equal to temporal_id[ i ]
		stRpsIdx[ j ] shall be equal to st_rps_idx[ i ]
		activeShortTermRefPicSet[ j ][ stRpsIdx[ j ] ] shall be present and equal to 
			currShortTermRefPicSet[ stRpsIdx[ j ] ]
		long_term_ref_pics_present_flag in activePicParamSet[ j ] shall be equal to 0
		j++
	}
}
picOrderCntVal[ j – 1 ] shall be less than or equal to picOrderCntExp[ num_pics_in_sop_minus1 ]

# References

JCTVC-D200	J. Boyce, D. Hong, A. Eleftheriadis, “High layer syntax to improve support for temporal scalability”.

JCTVC-E279	J. Boyce, D. Hong, S. Wenger, “Extensible High Layer Syntax for Scalability”.

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).