| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0424 |
| --- | --- |

| Title: | Non-CE10 Subtest 4: On Deblocking Parameter Signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera, Ye-Kui Wang, Xianglin Wang, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 | Email: | geertv@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to add a condition in the slice header that checks the deblocking_filter_in_aps_enabled_flag to determine whether deblocking parameters are signalled in the APS and in only that case it is proposed to include the inherit_dbl_params_from_APS_flag in the slice header. The benefit is that the signalling overhead of the inherit_dbl_params_from_APS_flag is avoided in case deblocking parameters are only signalled in the slice header. In addition, this contribution addresses a minor mismatch between the working draft text and the HM5.1 source code, and one potential cleanup for the deblocking filter source code is suggested.

# Introduction

The Adaptation Parameter Set (APS) was adopted during the 6th JCT-VC meeting (July 2011) in Torino, Italy [1]. The APS is a picture layer parameter set intended for use with picture-adaptive data; however it is independent of the PPS. The PPS contains data that is unlikely to change from picture to picture. In the working draft [2], the APS includes parameters associated with the Adaptive Loop Filter (ALF) and Sample Adaptive Offset (SAO). As opposed to including these parameters in the PPS, separating the PPS and APS saves the transmission of bits, because the constant PPS data does not need to be repeated when the ALF or SAO parameters change.

During the 7th JCT-VC meeting (Nov. 2011) in Geneva, Switzerland, the signaling of deblocking filter adjustment parameters was adopted [3]. This includes the signaling of the disable_deblocking_filter_flag and deblocking parameter offsets for tC and β in the APS and/or the slice header. The slice header contains a flag indicating to “inherit” the deblocking parameters from the APS or to use the provided parameters in the slice header. The Tables 1-3 are based on [3] and specify the deblocking parameter changes (highlighted) in the SPS, APS and slice header.

Table 1 Sequence Parameter Set (SPS) syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| ..... |  |
| chroma_pred_from_luma_enabled_flag | u(1) |
| deblocking_filter_in_aps_enabled_flag | u(1) |
| Note that this flag is not included yet in WD5 [2], but adopted from [3] |  |
| loop_filter_across_slice_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| pcm_loop_filter_disable_flag | u(1) |
| cu_qp_delta_enabled_flag | u(1) |
| temporal_id_nesting_flag | u(1) |
| inter_4x4_enabled_flag | u(1) |
| .... |  |
| rbsp_trailing_bits( ) |  |
| } |  |

Table 2 Adaptation Parameter Set (APS) syntax

| aps_rbsp( ) { | Descriptor |
| --- | --- |
| aps_id | ue(v) |
| aps_sample_adaptive_offset_flag | u(1) |
| aps_adaptive_loop_filter_flag | u(1) |
| aps_deblocking_filter_flag | u(1) |
| ..... |  |
| if( aps_sample_adaptive_offset_flag ) { |  |
| ..... |  |
| } |  |
| if( aps_adaptive_loop_filter_flag ) { |  |
| ..... |  |
| } |  |
| if (aps_deblocking_filter_flag) { |  |
| disable_deblocking_filter_flag | u(1) |
| if ( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

Table 3 Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| if( sample_adaptive_offset_enabled_flag ¦¦ adaptive_loop_filter_enabled_flag ) |  |
| aps_id | ue(v) |
| .... |  |
| if( !entropy_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| inherit_dbl_params_from_APS_flag | u(1) |
| if ( !inherit_dbl_params_from_APS_flag ) { |  |
| disable_deblocking_filter_flag | u(1) |
| if ( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| ..... |  |
| } |  |

deblocking_filter_in_aps_enabled_flag: value 0 means that deblocking parameters are present in the slice header; value 1 means that deblocking parameters are present in the APS

aps_deblocking_filter_flag: indicates that deblocking parameters are present in the APS (value 1) or not present (value 0)

disable_deblocking_filter_flag: value 0 means that the deblocking filter is enabled; value 1 means deblocking filter is disabled

beta_offset_div2 and tc_offset_div2: deblocking parameter offsets for tC and β (divided by 2)

inherit_dbl_params_from_APS_flag: value 1 means that deblocking parameters from the APS shall be used for deblocking the current slice; value 0 means that deblocking parameters that follow in the slice header shall be used

# Proposals

## Check deblocking_filter_in_aps_enabled_flag before signaling inherit_dbl_params_from_APS_flag in slice header

To avoid overhead in the slice header for the inherit_dbl_params_from_APS_flag if deblocking_filter_in_aps_enabled_flag equals 0 (deblocking adjustment parameters are present in slice header), it is proposed to add a condition that checks this flag first (see Table 4 for working draft changes).

Remark: The name of the syntax element inherit_dbl_params_from_APS_flag may be changed to something like dbl_params_present_in_slice_flag (value 1 means that parameters are present in the slice header with priority over the APS deblocking parameters).

Table 4 Working draft changes for proposal 2.1 in blue highlight

| slice_header( ) { | Descriptor |
| --- | --- |
| .... | u(v) |
| if( !entropy_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if (deblocking_filter_in_aps_enabled_flag) { |  |
| inherit_dbl_params_from_APS_flag | u(1) |
| Note that inherit_dbl_params_from_APS_flag = 0 is inferred if deblocking_filter_in_aps_enabled_flag = 0. |  |
| } |  |
| if (!inherit_dbl_params_from_APS_flag){ |  |
| disable_deblocking_filter_flag | u(1) |
| if (!disable_deblocking_filter_flag) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| ..... |  |

## Correspondence issue between slice header syntax description and HM5.1 source code

There is a minor correspondence issue between the HM5.1 source code and the slice header syntax description [2]: before signaling the aps_id in the slice header the deblocking_filter_in_aps_enabled_flag is checked in HM5.1. The slice header syntax addition in Table 5 fixes this correspondence issue.

Table 5 Working draft changes for section 2.2

| slice_header( ) { | Descriptor |
| --- | --- |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| if( sample_adaptive_offset_enabled_flag ¦¦ adaptive_loop_filter_enabled_flag ¦¦ deblocking_filter_in_aps_enabled_flag ) |  |
| aps_id | ue(v) |
| frame_num | u(v) |
| if( IdrPicFlag ) |  |
| idr_pic_id | ue(v) |
| .... |  |

## Deblocking Filter Source Code Cleanup

In the HM5 source code, the deblocking filter will check on/off decisions and strong/weak decisions for luma edges even when the β and/or tC thresholds are zero. It is proposed to introduce a condition to check whether both thresholds are larger than zero before proceeding with checking the deblocking conditions. If β equals 0, then the on/off decision will always be false and the luma edge will not be deblocking filtered. If β is greater than 0 and tC equals 0 then the strong/weak filter decision will result in selecting the weak filter, however, if tC equals 0 then the natural edge condition of the weak filter will always be false and weak filtering will not be performed. Similarly for a chroma edge, if tC equals 0 then the chroma edge filtering does not need to be performed, because the chroma filter delta will be clipped to 0.

The working draft changes are included in section 6.

# Results

The deblocking filter source code cleanup from section 2.3 is tested by successfully decoding the HM5.0 anchor bitstreams.

The BD-rate results for section 2.1 follow the test conditions outlined in JCTVC-G1200, however, the anchor is generated using the software that was released in the context of CE10 Subtest 4 by Toshiba on 1/9/12. The change from sections 2.1 is integrated into this software version. Table 6 enumerates the BD-rate results, which are virtually identical to the anchor. The execution timings are measured in a heterogeneous computing environment and are approximate.

Table 6 BD-rate results

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 102% |  |  | 102% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 102% |  |  | 99% |  |  | 97% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 102% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |

# Conclusion

This contribution proposed to add a condition to the slice header syntax, checking the deblocking_filter_in_aps_enabled_flag (SPS) to determine whether the inherit_dbl_params_from_APS_flag should be present in the slice header. In case the deblocking filter adjustment parameters are only signaled in the slice header, then it is not useful to include the inherit_dbl_params_from_APS_flag in the slice header. This contribution additionally addresses a minor correspondence issue between the slice header syntax and the HM5.1 source code, as well as a suggestion to clean up the deblocking filter source code.

# References

[1] S. Wenger, J. Boyce, Y.-W. Huang, C.-Y. Tsai, P. Wu, M. Li, “Adaptation Parameter Set (APS),” 6th JCT-VC Meeting, Torino, Italy, July 2011, Doc. JCTVC-F747

[2] B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “WD5: Working Draft 5 of High-Efficiency Video Coding,” 7th JCT-VC Meeting, Geneva, CH, Nov. 2011, Doc. JCT-VC G1103_d6

[3] A. Norkin, “BoG report on resolving deblocking filter description issues,” 7th JCT-VC Meeting, Geneva, CH, Nov. 2011, Doc. JCT-VC G1035_r1 + update on 12/18/2011 in email from Tomoo Yamakage (Toshiba) on CE10 reflector

# Working Draft Text (Adopted Part)

The following text is based on WD5 (JCTVC-G1103_d9). It covers the working draft changes from the adopted parts of JCTVC-H0424 (this document) and also JCTVC-H0398, since both are related to the signalling of deblocking adjustment parameters.

The deblocking_filter_control_present_flag proposed in JCTVC-H0398 is included in the PPS as follows (yellow highlights):

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| num_short_term_ref_pic_sets | ue(v) |
| for(idx = 0; idx < num_short_term_ref_pic_sets; idx++) |  |
| short_term_ref_pic_set( idx ) |  |
| long_term_ref_pics_present_flag | u(1) |
| entropy_coding_synchro | u(v) |
| cabac_istate_reset_flag | u(1) |
| if( entropy_coding_synchro ) |  |
| num_substreams_minus1 | ue(v) |
| num_temporal_layer_switching_point_flags | ue(v) |
| for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ) |  |
| temporal_layer_switching_point_flag[ i ] | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| [Ed. (BB): not present in HM software ] |  |
| pic_init_qp_minus26 | se(v) |
| constrained_intra_pred_flag | u(1) |
| enable_temporal_mvp_flag | u(1) |
| slice_granularity | u(2) |
| max_cu_qp_delta_depth | ue(v) |
| chroma_cb_qp_offset | se(v) |
| chroma_cr_qp_offset | se(v) |
| weighted_pred_flag | u(1) |
| weighted_bipred_idc | u(2) |
| tile_info_present_flag | u(1) |
| tile_control_present_flag | u(1) |
| if( tile_info_present_flag = = 1 ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| if( num_tile_columns_minus1 != 0 ¦ ¦ num_tile_rows_minus1 != 0 ) { |  |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[i] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[i] | ue(v) |
| } |  |
| } |  |
| } |  |
| if ( tile_control_present_flag ) { |  |
| if ( num_tile_columns_minus1 != 0 ¦ ¦ num_tile_rows_minus1 != 0 ) { |  |
| tile_boundary_independence_flag | u(1) |
| if ( tile_boundary_independence_flag = = 1 ) |  |
| loop_filter_across_tile_flag | u(1) |
| } |  |
| } |  |
| deblocking_filter_control_present_flag | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

Semantics (copied from Recommendation ITU-T H.264 / AVC):

deblocking_filter_control_present_flag equal to 1 specifies that a set of syntax elements controlling the characteristics of the deblocking filter is present in the slice header. deblocking_filter_control_present_flag equal to 0 specifies that the set of syntax elements controlling the characteristics of the deblocking filter is not present in the slice headers and their inferred values are in effect.

The deblocking_filter_control_present_flag proposed in JCTVC-H0398 is applied in the slice header as follows (yellow highlights). The adopted condition from JCTVC-H0424 (this document) is also included (green highlight).

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag = = 0 ) |  |
| slice_address | u(v) |
| slice_type | ue(v) |
| entropy_slice_flag | u(1) |
| if( !entropy_slice_flag ) { |  |
| pic_parameter_set_id | ue(v) |
| if( IdrPicFlag ) { |  |
| idr_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } |  |
| else { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_pps_flag | u(1) |
| if( !short_term_ref_pic_set_pps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| delta_poc_lsb_lt_minus1[ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( scaling_list_enable_flag ¦ ¦ deblocking_filter_in_APS_enabled_flag ¦ ¦ sample_adaptive_offset_enabled_flag ¦ ¦ adaptive_loop_filter_enabled_flag ) { |  |
| if( sample_adaptive_offset_enabled_flag ) |  |
| slice_sample_adaptive_offset_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag) |  |
| slice_adaptive_loop_filter_flag | u(1) |
| aps_id | ue(v) |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| } |  |
| if( slice_type != I ) |  |
| cabac_init_idc | ue(v) |
| if( !entropy_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if (deblocking_filter_control_present_flag) { |  |
| if (deblocking_filter_in_aps_enabled_flag) { |  |
| inherit_dbl_params_from_APS_flag | u(1) |
| Note: inherit_dbl_params_from_aps_flag is inferred 0 when not present |  |
| } |  |
| if ( !inherit_dbl_params_from_APS_flag ) { |  |
| disable_deblocking_filter_flag | u(1) |
| if ( !disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) |  |
| 5_minus_max_num_merge_cand | ue(v) |
| if( adaptive_loop_filter_enabled_flag && aps_adaptive_loop_filter_flag ) |  |
| alf_cu_control_param( ) |  |
| for( i = 0; i < num_substreams_minus1 + 1; i++ ){ |  |
| substream_length_mode | u(2) |
| substream_length[i] | u(v) |
| } |  |
| } |  |

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).