| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0425 |
| --- | --- |

| Title: | POC granularity for reference picture sets and slice header |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Srikanth Manchenahally Gopalakrishna Nokia Research Center Visiokatu 1, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

It is asserted that encoders may choose to keep the POC value difference of any two consequent pictures in output order having the same temporal_id remains unchanged. For example, encoders may use a regular GOP pattern and assign POC values proportional to a constant picture sampling interval. It is therefore proposed that a POC granularity value is provided for each temporal_id in the sequence parameter set. The POC granularity value for a particular temporal_id indicates the POC interval between any two consecutive pictures in output order having that temporal_id. It is noted that POC granularity value equal to 1 can be indicated for bitstreams having an irregular POC assignment pattern.

This contribution also proposes to use the temporal_id-based POC granularity for POC difference coding in reference picture sets and POC value signalling in slice headers.

# Background

JCTVC-G643 proposed describing reference picture list 0 and 1 directly instead of reference picture sets and potential subsequent reference picture list modification. In order to describe reference picture lists efficiently JCTVC-G643 proposed temporal_id-based POC granularity for POC difference coding. While it was stated in JCTVC-G643 that temporal_id-based POC granularity for POC difference coding could be used for reference picture sets, no specification text was provided. Hence, this contribution proposes temporal_id-based POC granularity for POC difference coding for reference picture sets and POC signalling in slice headers. Specification text relative to the latest available reference picture set related specification text is provided.

# Bit Count Overhead

The bit count overhead relative to the common conditions specified in JCTVC-G1036 will be provided in this section in a later version of this document.

# Proposed Changes to Specification Text

The following sub-sections provide the proposed changes to specification text categorized according to the syntax structure or decoding process. The changes are relative to the latest adopted specification text as far as the authors have been aware of – that is a combination of JCTVC-G1002-output-WD-text-r1 for most parts, JCTVC-G192r2 for the inter-RPS prediction, and JCTVC-F803_d5_RPS_r2.doc, which was released as part of JCTVC-G021-v2.zip, for the decoding process of picture order count.

The following sub-sections are arranged as follows:

- First, a short motivation and explanation of the changes is provided.

- Then, in enumerated italics, the change itself is introduced (as an “MPEG amendment style” of an instruction).

- Finally, the change is given with change-marked text, or if a whole section was added, as plain text.

## Sequence Parameter Set

The POC granularity is provided for each temporal_id.

- Add the following syntax to the sequence parameter set:

| for( i = 0; i <= max_temporal_layers_minus1; i++ ) |  |
| --- | --- |
| poc_granularity_minus1[ i ] | ue(v) |
| if( max_temporal_layers_minus1 = = 0 ) |  |
| ref_pic_poc_granularity_delta | ue(v) |

- Add the following to the sequence parameter set RBSP semantics:

poc_granularity_minus1[ i ] + 1 specifies that the POC value interval of any two consecutive pictures in output order having temporal_id equal to or smaller than i shall be j * (poc_granularity_minus1[ i ] + 1), where j is a positive integer. (poc_granularity_minus1[ i ] + 1) shall be equal to k * (poc_granularity_minus1[ i + 1 ] + 1), where k is a positive integer and i is greater than 0.

ref_pic_poc_granularity_delta specifies that, when all pictures have temporal_id equal to 0, the POC value difference of any two consecutive reference pictures in output order shall be an integer multiple of ( poc_granularity_minus1[ 0 ] + ref_pic_poc_granularity_delta + 1 ).

## Picture Parameter Set

When a picture having temporal_id equal to TID does not use any other picture having the same temporal_id as reference, a POC granularity governed by TID is required for the POC difference between the POC values of the current picture and the closest reference picture in terms of POC, whereas POC granularity governed by TID – 1 may be used for the other reference pictures. The tid_for_rps_pred_offset syntax element is used to control the temporal_id values to which this method can be used.

- Add the following syntax to the picture parameter set:

| tid_for_rps_pred_offset | u(3) |
| --- | --- |

- Add the following to the picture parameter set RBSP semantics:

tid_for_rps_pred_offset specifies the lowest temporal_id value from which, if non-zero, 1 is subtracted in the decoding process of reference picture set when determining the temporal_id based POC granularity to be used for the prediction value of the picture order count corresponding to delta_poc_s0[ í ] and delta_poc_s1[ i ], when i is greater than 0.

NOTE – tid_for_rps_pred_offset is typically the lowest temporal_id value TID such that no picture having temporal_id greater than or equal to TID uses another picture having the same temporal_id value as reference picture in any reference picture list.

## Reference Picture Set

The differential POC values are now derived solely in the decoding process for the reference picture set. Hence, a part of the derivation was removed from the semantics.

The following remarks can be made about the reference picture set decoding process:

- When no inter-RPS prediction is used, the POC granularity to be used is first derived. Then, signed POC difference (DeltaPoc) relative to the current POC is derived. Otherwise, the short-term reference picture set construction is unchanged.

- When inter-RPS prediction is used, the prediction is performed in final POC values. No essential change in was done in the construction process except that a description of the process was moved from the semantics as part of the decoding process.

- POC granularity is used also in the long-term reference picture set construction.

- Change short-term reference picture set semantics as follows

…

…

The variable DeltaPocS0[ idx ][ i ] is derived as follows.

DeltaPocS0[ idx ][ i ] = delta_poc_s0_minus1[ i ] + 1	
						-

…

The variable DeltaPocS1[ idx ][ i ] is derived as follows.

DeltaPocS1[ idx ][ i ] = delta_poc_s1_minus1[ i ] + 1	
							-

…

- Change the decoding process for reference picture set as follows

This process is invoked once per picture, after decoding of a slice header but prior to the decoding of any coding unit and prior to the decoding process for reference picture list construction of the slice as specified in subclause Error: Reference source not found. The process may result in marking one or more reference pictures as "unused for reference".

The reference picture set consists of six subsets, namely RefPicSetStCurr0, RefPicSetStCurr1, RefPicSetStFoll0, RefPicSetStFoll1, RefPicSetLtCurr, and RefPicSetLtFoll. Each subset consists of a list of picture order count values. The number of entries in RefPicSetStCurr0, RefPicSetStCurr1, RefPicSetStFoll0, RefPicSetStFoll1, RefPicSetLtCurr, and RefPicSetLtFoll are NumRpsStCurr0, NumRpsStCurr1, NumRpsStFoll0, NumRpsStFoll1, NumRpsLtCurr, and NumRpsLtFoll respectively.

RefPicSetStCurr0, RefPicSetStCurr1, RefPicSetStFoll0 and RefPicSetStFoll1 are collectively referred to as the short-term subset of the reference picture set. RefPicSetLtCurr and RefPicSetLtFoll are collectively referred to as the long-term subset of the reference picture set.

If the current picture is an IDR picture, RefPicSetStCurr0, RefPicSetStCurr1, RefPicSetStFoll0, RefPicSetStFoll1, RefPicSetLtCurr, and RefPicSetLtFoll are all set to empty, and NumRpsStCurr0, NumRpsStCurr1, NumRpsStFoll0, NumRpsStFoll1, NumRpsLtCurr, and NumRpsLtFoll are all set to 0.

Otherwise, the following applies for derivation of the six subsets and the numbers of entries.

When inter_ref_pic_set_prediction_flag is equal to 0, the following applies

if( temporal_id >= tid_for_rps_pred_offset  &&  temporal_id > 0 )
	granTID = temporal_id - 1
else
	granTID = temporal_id

for( i = 0, j = 0, k = 0; i < NumNegativePics[ StRpsIdx ] ; i++ ) {
	if( ( i > 0 || nal_ref_flag > 0 )  &&  max_num_temporal_layers_minus1 = = 0 )
		absDeltaPocPred = poc_granularity_minus1[ 0 ] + ref_pic_poc_granularity_delta + 1
	else if( i  = =  0)
		absDeltaPocPred = poc_granularity_minus1[ temporal_id ] + 1
	else
		absDeltaPocPred = poc_granularity_minus1[ granTID ] + 1
	
	if( i  = =  0)
		DeltaPoc[ StRpsIdx ][ i ] = -absDeltaPocPred - DeltaPocS0[ StRpsIdx ][ i ] * absDeltaPocPred
	else
		DeltaPoc[ StRpsIdx ][ i ] = DeltaPoc[ StRpsIdx ][ i – 1 ] – absDeltaPocPred – 
			DeltaPocS0[ StRpsIdx ][ i ] * absDeltaPocPred
	if( UsedByCurrPicS0[ StRpsIdx ][ i ] )
		RefPicSetStCurr0[ j++ ] = PicOrderCnt + DeltaPoc[ StRpsIdx ][ i ]
	else
		RefPicSetStFoll0[ k++ ] = PicOrderCnt + DeltaPoc[ StRpsIdx ][ i ]
}
NumRpsStCurr0 = j
NumRpsStFoll0 = k

for( i = 0, j = 0, k = 0, m = NumNegativePics[ StRpsIdx ]; i < NumPositivePics[ StRpsIdx ]; i++, m++ ) {
	if( ( i > 0 || nal_ref_flag > 0 )  &&  max_num_temporal_layers_minus1 = = 0 )
		absDeltaPocPred = poc_granularity_minus1[ 0 ] + ref_pic_poc_granularity_delta + 1
	else if( i  = =  0)
		absDeltaPocPred = poc_granularity_minus1[ temporal_id ] + 1
	else
		absDeltaPocPred = poc_granularity_minus1[ granTID ] + 1
	if( i  = =  0)
		DeltaPoc[ StRpsIdx ][ m ] = absDeltaPocPred + DeltaPocS1[ StRpsIdx ][ i ] * absDeltaPocPred
	else
		DeltaPoc[ StRpsIdx ][ m ] = DeltaPoc[ StRpsIdx ][ m – 1 ] + absDeltaPocPred + 
			DeltaPocS1[ StRpsIdx ][ i ] * absDeltaPocPred
	if( UsedByCurrPicS1[ StRpsIdx ][ i ] )
		RefPicSetStCurr1[ j++ ] = PicOrderCnt + DeltaPoc [ StRpsIdx ][ m ]
	else
		RefPicSetStFoll1[ k++ ] = PicOrderCnt + DeltaPoc [ StRpsIdx ][ m ]
}
NumRpsStCurr1 = j
NumRpsStFoll1 = k		(8-x)

When inter_ref_pic_set_prediction_flag is equal to 1, the following applies.

for( j = 0, i0 = 0, i1 = 0, k0 = 0, j0 = 0, k1 = 0, j1 = 0, m = 0; j <= NumDeltaPocs[ RIdx ]; j++ ) {
	if( ref_idc[ j ] = = “1” || ref_idc[ j ] = = “01” ) {
		DPoc = DeltaPoc[ StRpsIdx ][ m ] = DeltaPoc[ RIdx ][ j ] + DeltaRPS
		UsedByCurrPic = (ref_idc[ j ] = = “1”) ? 1 : 0
		if( DPoc < 0 ) {
			if( UsedByCurrPic )
			   RefPicSetStCurr0[ j0++ ] = PicOrderCnt + DPoc
			else 
			   RefPicSetStFoll0[ k0++ ] = PicOrderCnt + DPoc
			
			UsedByCurrPicS0[ StRpsIdx ][ i0 ] = UsedByCurrPic
			i0++
		}
		else {
			if( UsedByCurrPic )
			   RefPicSetStCurr1[ j1++ ] = PicOrderCnt + DPoc
			else 
			   RefPicSetStFoll1[ k1++ ] = PicOrderCnt + DPoc
			
			UsedByCurrPicS1[ StRpsIdx ][ i1 ] = UsedByCurrPic
			i1++
		}
			
		m++
	}
}
NumNegativePics[ StRpsIdx ] = i0
NumPositivePics[ StRpsIdx ] = i1

When DeltaPoc[ RIdx ][ j ] is unavailable, it is set to 0.

Sort DeltaPocS0[ StRpsIdx ][ i0 ] and DeltaPocS1[ StRpsIdx ][ i1 ] in decreasing order and in increasing order, respectively.

The variable NumDeltaPocs[ StRpsIdx ] is derived as follows.

NumDeltaPocs[ StRpsIdx ] = NumNegativePics[ StRpsIdx ] + NumPositivePics[ StRpsIdx ]

The long-term subset of the reference picture set is derived as follows.

for( i = 0, j = 0, k = 0; i < num_long_term_pics; i++ ) {
	if( ( i > 0 || nal_ref_flag > 0 )  &&  max_num_temporal_layers_minus1 = = 0 )
		deltaPocPred = poc_granularity_minus1[ 0 ] + ref_pic_poc_granularity_delta + 1
	else if( i  = =  0)
		deltaPocPred = poc_granularity_minus1[ temporal_id ] + 1
	else
		deltaPocPred = poc_granularity_minus1[ granTID ] + 1
	deltaPoc = deltaPocPred + DeltaPocLt[ i ] * deltaPocPred
	if( used_by_curr_pic_lt_flag[ i ] )
		RefPicSetLtCurr[ j++ ] = ( PicOrderCnt − deltaPoc + MaxPicOrderCntLsb ) % 
			MaxPicOrderCntLsb
	else
		RefPicSetLtFoll[ k++ ] = ( PicOrderCnt − deltaPoc + MaxPicOrderCntLsb ) %
			MaxPicOrderCntLsb
}
NumRpsLtCurr = j
NumRpsLtFoll = k

…

## Slice Header

pic_order_cnt_lsb is changed to pic_order_cnt_cycle, which is coded at the POC granularity governed by the temporal_id value. A shorter codeword can be used for pic_order_cnt_cycle compared than for pic_order_cnt_lsb to achieve the same loss robustness.

The POC granularity can also be used for long-term pictures, as a long-term picture having temporal_id TID can only be referred to from a picture having temporal_ID greater than or equal to TID.

- Rename pic_order_cnt_lsb to pic_order_cnt_cycle in the slice header syntax.

- Change slice header semantics as follows:

When present, the value of the slice header syntax elements pic_parameter_set_id, idr_pic_id no_output_of_prior_pics_flag, pic_order_cnt_cycle, short_term_ref_pic_set_pps_flag, short_term_ref_pic_set_idx and num_long_term_pics shall be the same in all slice headers of a coded picture. When present, the value of the slice header syntax elements delta_poc_lsb_lt_minus1[ i ] and used_by_curr_pic_lt_flag[ i ] shall be the same in all slice headers of a coded picture for each i in the range of 0 to num_long_term_pics, inclusive.

…

pic_order_cnt_cycle specifies the picture order count for the current picture as specified in 8.2.1. The length of the pic_order_cnt_cycle syntax element is Ceil( Log2 ( MaxPicOrderCntLsb / poc_granularity[ temporal_id ] + ( ( MaxPicOrderCntLsb % poc_granularity[ temporal_id ] ) > 0 ) ) ) bits. The value of the pic_order_cnt_cycle shall be in the range of 0 to MaxPicOrderCntLsb / poc_granularity[ temporal_id ] – ( ( MaxPicOrderCntLsb % poc_granularity[ temporal_id ] )  = =  0 ), inclusive. When pic_order_cnt_cycle is not present, pic_order_cnt_cycle shall be inferred to be equal to 0.

…

delta_poc_lsb_lt_minus1[ i ] is used to determine the value of the least significant bits of the picture order count value of the i-th long-term reference picture that is included in the long-term reference picture set of the current picture. delta_poc_lsb_lt_minus1[ i ] shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive.

The variable DeltaPocLt[ i ] is derived as follows.

if (i  = =  0) 
		DeltaPocLt[ i ] = ( delta_poc_lsb_lt_minus1[ i ] + 1 ) * ( poc_granularity_minus1[ temporal_id ] + 1 )					-
	else													(7-x)
		DeltaPocLt[ i ] = ( delta_poc_lsb_lt_minus1[ i ] + 1 ) * ( poc_granularity_minus1[ temporal_id ] + 1 ) + DeltaPocLt[ i – 1 ]

The value of DeltaPocLt[ i ] shall be in the range of 0 to MaxPicOrderCntLsb, inclusive.

## Decoding Process for Picture Order Count

The LSB of the picture order count is obtained by multiplying pic_order_cnt_cycle with the POC granularity goverened by temporal_id.

- Change the decoding process for picture order count as follows

Output of this process is PicOrderCnt, the picture order count of the current picture.

Picture order counts are used to identify pictures, to determine co-located pictures (see subclause Error: Reference source not found) for deriving motion parameters in temporal or spatial direct mode, to represent picture order differences between frames for motion vector derivation in temporal direct mode (see subclause Error: Reference source not found), for implicit mode weighted prediction in B slices (see subclause Error: Reference source not found), and for decoder conformance checking (see subclause Error: Reference source not found).

Each coded picture is associated with one picture order count, called PicOrderCnt. PicOrderCnt indicates the picture order of the corresponding picture relative to the previous IDR picture in decoding order.

The variables prevPicOrderCntLsb and prevPicOrderCntMsb are derived as follows.

- If the current picture is an IDR picture, both prevPicOrderCntLsb and prevPicOrderCntMsb are set equal to 0.

- Otherwise (the current picture is not an IDR picture), the following applies.

  - Let prevRefPic be the previous reference picture in decoding order that has temporal_id equal to 0. The variable prevPicOrderCntLsb is set equal to picOrderCntLsb of prevRefPic, and the variable prevPicOrderCntMsb is set equal to PicOrderCntMsb of prevRefPic.

PicOrderCntMsb of the current picture is derived as specified by the following pseudo-code:

picOrderCntLsb = pic_order_cnt_cycle * ( poc_granularity_minus1[ temporal_id ] + 1 )
if( ( picOrderCntLsb  <  prevPicOrderCntLsb )  &&
		( ( prevPicOrderCntLsb − picOrderCntLsb )  >=  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb + MaxPicOrderCntLsb	(8-x)
else if( (picOrderCntLsb  >  prevPicOrderCntLsb )  &&
		( (picOrderCntLsb − prevPicOrderCntLsb )  >  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb − MaxPicOrderCntLsb
else
	PicOrderCntMsb = prevPicOrderCntMsb

PicOrderCnt is derived as

PicOrderCnt = PicOrderCntMsb + picOrderCntLsb		(8-x)

NOTE 1 – All IDR pictures will have PicOrderCnt equal to 0 since pic_order_cnt_cycle is inferred to be 0 for IDR pictures and prevPicOrderCntLsb and prevPicOrderCntMsb are both set equal to 0.

The value of PicOrderCnt shall be in the range of −231 to 231 − 1, inclusive. In one coded video sequence, the PicOrderCnt values for any two coded pictures shall be different.

The function PicOrderCnt( picX ) is specified as follows:

PicOrderCnt( picX ) = PicOrderCnt of the picture picX	(8-x)

The function DiffPicOrderCnt( picA, picB ) is specified as follows:

DiffPicOrderCnt( picA, picB ) = PicOrderCnt( picA ) − PicOrderCnt( picB )	(8-x)

The bitstream shall not contain data that result in values of DiffPicOrderCnt( picA, picB ) used in the decoding process that exceed the range of −215 to 215 − 1, inclusive.

NOTE 2 – Let X be the current picture and Y and Z be two other pictures in the same sequence, Y and Z are considered to be in the same output order direction from X when both DiffPicOrderCnt( X, Y ) and DiffPicOrderCnt( X, Z ) are positive or both are negative.

NOTE 3 – Many encoders assign PicOrderCnt proportional to the sampling time of the corresponding picture relative to the sampling time of the previous IDR picture.

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).