| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0429 |
| --- | --- |

| Title: | Non-CE9: Moving fifth spatial MV candidate after TMVP for merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Xianglin Wang Jianle Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Email: | vseregin@qualcomm.com xianglin@qualcomm.com cjianle@qualcomm.com martak@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes to switch the fetching order between the fifth spatial (i.e. above-left) MV candidate and TMVP candidate in the merge list construction. According to the current HM, only the first four spatial candidates are used in the merge list generation, when all of them are available. In this case, the fifth one is not used and wasted if TMVP candidate does not exist. With the proposed modification, the fifth spatial MV candidate is added to the merge list in such a scenario.

# Introduction and proposal

As shown on Fig. 1(a), the merge list before pruning consists of up to five MV candidates in current HM.   The fifth spatial candidate is added only if at least one of the previous four candidates is not available, followed by the TMVP candidate. In the case that the first four spatial candidates are all available but TMVP is not available, this fifth MV candidate will never be considered and the initial merge list only consists of four candidates.

It is proposed, simply change the fetching order of fifth spatial MV candidate and check it right after TMVP candidate as shown on Fig. 1(b).

Fig. 1. Moving fifth spatial MV candidate after TMVP

This change of order was implemented on top of HM5.0 and results are summarised in the following Table 1 based on common test conditions [1]. Small coding gain can be observed for most cases, and for some particular sequences, for example for class E and F, gain is up to 0.4%.

Table 1. Simulation results with the proposed change

# Conclusion

With the proposed order switch in merge list construction, the fifth spatial MV can be better utilized. Small coding gain can be observed in most cases. We recommend to adopt it in the HM.

# References

- F. Bossen, “Common HM test conditions and software reference configurations,” JCTVC-G1200, 7th JCT-VC Meeting, Geneva, CH, November 2011.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Working Draft text

This text modification for working draft is based on JCTVC-G1103 version d8.

8.4.2.1.1	Derivation process for luma motion vectors for merge mode

…

4.	The merging candidate list, mergeCandList, is constructed as follows.

1.	A1, if availableFlagA1 is equal to 1

2.	B1, if availableFlagB1 is equal to 1

3.	B0, if availableFlagB0 is equal to 1

4.	A0, if availableFlagA0 is equal to 1

5.	B2, if availableFlagB2 is equal to 1

6.	Col, if availableFlagCol is equal to 1

5.	Col, if availableFlagCol is equal to 1

6.	B2, if availableFlagB2 is equal to 1

…