| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0430 |
| --- | --- |

| Title: | Non-CE9: Single TMVP for merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Xianglin Wang Jianle Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Email: | vseregin@qualcomm.com xianglin@qualcomm.com cjianle@qualcomm.com martak@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In current HEVC design, temporal motion vector predictor is derived for every partition unit to generate merge candidate list. To obtain temporal candidate it is necessary to derive a reference index, locate a position of collocated block in reference frame and finally apply scaling to collocated motion vector. This operation is much more complex compared with motion vector candidates fetched from spatial neighbors. This contribution proposes to derive only one single temporal candidate per coding unit and use this single temporal candidate for all prediction units with the coding unit.

# Introduction and proposal

In current HM, merge candidate list is constructed from neighboring spatial and temporal motion vectors, and the merge list is generated for each partition unit (PU). However, complexity of deriving temporal motion vector predictor (TMVP) is significantly higher than deriving spatial candidates, since TMVP derivation includes locating the collocated block in a reference frame and scaling the collocated MV.

Instead of deriving TMVP for every PU, in this contribution, it is proposed to obtain TMVP candidate once per coding unit (CU) and use this single TMVP for every PU merge list generation within the same CU.

More specifically, it is proposed to derive the single TMVP per coding unit at the same location T as it is currently done for 2N×2N mode as shown in Fig. 1 (a). The same TMPV is used for all PUs in the CU, regardless of its partition type including AMP mode.

Fig. 1. Single TMVP candidate for different partition modes.

# Simulation

The proposed single TMVP derivation per coding unit for merge list construction was implemented on top of HM5.0 and experiment results under common test conditions [1] are summarized in the Tab.1.

Table 1. Experiments results for single temporal merge candidate.

# Conclusion

This contribution proposes to use a single CU based TMVP candidate in merge list derivation process for all PUs inside the CU, regardless of its motion partition type. With the proposed simplification, the same module of TMVP derivation can be shared for all blocks within a CU. With such simplification, the impact on coding performance is about 0.06% loss. We recommend to adopt the CU-based single merge temporal candidate in the HM.

# References

- F. Bossen, “Common HM test conditions and software reference configurations,” JCTVC-G1200, 7th JCT-VC Meeting, Geneva, CH, November 2011.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Working Draft text

This text modification for working draft is based on JCTVC-G1103 version d8.

8.4.2.1.1	Derivation process for luma motion vectors for merge mode

This process is only invoked when PredMode is equal to MODE_SKIP or PredMode is equal to MODE_ INTER and merge_flag [ xP ][ yP ] is equal to 1, where ( xP, yP ) specify the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture.

Inputs of this process are

- a luma location ( xP, yP ) of the top-left luma sample of the current prediction unit relative to the top-left luma sample of the current picture,

- a luma location ( xC, yC ) of the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- variables specifying the width and the height of the coding unit for luma, nCSW and nCSH,

- a variable PartIdx specifying the index of the current prediction unit within the current coding unit.

Outputs of this process are

–	the luma motion vectors mvL0 and mvL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags predFlagL0 and predFlagL1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

- The derivation process for merging candidates from neighboring prediction unit partitions in subclause Error: Reference source not found is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N and the motion vectors mvL0N and mvL1N with N being replaced by A0, A1, B0, B1 or B2.

- The reference index for temporal merging candidate is derived as follows..

- If the prediction unit covering luma location ( xC − 1, yC + nCSH − 1 ) is available and PredMode is not MODE_INTRA, refIdxLX is set equal to refIdxLX[ xC − 1, yC + nCSH − 1 ].

- Otherwise, refIdxLX is set equal to 0.

...

8.4.2.1.8	Derivation process for temporal luma motion vector prediction

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current coding prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the coding prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are

- the motion vector prediction mvLXCol,

- the availability flag availableFlagLXCol.

…