| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0431 |
| --- | --- |

| Title: | Non-CE9: Splitting contexts for MVD coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jianle Chen Xianglin Wang Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Email: | vseregin@qualcomm.com cjianle@qualcomm.com xianglin@qualcomm.com martak@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

For coding motion vector difference (MVD), after binarization the first two bins are coded using context modeling and the remaining bins using bypass mode. Context models for the first two bins are shared for all blocks regardless of their associated inter prediction direction or reference lists. However, MVD from different directions may have very different statistical properties, in which case context sharing may not be suitable. That is true especially if motion estimation is unbalanced at encoder side, for example some MVDs are set to zero for a particular list. This contribution proposes assigning a separate context to the first MVD bin respectively for uni-predicted, bi-predicted list L0 and bi-predicted list L1 MVD.

# Introduction

In current HM, the first two binarized bins of each MVD syntax are coded with two context models respectively and the remaining bins are processed through bypass mode with exgolomb code. The context models are shared among MVDs regardless of the MVDs belonging to horizontal or vertical direction, lists L0 or L1 and uni-predicted PU or bi-predicted PU. However, MVDs from different source may have different statistical properties, especially if MVD is set to zero for one of the lists for all bi-predicted PUs at encoder side as it is being tested in CE9. It is alleged that such kind of encoder side restriction can improve coding performance in certain cases. To adaptively accommodate such unbalanced statistical properties of MVD, it is suggested to use separate contexts for the first bin of the MVD syntax according to the reference picture list and inter prediction type.

# Proposed methods

In this document,  three methods of assigning separate context with different level of flexibility are studied. All methods are concerning only the context model of the first bin of MVD syntax.

- Assign a separate context to the first bin of L0 list MVD of bi-predicted PU; one context model is added.

- Assign a separate context to the first bin of L1 list MVD of bi-predicted PU; one context model is added

- Assign a separate context for the first bin respectively for uni-predicted, bi-predicted list L0 and bi-predicted list L1 MVD; two context models are added.

Context model used in coding the first bin of MVD syntax is summarized in Table 1, for anchor HM5.0 and each of the proposed methods.

Table 1. Contexts for the first MVD bin

|  | uni-predicted PU | bi-predicted PU, L0 | bi-predicted PU, L1 | Additional contexts |
| --- | --- | --- | --- | --- |
| HM5.0 | Ctx0 |  |  | 0 |
| Method1 | Ctx0 | Ctx1 | Ctx0 | 1 |
| Method2 | Ctx0 |  | Ctx1 | 1 |
| Method3 | Ctx0 | Ctx1 | Ctx2 | 2 |

# Simulation

These context separation schemes were implemented on top of both HM5.0 and the distributed software dedicated to Core Experiment 9 [1].  The CE9 tests the methods that set MVD to zero for either L0 or L1 reference list of bi-predicted PU at encoder side and skip the MVD syntax coding for that list. Correspondingly, the following tests were performed in this contribution.

- Set L0 MVD to zero for bi-predicted PU in GPB picture + proposed method1

- Set L1 MVD to zero for bi-predicted PU in GPB picture + proposed method2

- Set L0 MVD to zero for bi-predicted PU in GPB picture + proposed method3

- Set L1 MVD to zero for bi-predicted PU in GPB picture + proposed method3

- Proposed Method1 applied for HM5.0

- Proposed Method2 applied for HM5.0

- Proposed Method3 applied for HM5.0

Setting L1 MVD to zero (test 2 and test 4) corresponds to BP01 test in CE9 and these two cases may be compared to each other.  Similarly, test 1 and test3 could be compared to BP02 test in CE9 . All tests are performed based on the common test configuration [2] and summarized in Table 2.

Table 2. Experiment results for proposed methods

|  | RA-HE | RA-LC | RA-HE10 | LB-HE | LB-LC |
| --- | --- | --- | --- | --- | --- |
| Test 1 | -0.18 | -0.22 | -0.24 | -0.70 | -0.97 |
| Test 2 | -0.18 | -0.22 | -0.25 | -0.72 | -1.02 |
| Test 3 | -0.19 | -0.20 | -0.25 | -0.70 | -0.98 |
| Test 4 | -0.20 | -0.24 | -0.26 | -0.73 | -1.02 |
| Test 5 | 0.00 | 0.00 | 0.03 | -0.01 | -0.01 |
| Test 6 | -0.01 | 0.00 | 0.02 | 0.02 | -0.01 |
| Test 7 | -0.03 | 0.01 | 0.02 | -0.01 | 0.00 |

More detailed test data can be found in the accompanied excel sheets.

# Conclusion

This contribution proposes to add one or two contexts to separate first MVD bin for list L0 and L1 of bi-predicted PU. This allows taking into account different statistical properties of the MVD coming from different reference lists due to unbalanced ME performed for list L0 and L1 at encoder side. Instead of skipping the MVD syntax explicitly, the proposed methods implicitly and adaptively takes advantage of any encoder side ME tricks and offers greater ME flexibility.

Based on the results, we recommend adoption of  the proposed method3 to the HM.

# References

- I.-K. Kim, J. Jung, S. Oudin, “Description of Core Experiment 9 (CE9): MV Coding and Skip/Merge Operations”, JCTVC-G1209, Geneva, Nov. 2011.

- F. Bossen, “Common HM test conditions and software reference configurations,” JCTVC-G1200, Geneva, Nov. 2011.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Working Draft text

This text modification for working draft is based on JCTVC-G1103 version d8.

Table 9-1 – Association of ctxIdx and syntax elements for each slice type in the initialisation process

|  | Syntax element | ctxIdxTable | Slice Type |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | I | P | B |
| slice_header() | alf_cu_flag | Error: Reference source not found | 0 | 1 | 2 |
| coding_tree() | split_coding_unit_flag | Error: Reference source not found | 0..2 | 3..5 | 6..8 |
| coding_unit() | skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | cu_qp_delta | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | pred_mode_flag | Error: Reference source not found |  | 0 | 1 |
|  | part_mode | Error: Reference source not found | 2 | 3..5 | 6..8 |
| prediction_unit() | prev_intra_luma_pred_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | merge_flag | Error: Reference source not found |  | 0 | 1 |
|  | merge_idx | Error: Reference source not found |  | 0..3 | 4..7 |
|  | inter_pred_flag | Error: Reference source not found |  |  | 0..3 |
|  | ref_idx_lc, ref_idx_l0, ref_idx_l1 | Error: Reference source not found |  | 0..2 | 3..5 |
|  | abs_mvd_greater0_flag | Error: Reference source not found |  | 0 | 2..4 |
|  | abs_mvd_greater1_flag | Error: Reference source not found |  | 1 | 5 |
|  | mvp_lc_flag, mvp_l0_flag, mvp_l1_flag | Error: Reference source not found |  | 0 | 1 |
| transform_tree() | no_residual_data_flag | Error: Reference source not found |  | 0 | 1 |
|  | split_transform_flag | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | cbf_luma | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | cbf_cb, cbf_cr | Error: Reference source not found | 0..2 | 3..5 | 6..8 |
| residual_coding() | last_significant_coeff_x_prefix | Error: Reference source not found | 0..29 | 30..59 | 60..89 |
|  | last_significant_coeff_y_prefix | Error: Reference source not found | 0..29 | 30..59 | 60..89 |
|  | significant_coeff_group_flag | Table 9-26 | 0..3 | 4..7 | 8..11 |
|  | significant_coeff_flag | Error: Reference source not found | 0..47 | 48..95 | 96..143 |
|  | coeff_abs_level_greater1_flag | Error: Reference source not found | 0..31 | 32..63 | 64..95 |
|  | coeff_abs_level_greater2_flag | Error: Reference source not found | 0..23 | 24..47 | 48..71 |

Table 9-2 – Values of variable initValue for abs_mvd_greater0_flag and abs_mvd_greater1_flag ctxIdx

| Initialisation variable | abs_mvd_greater0_flag, abs_mvd_greater1_flag ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | 120 | 166 | 135 | 135 | 135 | 166 |

Table 9-3 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset

| … |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ref_idx_l1 | B | TU, cMax = num_ref_idx_l1_active_minus1 | 2 | Error: Reference source not found | 3 |
| abs_mvd_greater0_flag[ ] | P | FL, cMax = 1 | 0 | Error: Reference source not found | 0 |
|  | B inter_pred_flag != 3 |  | 0 | Error: Reference source not found | 2 |
|  | B inter_pred_flag = 3 CurRefPicList = RefPicList0 |  |  |  | 3 |
|  | B inter_pred_flag = 3 CurRefPicList = RefPicList1 |  |  |  | 4 |
| abs_mvd_greater1_flag[ ] | P | FL, cMax = 1 | 0 | Error: Reference source not found | 1 |
|  | B |  | 0 | Error: Reference source not found | 5 |
| abs_mvd_minus2[ ] | P/B | EG1 | na | na | na, (uses Decode Bypass) |
| … |  |  |  |  |  |