| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, 1-10 February, 2012 | Document: JCTVC-H0432 |
| --- | --- |

| Title: | Sub-Block Based Significance Map Region Classification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tianying Ji, Nguyen Nguyen, Dake He, Gaëlle Martin-Cocher 185 Columbia St. W. Waterloo, ON, Canada N2L 5Z5 | Tel: Email: | +1-519-8887465 tiji@rim.com nnguyen@rim.com dhe@rim.com gmartincocher@rim.com |
| Source: | Research In Motion Ltd. |  |  |

_____________________________

# Abstract

Two sub-block based classifications of significance coefficient flag context sets are proposed for Large Transform Units (LTU) of sizes greater than 8x8. In the HEVC latest working draft, the encoding and decoding of LTUs are modularized to 4x4 sub-blocks, yet the significance coefficient flag context sets are still classified based on coefficients’ positions. To embrace the modular processing method, context sets are classified on sub-block level. The first method provides a dynamic allocation of the context sets based on the coefficient group flag value, which unifies the solution for different TU sizes. The second method is position based applied to the coefficient group. Results show that the average BD-rate difference against HM5.0 is within 0.1% using anchor initial distribution. There is no noticeable difference in both encoding time and decoding time.

# Proposal

## Existing Significance Map Regions

The significance map of a LTU (16xM, Mx16, 32xN, Nx32, M=4,16, N=8,32) is divided into three regions, region 0, 2, and 3 (Figure 1). Each region has its own context set:

- Region 0 consists of DC coefficient;

- Region 1 covers coefficients whose two-dimensional coordination (x, y) satisfies: x+y<4, for 16xM and Mx16 TUs, and x+y<8 for 32xN and Nx32 TUs, excluding the DC coefficient.

The rest coefficients are classified to Region 2.

Region 0 (DC)

x+y=4 or 8

Region 1

Region 2

Figure 1. Significance Map Regions in the current HEVC working draft

## Proposed Dynamic Significance Map Regions

The proposed classification has the following advantages compared to the existing method:

- Dynamic: the region boundary can grow or shrink in different environment, e.g., for different QP values, there are different significance map regions.

- Unified solution: one solution for different TU sizes, e.g., 16xM, Mx16, 32xN, Nx32 all share the same classification algorithm, and context derivation is consistent with significant_coeff_group_flag coding

- Modular processing: the coefficient significance flag context sets are now determined by the property of the sub-blocks, which simplifies the algorithm.

In the latest HEVC working draft, LTUs are processed in two levels: the 4x4 sub-block level and the coefficient level within a sub-block. The contexts of sub-block significance flags, are determined by the right and lower neighbors:

- Context 0: if both neighbors are 0’s.

- Context 1: if one of the neighbors is 1.

- Inferred to be 1: The 1st sub-block that contains DC and the ones with both neighbors to be 1’s.

Based on this sub-block significance information, a dynamic classification of significance map regions is proposed as follows:

- Region 0 consists of DC coefficient.

- Region 1 consists of the sub-blocks whose significance flags are inferred to be 1, e.g., the 1st sub-block and the ones with both neighbors to be 1’s.

- The rest sub-blocks are classified to Region 2.

Figure 2 shows an example of sub-block significance flags in a 16x16 TU. “0” and “1” denote the values of sub-block significance flags: “0” means “all zero coefficient significance flags in sub-block”, and “1” means “at least one non-zero coefficient significance flags in sub-block”. “Inf” means the sub-block significance flag is inferred to be “1”. For the purpose of simplification, DC coefficient is not shown for sub-block #0.

| Inf | Inf | Inf | 1 |
| --- | --- | --- | --- |
| Inf | Inf | 1 | 0 |
| Inf | 1 | 1 |  |
| 1 | 0 |  |  |

Figure 2. Example of sub-block significance flags in a 16x16 TU (“Inf” means inferred to be one)

In this example, the context region classification is given in Figure 3.

| 1 | 1 | 1 | 2 |
| --- | --- | --- | --- |
| 1 | 1 | 2 | 2 |
| 1 | 2 | 2 | 2 |
| 2 | 2 | 2 | 2 |

Figure 3. Example of dynamic context region classification in a 16x16 TU

## Proposed Fixed Significance Map Regions

As an alternative solution, a simple fixed classification of significance map regions for 16xM and Mx16 TUs  is proposed as in Figure 4:

- Region 0 consists of DC coefficient;

- Region 1 consists of the 15 coefficients in sub-block #0 (SB#0), excluding the DC coefficient.

The rest sub-blocks are classified to Region 2.

Region 0 (DC)

Region 1

SB#0

Region 2

Figure 4. Proposed Significance Map Regions in 16xM and Mx16 TUs

The significance map regions for 32xN and Nx32 TUs are proposed as in Figure 5:

- Region 0 consists of DC coefficient;

- Region 1 consists of the coefficients in the 4 sub-block on the upper-left corner of TU (SB#0,1,2,3), excluding the DC coefficient.

- The rest sub-blocks are classified to Region 2.

Region 0 (DC)

Region 1

SB#0

Region 2

SB#1

SB#2

SB#3

Figure 5. Proposed Significance Map Regions in 32xN and Nx32 TUs

# Results

In this section, we report experimental results of the proposal (integrated into HM5.0) against HM5.0 anchor in common test conditions [3].

Table 1 below shows that the BD-rates of the proposal in section 1.2 against HM-5.0 anchor (a negative value indicates gain; and a positive value indicates loss).

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.1% | 0.1% | 0.0% | -0.1% | 0.0% |  |  |  |
| Class B | 0.0% | 0.2% | 0.2% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| Class E | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | -0.2% | 0.1% | -0.2% | -0.1% |
| Class B | 0.0% | 0.1% | 0.2% | 0.0% | 0.2% | 0.0% | 0.1% | 0.1% | 0.0% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.1% | -0.1% | 0.0% | 0.0% | 0.2% | -0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% | -0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | 0.1% | 0.0% | 0.0% |
| Class F | -0.1% | 0.0% | -0.2% | 0.0% | -0.1% | -0.1% |  |  |  |
| Enc Time[%] | 99% |  |  | 101% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | -0.2% | 0.0% |  |  |  |
| Class C | 0.0% | -0.2% | 0.1% | 0.0% | 0.2% | -0.1% |  |  |  |
| Class D | 0.0% | -0.3% | 0.1% | 0.0% | 0.1% | 0.3% |  |  |  |
| Class E | 0.0% | 0.4% | -0.5% | -0.1% | 0.4% | 0.6% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class F | -0.1% | 0.0% | 0.0% | 0.1% | -0.9% | -0.1% |  |  |  |
| Enc Time[%] | 99% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |

Table 2 below shows that the BD-rates of the proposal in section 1.3 against HM-5.0 anchor (a negative value indicates gain; and a positive value indicates loss).

|  | All Intra HE |  |  | All Intra LC |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | 0.0% | 0.1% | 0.1% | 0.2% | 0.2% |  |  |  |
| Class B | 0.1% | 0.2% | 0.2% | 0.2% | 0.3% | 0.3% |  |  |  |
| Class C | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| Class D | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| Class E | 0.1% | 0.1% | 0.2% | 0.1% | 0.3% | 0.3% |  |  |  |
| Overall | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% |  |  |  |
|  | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% |  |  |  |
| Class F | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | -0.3% | 0.0% | 0.2% | 0.0% | 0.3% | -0.5% | 0.0% |
| Class B | 0.1% | 0.0% | 0.1% | 0.1% | 0.2% | 0.3% | 0.1% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | -0.2% | 0.1% | 0.0% | 0.2% | -0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.1% | 0.2% | 0.1% | 0.2% | -0.2% | 0.0% |
|  | 0.0% | -0.1% | 0.0% | 0.1% | 0.2% | 0.1% | 0.1% | -0.1% | 0.1% |
| Class F | -0.1% | -0.1% | 0.0% | 0.1% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.1% | -0.1% | 0.1% | 0.1% | -0.3% | 0.5% |  |  |  |
| Class C | 0.0% | -0.3% | 0.0% | 0.0% | 0.3% | -0.1% |  |  |  |
| Class D | 0.0% | -0.3% | 0.2% | 0.0% | 0.8% | 0.1% |  |  |  |
| Class E | 0.0% | 1.0% | -0.2% | 0.0% | -0.9% | -0.5% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |  |  |  |
|  | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | 0.0% |  |  |  |
| Class F | -0.2% | -0.1% | -0.2% | 0.0% | -0.9% | -0.2% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

# Conclusion

In this proposal, two methods are proposed to classify the significance map regions on the sub-block level.  Both solutions unify with the sub-block based modularized encoding and decoding processes. On average, there is no bd-rate loss and no change to the encoding and decoding time. The dynamic allocation of context set has the additional advantages of unifiyng the solution for different TU sizes and context derivation is consistent with significant_coeff_group_flag coding. The dynamic allocation of context set based on sub-block flag is recommended to be added to the HM and working draft.

# Change to the HEVC working draft text

## Proposed changes to “9.2.3.1.1.5 Derivation of ctxIdxInc for the syntax element significant_coeff_flag” based on proposal in section 1.2

Inputs to this process are the color component index cIdx, the current coefficient scan position ( xC , yC ), the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the color component index cIdx, the transform block size, previously decoded bins of the syntax element significant_coeffgroup_flag and previsously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 2, sigCtx is derived using ctxIdxMap4x4[ ] specified in Error: Reference source not found as follows..

sigCtx  =  ctxIdxMap4x4[ ((cIdx > 0) ? 15 : 0) + (yC << 2) + xC ]	(9-1)

- Otherwise if log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 3, sigCtx is derived using ctxIdxMap8x8[ ] specified in Error: Reference source not found as follows.

sigCtx  =  ((xC + yC) = = 0) ? 10 : ctxIdxMap8x8[ ((yC >> 1 ) << 2) + (xC >> 1) ]	(9-2)

sigCtx  +=  ( cIdx > 0) ? 6: 9		(9-3)

- Otherwise if xC + yC is equal to 0, sigCtx is derived as follows.

sigCtx  =  ( cIdx > 0) ? 17: 20		(9-4)

- Otherwise (xC + yC is greater than 0), sigCtx is derived using previously decoded bins of the syntax element significant_coeff_flag as follows.

- The variable sigCtx is initialized as follows.

sigCtx  =  0		(9-5)

- When xC is less than ( 1 << log2TrafoWidth ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC ]	(9-6)

- When xC is less than ( 1 << log2TrafoWidth ) − 1 and yC is less than ( 1 << log2TrafoHeight ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]	(9-7)

- When xC is less than ( 1 << log2Width ) − 2, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 2 ][ yC ]	(9-8)

- When all of the following conditions are true,

  - yC is less than ( 1 << log2TrafoHeight ) − 1,

  - xC % 4 is not equal to 0 or yC % 4 is not equal to 0,

  - xC % 4 is not equal to 3 or yC % 4 is not equal to 2,

the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 1 ]	(9-9)

- When yC is less than ( 1 << log2TrafoHeight ) − 2 and sigCtx is less than 4, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-10)

- The variable sigCtx is modified as follows.

  - When all of the following conditions are true,

    - If cIdx is equal to 0,

    - xC < 4 and yC < 4, or when xC is less than (1 << (log2TrafoWidth – 1)) and yC is less than (1 << (log2TrafoHeight – 1)) and significant_coeffgroup_flag[(xC >> 2) + 1][yC >> 2] is equal to 1 and significant_coeffgroup_flag[xC >> 2][(yC >> 2) + 1] is equal to 1,

the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + 24		(9-11)

  - Otherwise, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + ( (cIdx > 0) ? 18 : 21 )	(9-12)

The context index increment ctxIdxInc is derived using the color component index cIdx and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx		(9-13)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  27  +  sigCtx

## Proposed changes to “9.2.3.1.1.5 Derivation of ctxIdxInc for the syntax element significant_coeff_flag” based on proposal in section 1.3

Inputs to this process are the color component index cIdx, the current coefficient scan position ( xC , yC ), the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the color component index cIdx, the transform block size and previsously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 2, sigCtx is derived using ctxIdxMap4x4[ ] specified in Error: Reference source not found as follows..

sigCtx  =  ctxIdxMap4x4[ ((cIdx > 0) ? 15 : 0) + (yC << 2) + xC ]	(9-14)

- Otherwise if log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 3, sigCtx is derived using ctxIdxMap8x8[ ] specified in Error: Reference source not found as follows.

sigCtx  =  ((xC + yC) = = 0) ? 10 : ctxIdxMap8x8[ ((yC >> 1 ) << 2) + (xC >> 1) ]	(9-15)

sigCtx  +=  ( cIdx > 0) ? 6: 9		(9-16)

- Otherwise if xC + yC is equal to 0, sigCtx is derived as follows.

sigCtx  =  ( cIdx > 0) ? 17: 20		(9-17)

- Otherwise (xC + yC is greater than 0), sigCtx is derived using previously decoded bins of the syntax element significant_coeff_flag as follows.

- The variable sigCtx is initialized as follows.

sigCtx  =  0		(9-18)

- When xC is less than ( 1 << log2TrafoWidth ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC ]	(9-19)

- When xC is less than ( 1 << log2TrafoWidth ) − 1 and yC is less than ( 1 << log2TrafoHeight ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]	(9-20)

- When xC is less than ( 1 << log2Width ) − 2, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 2 ][ yC ]	(9-21)

- When all of the following conditions are true,

  - yC is less than ( 1 << log2TrafoHeight ) − 1,

  - xC % 4 is not equal to 0 or yC % 4 is not equal to 0,

  - xC % 4 is not equal to 3 or yC % 4 is not equal to 2,

the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 1 ]	(9-22)

- When yC is less than ( 1 << log2TrafoHeight ) − 2 and sigCtx is less than 4, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-23)

- The variable sigCtx is modified as follows.

  - If cIdx is equal to 0 and both xC and yC are greater than or equal to 1 << (max(log2TrafoWidth, log2TrafoHeight) – 2), the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + 24		(9-24)

  - Otherwise, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + ( (cIdx > 0) ? 18 : 21 )	(9-25)

The context index increment ctxIdxInc is derived using the color component index cIdx and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx		(9-26)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  27  +  sigCtx

# References

[1] D. Marpe, H. Schwarz, and T. Wiegand, “Context-based adaptive binary arithmetic coding in the h.264/AVC video compression standard,” IEEE Transactions on Circuits and Systems for Video Technology, 13(7):620–636, Jul. 2003.

[2] B. Bross, W-J Han, J-R Ohm, G. J. Sullivan, and T. Wiegand, “WD5: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 7th Meeting, Geneva, Nov 2011.

[3] F. Bossen, “Common test conditions and software reference configurations,” JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 6th Meeting, Torino, July 2011.

# Patent rights declaration(s)

Research In Motion Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).