#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| entropy_coding_mode_flag | u(1) |
| num_short_term_ref_pic_sets | ue(v) |
| for(idx = 0; idx < num_short_term_ref_pic_sets; idx++) |  |
| short_term_ref_pic_set( idx ) |  |
| delete_pics _info_present_flag | u(1) |
| long_term_ref_pics_present_flag | u(1) |
| num_temporal_layer_switching_point_flags | ue(v) |
| for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ) |  |
| temporal_layer_switching_point_flag[ i ] | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| pic_init_qp_minus26 /* relative to 26 */ | se(v) |
| constrained_intra_pred_flag | u(1) |
| slice_granularity | u(2) |
| shared_pps_info_enabled_flag | u(1) |
| if( shared_pps_info_enabled_flag ) |  |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_param( ) |  |
| if( cu_qp_delta_enabled_flag ) |  |
| max_cu_qp_delta_depth | u(4) |
| rbsp_trailing_bits( ) |  |
| } |  |

## Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| if( IdrPicFlag ) { |  |
| idr_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } |  |
| else { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_pps_flag | u(1) |
| if( !short_term_ref_pic_set_pps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( delete_pics_info_present_flag ) { |  |
| keep_pics_info_present_flag | u(1) |
| if( keep_pics_info_present_flag ) { |  |
| for( j = 0; j < (num_negative_pics+num_positive_pics); j++ ) { |  |
| keep_pic_flag[j] | u(1) |
| } |  |
| } |  |
| } |  |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| delta_poc_lsb_lt_minus1[ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

#### Short-term reference picture set syntax

| short_term_ref_pic_set( idx ) { | Descriptor |
| --- | --- |
| num_negative_pics | ue(v) |
| num_positive_pics | ue(v) |
| for( i = 0; i < num_negative_pics; i++ ) { |  |
| delta_poc_s0_minus1[ i ] | ue(v) |
| used_by_curr_pic_s0_flag[ i ] | u(1) |
| } |  |
| for( i = 0; i < num_positive_pics; i++ ) { |  |
| delta_poc_s1_minus1[ i ] | ue(v) |
| used_by_curr_pic_s1_flag[ i ] | u(1) |
| } |  |
| } |  |

#### Reference picture list modification syntax

| ref_pic_list_modification( ) { | Descriptor |
| --- | --- |
| if( slice_type != 2 ) { |  |
| ref_pic_list_modification_flag_l0 | u(1) |
| if( ref_pic_list_modification_flag_l0 ) |  |
| do { |  |
| list_modification_idc | ue(v) |
| if( list_modification_idc != 3 ) |  |
| ref_pic_set_idx | ue(v) |
| } while( list_modification_idc != 3 ) |  |
| } |  |
| if( slice_type = = 1 ) { |  |
| ref_pic_list_modification_flag_l1 | u(1) |
| if( ref_pic_list_modification_flag_l1 ) |  |
| do { |  |
| list_modification_idc | ue(v) |
| if( list_modification_idc != 3 ) |  |
| ref_pic_set_idx | ue(v) |
| } while( list_modification_idc != 3 ) |  |
| } |  |
| } |  |

### Picture parameter set RBSP semantics

…

num_short_term_ref_pic_sets specifies the number of short-term reference picture sets that are specified in the picture parameter set. The value of num_short_term_ref_pic_sets shall be in the range of 0 to 64, inclusive.

NOTE – A decoder must allocate space for a total number of num_short_term_ref_pic_sets + 1 short-term reference picture sets since a coded video sequence may contain up to one short-term reference picture set explicitly signalled in the slice headers of a current picture. An explicitly signalled short-term reference picture set will always have an index equal to num_short_term_ref_pic_sets in the list of short-term reference picture sets.

delete_pics_info_present_flag equal to 1 specifies that slice header may signal delete operation from the short-term reference picture set included in the picture parameter set. delete_pics_info_present_flag equal to 0 specifies that no delete operation is signaled in the slice header for the short-term reference picture set included in the picture parameter set

long_term_ref_pics_present_flag equal to 0 specifies that no long-term reference picture is used for inter prediction of any coded picture referring to the picture parameter. long_term_ref_pics_present_flag equal to 1 specifies that long-term reference pictures may be used for inter prediction of one or more coded pictures referring to the picture parameter set.

…

### Slice header semantics

When present, the value of the slice header syntax elements pic_parameter_set_id, idr_pic_id no_output_of_prior_pics_flag, pic_order_cnt_lsb, short_term_ref_pic_set_pps_flag, short_term_ref_pic_set_idx and num_long_term_pics shall be the same in all slice headers of a coded picture. When present, the value of the slice header syntax elements delta_poc_lsb_lt_minus1[ i ] and used_by_curr_pic_lt_flag[ i ] shall be the same in all slice headers of a coded picture for each i in the range of 0 to num_long_term_pics, inclusive.

…

short_term_ref_pic_set_pps_flag equal to 1 specifies that the short-term reference picture set of the current picture shall be created using syntax elements in the active picture parameter set. short_term_ref_pic_set_pps_flag equal to 0 specifies that the short-term reference picture set of the current picture shall be created using syntax elements in the short_term_ref_pic_set( ) syntax structure in the slice header.

short_term_ref_pic_set_idx specifies the index to the list of the short-term reference picture sets specified in the active picture parameter set that shall be used for creation of the reference picture set of the current picture. The syntax element short_term_ref_pic_set_idx shall be represented by ceil( log2( num_short_term_ref_pic_sets ) ) bits. The value of short_term_ref_pic_set_idx shall be in the range of 0 to num_short_term_ref_pic_sets – 1, inclusive, where num_short_term_ref_pic_sets is the syntax element from the active picture parameter set.

The variable StRpsIdx is derived as follows.

if( short_term_ref_pic_set_pps_flag )
		StRpsIdx = short_term_ref_pic_set_idx
	else													(7-x)
		 StRpsIdx = num_short_term_ref_pic_sets

keep_pics_info_present_flag equal to 1 specifies that this slice header includes keep_pic_flag values indicating information regarding keeping or deleting short term reference pictures from the reference picture set with index short_term_ref_pic_set_idx. keep_pics_info_present_flag equal to 0 specifies that this slice header does not include information regarding keeping or deleting short term reference pictures from the reference picture set with index short_term_ref_pic_set_idx.

keep_pic_flag[j] equal to 1 specifies that the corresponding j th short term reference picture from the reference picture set in the picture parameter set with index short_term_ref_pic_set_idx should be kept. keep_pic_flag[j] equal to 0 specifies that the corresponding j th short term reference picture from the reference picture set in the picture parameter set with index short_term_ref_pic_set_idx should be deleted (omitted).

num_long_term_pics specifies the number of the long-term reference pictures that are to be included in the long-term reference picture set of the current picture. The value of num_long_term_pics shall be in the range of 0 to max_num_ref_frames – NumNegativePics[ StRpsIdx ] – NumPositivePics[ StRpsIdx ] , inclusive. When not present, the value of num_long_term_pics shall be inferred to be equal to 0.

delta_poc_lsb_lt_minus1[ i ] is used to determine the value of the least significant bits of the picture order count value of the i-th long-term reference picture that is included in the long-term reference picture set of the current picture. delta_poc_lsb_lt_minus1[ i ] shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive.

The variable DeltaPocLt[ i ] is derived as follows.

if (i  = =  0) 
		DeltaPocLt[ i ] = delta_poc_lsb_lt_minus1[ i ] + 1					(7-x)
	else
		DeltaPocLt[ i ] = delta_poc_lsb_lt_minus1[ i ] + 1 + DeltaPocLt[ i – 1 ]

The value of DeltaPocLt[ i ] shall be in the range of 0 to MaxPicOrderCntLsb, inclusive.

used_by_curr_pic_lt_flag[ i ] equal to 0 specifies that the i-th long-term reference picture included in the long-term reference picture set of the current picture is not used for reference by the current picture.

...

#### Reference picture list modification semantics

The syntax elements list_modification_idc and ref_pic_set_idx specify the change from the initial reference picture lists to the reference picture lists to be used for decoding the slice.

ref_pic_list_modification_flag_l0 equal to 1 specifies that the syntax element list_modification_idc is present for specifying reference picture list 0. ref_pic_list_modification_flag_l0 equal to 0 specifies that this syntax element is not present.

When ref_pic_list_modification_flag_l0 is equal to 1, the number of times that list_modification_idc is not equal to 3 following ref_pic_list_modification_flag_l0 shall not exceed num_ref_idx_l0_active_minus1 + 1.

ref_pic_list_modification_flag_l1 equal to 1 specifies that the syntax element list_modification_idc is present for specifying reference picture list 1. ref_pic_list_modification_flag_l1 equal to 0 specifies that this syntax element is not present.

When ref_pic_list_modification_flag_l1 is equal to 1, the number of times that list_modification_idc is not equal to 3 following ref_pic_list_modification_flag_l1 shall not exceed num_ref_idx_l1_active_minus1 + 1.

list_modification_idc together with ref_pic_set_idx specifies which of the reference pictures are re-mapped. The values of list_modification_idc are specified in Table 7-1. The value of the first list_modification_idc that follows immediately after ref_pic_list_modification_flag_l0 or ref_pic_list_modification_flag_l1 shall not be equal to 3.

Table 7-1 – list_modification_idc operations for modification of reference picture lists

| list_modification_idc | modification specified |
| --- | --- |
| 0 | For list 0: ref_pic_set_idx is present and corresponds to an index to RefPicSetStCurr0; For list 1: ref_pic_set_idx is present and corresponds to an index to RefPicSetStCurr1 |
| 1 | For list 0: ref_pic_set_idx is present and corresponds to an index to RefPicSetStCurr1; For list 1: ref_pic_set_idx is present and corresponds to an index to RefPicSetStCurr0 |
| 2 | ref_pic_set_idx is present and corresponds to an index to RefPicSetLtCurr |
| 3 | End loop for modification of the initial reference picture list |

ref_pic_set_idx specifies the index, to RefPicSetStCurr0, RefPicSetStCurr1 or RefPicSetLtCurr, of the reference picture being moved to the current index in the reference picture list. The value of ref_pic_set_idx shall be in the range of 0 to max_num_ref_frames, inclusive.

…

### Decoding process for reference picture set

This process is invoked once per picture, after decoding of a slice header but prior to the decoding of any coding unit and prior to the decoding process for reference picture list construction of the slice as specified in subclause Error: Reference source not found. The process may result in marking one or more reference pictures as "unused for reference".

The reference picture set consists of six subsets, namely RefPicSetStCurr0, RefPicSetStCurr1, RefPicSetStFoll0, RefPicSetStFoll1, RefPicSetLtCurr, and RefPicSetLtFoll. Each subset consists of a list of picture order count values. The number of entries in RefPicSetStCurr0, RefPicSetStCurr1, RefPicSetStFoll0, RefPicSetStFoll1, RefPicSetLtCurr, and RefPicSetLtFoll are NumRpsStCurr0, NumRpsStCurr1, NumRpsStFoll0, NumRpsStFoll1, NumRpsLtCurr, and NumRpsLtFoll respectively.

RefPicSetStCurr0, RefPicSetStCurr1, RefPicSetStFoll0 and RefPicSetStFoll1 are collectively referred to as the short-term subset of the reference picture set. RefPicSetLtCurr and RefPicSetLtFoll are collectively referred to as the long-term subset of the reference picture set.

If the current picture is an IDR picture, RefPicSetStCurr0, RefPicSetStCurr1, RefPicSetStFoll0, RefPicSetStFoll1, RefPicSetLtCurr, and RefPicSetLtFoll are all set to empty, and NumRpsStCurr0, NumRpsStCurr1, NumRpsStFoll0, NumRpsStFoll1, NumRpsLtCurr, and NumRpsLtFoll are all set to 0.

Otherwise, the following applies for derivation of the six subsets and the numbers of entries.

for( i = 0, j = 0, k = 0; i < NumNegativePics[ StRpsIdx ] ; i++ ) {

if((keep_pics_info_present_flag==0)  ||  ((keep_pics_info_present_flag==1) && (keep_pic_flag[i]==1))  )

{
	if( UsedByCurrPicS0[ StRpsIdx ][ i ] )
		RefPicSetStCurr0[ j++ ] = PicOrderCnt + DeltaPocS0[ StRpsIdx ][ i ]
	else
		RefPicSetStFoll0[ k++ ] = PicOrderCnt + DeltaPocS0[ StRpsIdx ][ i ]

}
}
NumRpsStCurr0 = j
NumRpsStFoll0 = k

for( i = 0, j = 0, k = 0; i < NumPositivePics[ StRpsIdx ]; i++ ) {

if((keep_pics_info_present_flag==0)  ||  ((keep_pics_info_present_flag==1) && (keep_pic_flag[i+ NumNegativePics[ StRpsIdx ]]==1))  )

{

	if( UsedByCurrPicS1[ StRpsIdx ][ i ] )
		RefPicSetStCurr1[ j++ ] = PicOrderCnt + DeltaPocS1[ StRpsIdx ][ i ]
	else
		RefPicSetStFoll1[ k++ ] = PicOrderCnt + DeltaPocS1[ StRpsIdx ][ i ]

}
}
NumRpsStCurr1 = j
NumRpsStFoll1 = k		(8-x)

for( i = 0, j = 0, k = 0; i < num_long_term_pics; i++ ) {
	if( used_by_curr_pic_lt_flag[ i ] )
		RefPicSetLtCurr[ j++ ] = ( PicOrderCnt − DeltaPocLt[ i ] + MaxPicOrderCntLsb ) % 
			MaxPicOrderCntLsb
	else
		RefPicSetLtFoll[ k++ ] = ( PicOrderCnt − DeltaPocLt[ i ] + MaxPicOrderCntLsb ) %
			MaxPicOrderCntLsb
}
NumRpsLtCurr = j
NumRpsLtFoll = k

Where PicOrderCnt is the picture order count of the current picture as specified in subclause 8.2.1.

NOTE 1 – A value of StRpsIdx in the range from 0 to num_short_term_ref_pic_sets – 1, inclusive, indicates that a short-term reference picture set from the active parameter set is being used, where StRpsIdx is the index of the short-term reference picture set to the list of short-term reference picture sets in the order in which they are signalled in the picture parameter set. StRpsIdx equal to num_short_term_ref_pic_sets indicates that a short-term reference picture set explicitly signalled in the slice header is being used.

A reference picture is referred to as included in the reference picture set of a coded picture if the reference picture is included in any of the six subsets of the reference picture set.

The following applies:

- If there are two reference pictures in the decoded picture buffer with the same pic_order_cnt value and that value is equal to RefPicSetLtCurr[ i ] for any i in the range of 0 to NumRpsLtCurr – 1, inclusive, or equal to RefPicSetLtFoll[ j ] for any j in the range of 0 to NumRpsLtFoll – 1, inclusive, the one that is marked as "used for long-term reference" is referred to as included in the long-term subset of the reference picture set.

- Otherwise if there is one reference picture in the DPB with the pic_order_cnt value equal to RefPicSetLtCurr[ i ] for any i in the range of 0 to NumRpsLtCurr – 1, inclusive, or equal to RefPicSetLtFoll[ j ] for any j in the range of 0 to NumRpsLtFoll – 1, inclusive, that reference picture is referred to as included in the long-term subset of the reference picture set. When that reference picture is a reference picture that is marked as "used for short-term reference" the marking is changed to "used for long-term reference".

A reference picture in the decoded picture buffer that is not included in the long-term subset of the reference picture set is referred to as included in the short-term subset of the reference picture set when its value of PicOrderCnt is equal to RefPicSetStCurr0[ i ] for any i in the range of 0 to NumRpsStCurr0 − 1, inclusive, or equal to RefPicSetStFoll0[ j ] for any j in the range of 0 to NumRpsStFoll0 − 1 or equal to RefPicSetStCurr1[ k ] for any k in the range of 0 to NumRpsStCurr1 − 1, inclusive or equal to RefPicSetStFoll1[ l ] for any l in the range of 0 to NumRpsStFoll1 − 1.

The derived reference picture set is restricted as follows:

- There shall be no reference picture with temporal_id greater than that of the current picture included in RefPicSetStCurr0, RefPicSetStCurr1 and RefPicSetLtCurr.

- There shall be no reference picture included in the reference picture set which precedes, in output order, any CRA picture that precedes the current picture both in decoding order and output order.

- A particular reference picture with a particular value of PicOrderCnt shall not be included in more than one of the six subsets of the reference picture set.

- A reference picture that is included in RefPicSetStCurr0, RefPicSetStCurr1 shall be present in the decoded picture buffer marked as "used for short-term reference".

- A reference picture that is included in RefPicSetLtCurr shall be present in the decoded picture buffer marked as "used for reference".

After the reference picture set is derived, the following applies:

- If the current picture is an IDR picture, all reference pictures are marked as "unused for reference".

- Otherwise (the current picture is not an IDR picture), all reference pictures marked as "used for reference" that are not included in the reference picture set are marked as "unused for reference".

NOTE 2 – There may be one or more reference pictures that are included in the reference picture set but that are not present in the decoded picture buffer. If such a reference picture is included in RefPicSetStFoll0, RefPicSetStFoll1 or RefPicSetLtFoll the absence should be ignored. If a reference picture that is included in the reference picture set but not present in the decoded picture buffer is included in RefPicSetStCurr0, RefPicSetStCurr1 or RefPicSetLtCurr, an unintentional picture loss should be inferred.

#### Initialisation process for reference picture lists

This initialisation process is invoked when decoding a P or B slice header.

- When decoding a P or B slice, there shall be at least one reference picture in RefPicSetStCurr0, RefPicSetStCurr1 or RefPicSetLtCurr.

The following procedure is conducted to construct the initial RefPicList0:

cIdx = 0
while( cIdx <= num_ref_idx_l0_active_minus1 )
{
	for( i=0; i < NumRpsStCurr0 && cIdx <= num_ref_idx_l0_active_minus1; cIdx++, i++ )
		RefPicList0[ cIdx ] = RefPicSetStCurr0[ i ] 
	for( i=0;  i < NumRpsStCurr1 && cIdx <= num_ref_idx_l0_active_minus1; cIdx++, i++ )
		RefPicList0[ cIdx ] = RefPicSetStCurr1[ i ] 
	for( i=0; i < NumRpsLtCurr && cIdx <= num_ref_idx_l0_active_minus1; cIdx++, i++ )
		RefPicList0[ cIdx ] = RefPicSetLtCurr[ i ]
}					(8-x)

The following procedure is conducted to construct the initial RefPicList1:

cIdx = 0
while( cIdx <= num_ref_idx_l1_active_minus1 )
{
	for( i=0; i < NumRpsCurr1 && cIdx <= num_ref_idx_l1_active_minus1; cIdx++, i++ )
		RefPicList1[ cIdx ] = RefPicSetCurr1[ i ] 
	for( i=0; i < NumRpsCurr0 && cIdx <= num_ref_idx_l1_active_minus1; cIdx++, i++ )
		RefPicList1[ cIdx ] = RefPicSetCurr0[ i ] 
}					(8-x)

#### Modification process for reference picture lists

When ref_pic_list_modification_flag_l0 is equal to 1, the following applies:

- Let refIdxL0 be an index into the reference picture list RefPicList0. It is initially set equal to 0.

- The corresponding syntax elements list_modification_idc are processed in the order they occur in the bitstream. For each of these syntax elements, the following applies.

–	If list_modification_idc is equal to 0 or equal to 1, the process specified in subclause 0.1.1.2 is invoked with refIdxL0 as input, and the output is assigned to refIdxL0.

–	Otherwise (list_modification_idc is equal to 2), the modification process for reference picture list RefPicList0 is finished.

When the current slice is a B slice and ref_pic_list_modification_flag_l1 is equal to 1, the following applies:

- Let refIdxL1 be an index into the reference picture list RefPicList1. It is initially set equal to 0.

- The corresponding syntax elements list_modification_idc are processed in the order they occur in the bitstream. For each of these syntax elements, the following applies.

–	If list_modification_idc is equal to 0 or equal to 1, the process specified in subclause 0.1.1.2 is invoked with refIdxL1 as input, and the output is assigned to refIdxL1.

–	Otherwise (list_modification_idc is equal to 2), the modification process for reference picture list RefPicList1 is finished.

#### Moving of one reference picture to a reference picture list index

Input to this process is an index refIdxLX (with X being 0 or 1).

Output of this process is an incremented index refIdxLX.

–	If list_modification_idc is equal to 0, the following applies.

–	If the current reference picture list is RefPicList0, curRefPicSet is set to RefPicSetCurr0.
–	Otherwise (the current reference picture list is RefPicList1), curRefPicSet is set to RefPicSetCurr1.

–	Otherwise, if list_modification_idc is equal to 1, the following applies.

–	If the current reference picture list is RefPicList0, curRefPicSet is set to RefPicSetSurr1.
–	Otherwise (the current reference picture list is RefPicList1), curRefPicSet is set to RefPicSetCurr0.

The variable pocLX is derived as follows.

pocLX = curRefPicSet[ ref_pic_set_idx ]		(8-x)

The following procedure is conducted to place the picture picR with PicOrderCnt( picR ) equal to pocLX into the index position refIdxLX, shift the position of any other remaining pictures to later in the list, and increment the value of refIdxLX.

for( cIdx = num_ref_idx_lX_active_minus1 + 1; cIdx > refIdxLX; cIdx-- )
	RefPicListX[ cIdx ] = RefPicListX[ cIdx − 1]
RefPicListX[ refIdxLX++ ] =  pocLX
nIdx = refIdxLX
for( cIdx = refIdxLX; cIdx <= num_ref_idx_lX_active_minus1 + 1; cIdx++ ) 	(Error! No text of specified style in document.-x)
	if( PicOrderCnt( RefPicListX[ cIdx ] ) != pocLX )
		RefPicListX[ nIdx++ ] = RefPicListX[ cIdx ]

NOTE – Within this pseudo-code procedure, the length of the list RefPicListX is temporarily made one element longer than the length needed for the final list. After the execution of this procedure, only elements 0 through num_ref_idx_lX_active_minus1 of the list need to be retained.