| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, CA, USA, 1-10 February, 2012 | Document: JCTVC-H0433 WG11 Number: m23309 |
| --- | --- |

| Title: | AHG21: Reference Picture Set Signaling Method |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande, Kiran Misra, Andrew Segall 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This document proposes syntax to signal a delete operation for removing select reference pictures from a previously signaled reference picture set (RPS). This approach can be used to signal a new partial RPS by referring to a RPS signaled in PPS. It is asserted that this change supports efficient signaling of reference picture set for the current picture if the desired reference picture set differs from that defined in the picture parameter set for only a small subset of entries.

# Introduction

JCTVC-F493 [1] proposed absolute signaling of reference pictures, which supports error robust management of reference pictures using Buffer Descriptions.

AHG21 WD [2] and output text [3] does not support signaling a delete operation on a previously signaled (via picture parameter set) reference picture set (RPS) to signal a new partial RPS.

JCTVC-G637 [4] proposed pruning of reference picture sets. The use case considered was first GOP following a random access point where some of the pictures signaled in RPS are not available. JCTVC-G637 proposes a syntax, which can signal number of short-term reference pictures that are omitted from a particular RPS signaled in the PPS.

However the syntax in JCTVC-G637 only supports omitting consecutive negative short-term reference pictures from a reference picture set. There are situations where a selective omission of one or more reference pictures from a reference picture set is desired.

Thus we propose a method to signal delete operation on a previously signaled RPS to efficiently signal a new RPS.

Our approach is useful in following use cases:

- U1: Signaling RPS for pictures in first GOP after a random access point.

- U2: Signaling RPS efficiently via a delete operation on a previous signaled RPS for the case where a reference picture was lost and was signaled by decoder to the encoder via feedback and the lost picture is no longer used as a reference picture by the encoder. (e.g. use case 3.3 in JCTVC-G1036d03 [5] and similar use case in non low-delay conditions).

- U3: A memory optimizing encoder, which uses one (or more) LTRP for portion of the video.

Our proposed signaling (explained in section 2 below) consists of a flag in PPS and in slice header and a bit field in slice header for each picture in the indicated RPS. The bit field signals 1 or 0 values to indicate keeping/ deleting corresponding reference picture from the indicated reference picture set.

We will provide more explanation about use cases supported by our proposed approach next.

- Use Case U1: Signaling RPS for picture in first GOP after RAP  (e.g. use case 2.1 in JCTVC-G1036d03)

For the first GOP after random access point some of the reference pictures indicated in RPS are not available. This can be signaled efficiently using our proposed method. As an example for first GOP, for picture with POC 4 the reference picture set {-4, 4} can be signaled efficiently by referring to RPS {-4,-6,4} sent in PPS and signaling a delete for -6. This will be more efficient than signaling the new correct RPS  {-4,4} in slice header.

First GOP (also applies to first GOP after a random access point including CRA)

| POC | pic type | nal_ref_flg | t_id | RefPicList0 | RefPicList1 | Other pictures marked as “used for reference” | Reference Picture Set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | IDR | 1 | 0 | - | - |  |  |
| 8 | B | 1 | 0 | 0 | 0 |  | -8, -10,-12,-16 |
| 4 | B | 1 | 0 | 0 8 | 8 0 |  | -4, -6,+4 |
| 2 | B | 1 | 0 | 0 4 | 4 8 |  | -2, -4,2,6 |
| 1 | b | 0 | 0 | 0 2 | 2 4 | 8 | -1,1,3,7 |
| 3 | b | 0 | 0 | 2 0 | 4 8 |  | -1,-3,1,5 |
| 6 | B | 1 | 0 | 4 2 | 8 4 | 0 | -2,-4,-6,2 |
| 5 | b | 0 | 0 | 4 0 | 6 8 |  | -1,-5,1,3 |
| 7 | b | 0 | 0 | 6 4 | 8 6 | 0 | -1,-3,-7,1 |
| 16 | B | 1 | 0 | 8 6 4 0 | 8 6 4 0 |  | -8,-10,-12,-16 |

The comparison of the bits required using our approach versus signaling these (new) partial RPS via slice header is shown below:

|  | Bits to signal partial RPS | Bits to signal partial RPS by reference and proposed delete operation in slice header |
| --- | --- | --- |
| {-8} | 12 | 5 |
| {-4,4} | 18 | 4 |
| {-2,2,6} | 20 | 5 |

- Use Case U2: Signaling RPS via a delete operation on a previous signaled RPS for the case where a reference picture was lost (e.g. use case similar to 3.3 in JCTVC-G1036d03 and similar use case in non low delay conditions)

Without any picture loss:

| POC | pic type | nal_ref_flg | t_id | RefPicList0 | RefPicList1 | Other pictures marked as “used for reference” | Reference Picture Set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 16 | B | 1 | 0 | 8 6 4 0 | 8 6 4 0 |  | -8,-10,-12,-16 |
| 12 | B | 1 | 0 | 8 6 | 16 8 |  | -4,-6,+4 |
| 10 | B | 1 | 0 | 8 6 | 12 16 |  | -2,-4,2,6 |
| 9 | b | 0 | 0 | 8 10 | 10 12 | 16 | -1,1,3,7 |
| 11 | b | 0 | 0 | 10 8 | 12 16 |  | -1,-3,1,5 |
| 14 | B | 1 | 0 | 12 10 | 16 12 | 8 | -2,-4,-6,2 |
| 13 | b | 0 | 0 | 12 8 | 14 16 |  | -1,-5,1,3 |
| 15 | b | 0 | 0 | 14 12 | 16 14 | 8 | -1,-3,-7,1 |
| 24 | B | 1 | 0 | 16 14 12 8 | 16 14 12 8 |  | -8,-10,-12,-16 |

With picture corruption/ loss: Assume picture with POC 6 was received corrupted/ lost at the decoder, this was signaled by decoder by feedback to the encoder and encoder then did not use picture 6 as reference picture when encoding next GOP – including pictures with POC 10, 12, 16.

| POC | pic type | nal_ref_flg | t_id | RefPicList0 | RefPicList1 | Other pictures marked as “used for reference” | Reference Picture Set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 16 | B | 1 | 0 | 8 4 0 | 8 4 0 |  | -8, -10,-12,-16 |
| 12 | B | 1 | 0 | 8 | 16 8 |  | -4, -6,+4 |
| 10 | B | 1 | 0 | 8 | 12 16 |  | -2, -4,2,6 |
| 9 | b | 0 | 0 | 8 10 | 10 12 | 16 | -1,1,3,7 |
| 11 | b | 0 | 0 | 10 8 | 12 16 |  | -1,-3,1,5 |
| 14 | B | 1 | 0 | 12 10 | 16 12 | 8 | -2,-4,-6,2 |
| 13 | b | 0 | 0 | 12 8 | 14 16 |  | -1,-5,1,3 |
| 15 | b | 0 | 0 | 14 12 | 16 14 | 8 | -1,-3,-7,1 |
| 24 | B | 1 | 0 | 16 14 12 8 | 16 14 12 8 |  | -8,-10,-12,-16 |

Thus in the above use case for pictures 16, 12 and 10 the correct RPS can be signaled efficiently by referring to RPS sent in PPS (respectively {-8,-10,-12,-16} with our proposed delete signaling to delete    -10, {-4,-6,4} with our proposed delete signaling to delete -6, {-2,-4,2,6} with our proposed signaling to delete -4).

This will be more efficient than signaling the new correct RPS {-8, -12, -16}, {-4,4}, {-2,2,6} in slice header for each of these pictures.

The comparison of the bits required using our approach versus signaling these partial RPS via slide header is shown below:

|  | Bits to signal partial RPS | Bits to signal partial RPS by reference and proposed delete operation in slice header |
| --- | --- | --- |
| {-8,-12,-16} | 26 | 5 |
| {-4,4} | 18 | 4 |
| {-2,2,6} | 20 | 5 |

- Use Case U3: A memory optimizing encoder which uses one (or more) LTRP for portion of the video may decide to keep the decoder picture buffer memory requirement same as when using only STRPs. For example if initially the encoder is using random access configuration for encoding as in JCTVC-G1036d03 section 2.1 then decoder picture buffer needs memory to store 5 STRPs. Suppose for a portion of the video, encoder decides to use one LTRP, then to keep the same reduced memory requirement for the decoder (i.e. not to increase it to 6 frames) encoder may decide to only use 4 STRPs during this time when 1 LTRP is used. In this case the encoder will choose not to use one of the short-term reference pictures when encoding. Encoder can efficiently signal this by referring to RPS sent in PPS and signaling a delete operation for one of the STRPs using our proposed syntax.

# Proposed Change

We propose following syntax and semantics to signal a delete operation for removing select reference picture(s) from a previously signaled reference picture set (RPS).

Detailed text change document including decoding process for reference picture set with respect to output text from Geneva meeting [3] is also separately enclosed.

Also source code for the proposed functionality is implemented. It is provided as a patch to the source code branch HM-5.1-dev-ahg21.

## Syntax and Semantics:

We define the appropriate syntax and semantics based on syntax defined in output text [3].

### Picture Parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| entropy_coding_mode_flag | u(1) |
| num_short_term_ref_pic_sets | ue(v) |
| for(idx = 0; idx < num_short_term_ref_pic_sets; idx++) |  |
| short_term_ref_pic_set( idx ) |  |
| delete_pics _info_present_flag | u(1) |
| long_term_ref_pics_present_flag | u(1) |
| num_temporal_layer_switching_point_flags | ue(v) |
| for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ) |  |
| temporal_layer_switching_point_flag[ i ] | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| pic_init_qp_minus26 /* relative to 26 */ | se(v) |
| constrained_intra_pred_flag | u(1) |
| slice_granularity | u(2) |
| shared_pps_info_enabled_flag | u(1) |
| if( shared_pps_info_enabled_flag ) |  |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_param( ) |  |
| if( cu_qp_delta_enabled_flag ) |  |
| max_cu_qp_delta_depth | u(4) |
| rbsp_trailing_bits( ) |  |
| } |  |

delete_pics_info_present_flag equal to 1 specifies that slice header may signal delete operation from the short-term reference picture set included in the picture parameter set. delete_pics_info_present_flag equal to 0 specifies that no delete operation is signaled in the slice header for the short-term reference picture set included in the picture parameter set.

### Slice header RBSP syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| if( IdrPicFlag ) { |  |
| idr_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } |  |
| else { |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_pps_flag | u(1) |
| if( !short_term_ref_pic_set_pps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else |  |
| short_term_ref_pic_set_idx | u(v) |
| if( delete_pics_info_present_flag ) { |  |
| keep_pics_info_present_flag | u(1) |
| if( keep_pics_info_present_flag ) { |  |
| for( j = 0; j < (num_negative_pics+num_positive_pics); j++ ) { |  |
| keep_pic_flag[j] | u(1) |
| } |  |
| } |  |
| } |  |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |
| delta_poc_lsb_lt_minus1[ i ] | ue(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

keep_pics_info_present_flag equal to 1 specifies that this slice header includes keep_pic_flag values indicating information regarding keeping or deleting short term reference pictures from the reference picture set with index short_term_ref_pic_set_idx. keep_pics_info_present_flag equal to 0 specifies that this slice header does not include information regarding keeping or deleting short term reference pictures from the reference picture set with index short_term_ref_pic_set_idx.

keep_pic_flag[j] equal to 1 specifies that the corresponding j th short term reference picture from the reference picture set in the picture parameter set with index short_term_ref_pic_set_idx should be kept. keep_pic_flag[j] equal to 0 specifies that the corresponding j th short term reference picture from the reference picture set in the picture parameter set with index short_term_ref_pic_set_idx should be deleted (omitted).

(Note: It is possible to define alternate syntax where the keep_pic_flag[j] is signaled only for the negative pictures.

Also it is possible to define alternate syntax where only keep_pics_info_present_flag is sent in slice header with no delete_pics _info_present_flag sent in PPS.)

# Conclusion

The document proposed syntax to efficiently signal a delete operation for removing select reference pictures from a previously signaled reference picture set (RPS). This approach can be used to efficiently signal a new partial RPS by referring to a RPS signaled in PPS. We propose to adopt the proposed approach in HEVC.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] R. Sjöberg, J. Samuelsson (Ericsson), "Absolute signaling of reference pictures," JCTVC-F493/m20923 6th Meeting: Torino, IT, 14-22 July, 2011.

[2] ftp://ftp.hhi.de/ahg21/JCTVC-F803_d5_RPS_r2.doc Candidate Working Draft text by AHG 21, November 2011

[3] JCTVC-G1002-output-WD-text-r1.doc, Candidate output text by AHG21, November 2011

[4] R. Sjöberg, J. Samuelsson (Ericsson), “AHG21: Long term pictures and pruning of reference picture sets,” JCTVC-G637, 7th meeting Geneva, CH, 21-30 November, 2011

[5] Y. K. Wang, M. M. Hannuksela, T. K. Tan, R. Sjöberg, “Common conditions for reference picture marking and list construction proposals,” JCTVC-G1036d03, 7th meeting Geneva, CH, 21-30 November, 2011