| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0434 M23310 |
| --- | --- |

| Title: | CE8 Subset c: Necessity of Sign Bits for SAO Offsets |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim Do-Kyoung Kwon 12500 TI Blvd MS 8649 Dallas TX USA 75243 | Tel: Email: | 1-214-480-1562 wskim@ti.com d-kwon@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution presents the results of CE8 subset c to investigate the necessity of sign bits for sample adaptive offset (SAO), which was proposed in JCTVC-G680. SAO usually improves overall subjective quality. However, it is asserted that it can cause visual artifact occasionally by adding offset value determined globally in the given region to pixels of which statistics are very different from that of global ones. In this contribution the cause of such a coding artifact is analyzed, and a method to remove this artifact is proposed. In the proposed method the sign value of edge offset is restricted according to edge category, so that contrast between the current and neighboring pixels is not increased after adding the offset. It is reported that in 99.4% cases the sign matches to edge category, and the artifact occurs among the other 0.6% cases. Therefore, it is proposed to remove sign coding part to simplify the current SAO design in HM-5.0. It is reported that subjective tests reveal that the SAO visual artifact is removed by employing the proposed method. Experimental results also reportedly show that the proposed change does not compromise overall subjective quality and coding efficiency (Average BD-rate increase of 0.0% for luma and 0.2% for chroma).

# Introduction

Sample adaptive offset (SAO) [1] is a method compensating reconstructed pixel value by adding an offset according to each pixel’s context such as edge shape and direction in case of an edge offset (EO), and pixel intensity level band in case of a band offset (BO). The offset type and values are determined for each quad-tree partition according to local characteristics. SAO generally improves both subjective and objective qualities; however, we observed that it occasionally results in visual artifact.

Fig. 1 shows an example of the visual artifact caused by SAO, where various sequences are coded using HM-5.0. There are a few noticeable dots like pepper and salt noise. Once this artifact appears, it can propagate to future frames, which makes this artifact more visible. Moreover, this artifact can be emphasized by sharpness enhancement and/or edge strengthening filters, which are commonly used as post-processing filters in consumer appliances such as a digital TV.

In this document, we explain the reason for this visual artifact, and propose a solution to remove this artifact. Experimental results show that the artifact is effectively removed without compromising overall quality and complexity increment.

(a) BasketballDrive AI-HE QP37 Frame 286

(b) BQMall LB-HE QP37 Frame 29

(c) BasketballDrive LB-HE QP37 Frame 423

(d) BasketballDrillText LB-HE QP37 Frame 351

Fig. 1. Visual artifact in sample adaptive offset.

# Proposed Method

## Reason for visual artifact in SAO

As shown in Fig. 1, SAO occasionally produces visual artifact. We have investigated the reason of the artifact in SAO, and found that EO causes this problem. Fig. 2 shows EO categorization scheme according to edge shape. Each category has one offset value calculated by taking an average of differences between the values of original and reconstructed pixels belonging to each category in a partition. By adding the offset to each reconstructed pixel according to its category, the overall distortion in the partition can be reduced.

Fig. 2. Edge offset categorization according to edge shape.

However, if there is a region in a partition which has very different statistics from that of the other region in the partition, adding offset would degrade quality in this particular region. The visual artifact occurs when applying this inappropriate offset value to outlier pixels increases contrast between the current pixel and the neighboring pixels.

For example, in case of category 1 in Fig. 2, if an offset value has positive sign, this will reduce contrast between neighboring pixels, and causes no visual artifact. On the other hand, when it is a negative value, applying this offset will increase contrast between neighboring pixels, which results in visual artifact.

## Proposed solution to remove visual artifact in SAO

The edge shapes in each category illustrated in Fig. 2 suggest that the sign of offset can be related to the shape of the edge. For example, in category 1 and 2, the reconstructed pixel level is lower than the neighboring pixels. Therefore, the offset would tend to have positive sign. On the other hand, in category 3 and 4, the offset would have negative sign. We have verified this experimentally, and found the offset sign matches the category in 99.4% cases on average, i.e., 0 or positive offsets for categories 1 and 2, and 0 or negative offsets for categories 3 and 4. The statistics for all test configurations are given in Table 1.

Table 1. Statistics of edge offset sign match to edge category

|  | Y |  |  |  | Cb |  |  |  | Cr |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EO | 1 | 2 | 3 | 4 | 1 | 2 | 3 | 4 | 1 | 2 | 3 | 4 |
| AI-HE | 99.7 | 100.0 | 100.0 | 99.9 | 99.1 | 99.8 | 99.8 | 98.8 | 98.7 | 99.8 | 99.8 | 99.1 |
| RA-HE | 97.1 | 100.0 | 100.0 | 98.5 | 96.8 | 99.9 | 99.9 | 98.7 | 97.6 | 99.9 | 99.9 | 98.6 |
| LB-HE | 98.5 | 99.9 | 100.0 | 99.3 | 98.8 | 100.0 | 100.0 | 99.1 | 98.6 | 99.9 | 100.0 | 99.0 |
| LP-HE | 99.2 | 100.0 | 100.0 | 99.7 | 99.4 | 100.0 | 100.0 | 99.4 | 99.0 | 100.0 | 100.0 | 99.4 |
| Ave. | 98.6 | 100.0 | 100.0 | 99.3 | 98.5 | 99.9 | 99.9 | 99.0 | 98.5 | 99.9 | 99.9 | 99.0 |

However, as explained above, if a negative offset applies to pixels in category 1 and 2, this will increase contrast between neighboring pixels, and can result in visual artifact. Same will happen when a positive offset applies to pixels in category 3 and 4.

Therefore, we propose to restrict the offset values to be 0 or positive values for category 1 and 2, and 0 or negative values for category 3 and 4. Same scheme is applied for all color components. This also eliminates coding of sign value in entropy coding, which simplifies the current SAO design in HM-5.0.

Note that this only applies to the edge offset case. In case of band offset, the sign value will be coded.

# Experimental Results

The proposed solution is implemented using HM-5.0 reference code. We have performed subjective quality evaluation using the test sequences used in [2], and first verified that the visual artifact due to SAO is effectively removed as shown in Fig. 3. In addition, we observed there is subjective quality improvement in a few sequences, and no sequence suffers quality degradation due to the proposed method. Table 2 summarizes subjective quality test results.

Table 2. Vote percentage (%) in the subjective quality test results.

|  | Anchor | G680 | Neutral |
| --- | --- | --- | --- |
| G680 EncOnly | 20 | 23 | 57 |
| HM-5.0 | 21 | 27 | 52 |

(a) BasketballDrive AI-HE QP37 Frame 286

(b) BQMall LB-HE QP37 Frame 29

(c) BasketballDrive LB-HE QP37 Frame 423

(d) BasketballDrillText LB-HE QP37 Frame 351

Fig. 3. Visual artifact in sample adaptive offset.

Table 2 is the summary of the simulation results in terms of the objective quality generated using the common test conditions [3]. The simulations have used HM-5.0 software with the AMP encoder speedup disabled. The simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570 64 bits Linux machines of different frequencies. On average there is 0.0% bitrate reduction in Y, and 0.2% and 0.1% bitrate increment in U and V, respectively.

Table 2. BD-rate results of the proposed method compared to the encoder only method.

|  | All Intra HE |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 99% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.0% |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | -0.1% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class F | -0.1% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 99% |  |  | 101% |  |  |
| Dec Time[%] | 95% |  |  | 103% |  |  |
|  | Low delay B HE |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | -0.4% |  |  |  |
| Class C | 0.0% | 0.1% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | -0.1% | 0.5% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.1% | 0.0% |  |  |  |
| Class F | 0.0% | -0.3% | 0.2% |  |  |  |
| Enc Time[%] | 98% |  |  |  |  |  |
| Dec Time[%] | 97% |  |  |  |  |  |
|  | Low delay P HE |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | -0.2% |  |  |  |
| Class C | 0.0% | -0.2% | -0.1% |  |  |  |
| Class D | 0.0% | -0.2% | -0.2% |  |  |  |
| Class E | 0.0% | 0.1% | -0.2% |  |  |  |
| Overall | 0.0% | -0.1% | -0.2% |  |  |  |
|  | 0.0% | -0.1% | -0.2% |  |  |  |
| Class F | 0.0% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | 98% |  |  |  |  |  |
| Dec Time[%] | 95% |  |  |  |  |  |

Table 3. BD-rate results of the proposed method compared to HM-5.0.

|  | All Intra HE |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 99% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 1.5% | 1.2% |
| Class B | 0.0% | 0.2% | 0.2% | 0.0% | 0.2% | 0.0% |
| Class C | -0.1% | 0.1% | 0.2% |  |  |  |
| Class D | 0.0% | 0.0% | 0.1% |  |  |  |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.8% | 0.5% |
|  | 0.0% | 0.1% | 0.1% | 0.0% | 0.7% | 0.5% |
| Class F | -0.1% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | 101% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 107% |  |  |
|  | Low delay B HE |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.6% | 0.3% |  |  |  |
| Class C | 0.0% | 0.4% | 0.5% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | -0.1% | 0.8% |  |  |  |
| Overall | 0.0% | 0.3% | 0.4% |  |  |  |
|  | 0.0% | 0.3% | 0.3% |  |  |  |
| Class F | -0.1% | 0.3% | 0.2% |  |  |  |
| Enc Time[%] | 100% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  |  |  |  |
|  | Low delay P HE |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.5% | 0.2% |  |  |  |
| Class C | 0.0% | 0.3% | 0.1% |  |  |  |
| Class D | 0.0% | -0.1% | -0.2% |  |  |  |
| Class E | 0.1% | -0.1% | 0.8% |  |  |  |
| Overall | 0.0% | 0.2% | 0.2% |  |  |  |
|  | 0.0% | 0.2% | 0.1% |  |  |  |
| Class F | 0.0% | 0.7% | 0.4% |  |  |  |
| Enc Time[%] | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |  |

# Conclusion

SAO occasionally causes visual artifact by increasing contrast between neighboring pixels. We have proposed to restrict the offset to have positive sign for edge offset category 1 and 2, and negative sign for category 3 and 4. Experimental results show that the proposed method effectively removes the visual artifact without compromising visual quality. This also simplifies the current SAO design by eliminating sign coding part.

# Reference

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “WD5: Working Draft 5 of High-Efficiency Video Coding,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-G1103, Nov. 2011.

- W.-S. Kim, O. G. Sezer, and M. Budagavi, “Subjective Tests on ALF and SAO Using HM-4.0”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-G684, Nov., 2011.

- F. Bossen, “Common test conditions and software reference configurations,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-G1200, Nov. 2011.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD Text Changes (changes marked in yellow)

- 7.3.3.4 Sample adaptive offset parameter syntax

| sao_offset_param( rx, ry, saoDepth , cIdx ) { | Descriptor |
| --- | --- |
| if( sao_split_flag[ cIdx ][ saoDepth ][ rx ][ ry ] ) { |  |
| sao_offset_param( 2*rx + 0, 2*ry + 0, saoDepth + 1 , cIdx ) |  |
| sao_offset_param( 2*rx + 1, 2*ry + 0, saoDepth + 1 , cIdx ) |  |
| sao_offset_param( 2*rx + 0, 2*ry + 1, saoDepth + 1 , cIdx ) |  |
| sao_offset_param( 2*rx + 1, 2*ry + 1, saoDepth + 1 , cIdx ) |  |
| } else { |  |
| sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] | ue(v) ¦ ae(v) |
| if( sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] != 0 ) |  |
| if( sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] > 0 && sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] < 5 ) |  |
| for( i = 0; i < NumSaoClass[ sao_type_idx ]; i++ ) |  |
| sao_offset[ cIdx ][ saoDepth ][ x0 ][ y0 ][ i ] | ue(v) ¦ ae(v) |
| else if(sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] > 4 ) |  |
| for( i = 0; i < NumSaoClass[ sao_type_idx ]; i++ ) |  |
| sao_offset[ cIdx ][ saoDepth ][ x0 ][ y0 ][ i ] | se(v) ¦ ae(v) |
| } |  |
| } |  |

- 7.4.3.4 Sample adaptive offset parameter semantics

sao_offset[ cIdx ][ saoDepth ][ rx ][ ry ][ i ] indicates the offset value of i-th category for the color component cIdx of the region specified by saoDepth, rx and ry.

The variable bitDepth is derived as follows.

- If cIdx is equal to 0, bitDepth is set equal to BitDepthY..

- Otherwise (cIdx is equal to1 or 2), bitDepth is set equal to BitDepthC.

The offset value shall be in the range of [ -( 1 << ( SaoBitsRange – 1) ), ( 1<< ( SaoBitsRange – 1) ) – 1 ] where

sao_offset[ cIdx ][ saoDepth ][ rx ][ ry ][ i ] shall be in the range of [ 0, ( 1<< ( SaoBitsRange – 1) ) – 1 ] when sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] is 1, 2, 3, or 4, otherwise, in the range of [ -( 1 << ( SaoBitsRange – 1) ), ( 1<< ( SaoBitsRange – 1) ) – 1 ], where

SaoBitRange = Min( bitDepth, 10 ) – 4		(7-1)

An array SaoOffsetVal is specified as

SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ][ 0 ] = 0		(7-2)

if( sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] > 4 | | i < 2 ) )

SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ][ i + 1 ] =

sao_offset[ cIdx ][ saoDepth ][ rx ][ ry ][ i ] << ( bitDepth – Min( bitDepth, 10 ) )

else

SaoOffsetVal[ cIdx ][ saoDepth ][ rx ][ ry ][ i + 1 ] =

-1 * sao_offset[ cIdx ][ saoDepth ][ rx ][ ry ][ i ] << ( bitDepth – Min( bitDepth, 10 ) )

with i = 0..NumSaoCategory – 1