| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0438 M23314 |
| --- | --- |

| Title: | Non-CE8: New Chroma Band Offset (BO) Design in SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim Texas Instruments Inc., USA Do-Kyoung Kwon Texas Instruments Inc., USA Yukinobu Yasugi SHARP Corporation, JAPAN Tomohiro Ikai SHARP Corporation, JAPAN | Tel: Email: Tel: Email: Tel: Email: Tel: Email: | +1-214-480-1562 wskim@ti.com +1-214-480-1479 d-kwon@ti.com +81 43 299 8526 yasugi.yukinobu@sharp.co.jp +81 43 299 8526 ikai.tomohiro@sharp.co.jp |
| Source: | Texas Instruments Inc., SHARP Corporation |  |  |

_____________________________

# Abstract

In JCTVC-G682, it has been proposed to reduce BO buffer size by decreasing the number of band offsets from 16 to 8 while increasing the number of bands from 2 to 4. In this contribution, on top of JCTVC-G682, it is proposed a new BO design for Chroma to improve Chroma coding efficiency. It is asserted that the majority of reconstructed Chroma pixels have the values between 64 and 191 in case of 8-bit pixel. Based on this observation, this proposal excludes the lowest range (e.g. 0 ~ 63) and the highest range (e.g. 192 ~ 255) from Chroma BO table, and then divide the middle range (e.g. 64 ~ 191) into 4 bands. Each band is equally divided into 8 sub-bands again. No change is applied for Luma. Experimental results are asserted to show that the BD-rate reduction (averaged between U and V channel) by the proposed method is 0.7%, 0.8%, 1.7%, 1.5% and 0.9% in AI-HE, RA-HE, LB-HE, LP-HE and RA-HE10 respectively.

# Introduction

Sample adaptive offset (SAO) [1] improves visual quality by adding an offset to each reconstructed pixel. SAO parameters are encoded and included in adaptation parameter set (APS). At a decoder side, SAO parameters have to be stored in a buffer after decoding APS until SAO process is completed for each slice. The buffer size is proportional to the number of partitions and size of SAO parameters. In many implementations, the buffer has to be prepared beforehand, and need to provide enough space to store information in worst case. The worst case happens when a slice or frame is divided into maximum number of partitions, and all the partition uses maximum number of offsets.

In the current SAO design, BO requires 16 offsets to send. Considering edge offset (EO) only uses 4 offsets, BO seems to use too many offsets, and this causes large buffer size to store them. For this reason, in JCTVC-G682 [2], we proposed to reduce the number of band offsets from 16 to 8 for Luma and Chroma as shown in Fig. 1. This resulted in reducing the buffer size to store offsets by half.

Furthermore, we analyzed the statistics of reconstructed pixels and observed that the majority of reconstructed Chroma pixels are in the middle bands. In this contribution, based on our analysis, we propose a new BO design for Chroma. Experimental results show that the BD-rate reduction (averaged between U and V channel) by the proposed method is 0.7 %, 0.8 %, 1.7 %, 1.5 % and 0.9 % in AI-HE, RA_HE, LB_HE, LP_HE and RA_HE10 respectively.

Proposed Method

Fig. 1. Band division for Luma and Chroma BO in JCTVC-G682

The statistics of reconstructed pixels of Luma and Chroma are quite different. Figs. 2 and 3 show histograms of reconstructed Luma and Chroma (U) pixels, respectively, from several sequences. We classified all pixels according to the BO type of partition where each pixel is included and the histogram of each band is shown separately in these figures.

Fig. 2. Histograms of reconstructed Luma pixels in different BO types

As shown in Figs. 2 and 3, while reconstructed Luma pixels spread out a whole dynamic range, reconstructed Chroma pixels concentrate on the mid ranges. It implies that Chroma BO does not need to cover the lowest band (BO 3 in Fig. 1) and the highest band (BO 4 in Fig. 1) and that the coding efficiency can be improved by removing these two bands.

Based on this observation, we propose to use the different BO designs for Luma and Chroma, respectively. For Luma, we apply the BO design in Fig. 1. However, for Chroma, we excludes the lowest band (BO 3 in Fig. 1) and the highest band (BO 4 in Fig. 1), and then divide the middle 2 bands (BO 1 and BO 2 in Fig. 1) into new 4 bands. Each band is equally divided into 8 sub-bands again. The proposed Luma and Chroma BO designs are illustrated in Fig. 4.

Compared to the current BO design in HM-5.0, the proposed method has two advantages. First, we can reduce the buffer size that a decoder should have to store band offsets by half. Second, it is possible to estimate band offsets more accurately for Chroma because the each sub-band becomes narrower for Chroma, and it results in the coding efficiency improvement.

Fig. 3. Histograms of reconstructed Chroma (U) pixels in different BO types

(a) Band division for Luma BO in the proposed method

(b) Band division for Chroma BO in the proposed method

Fig. 4. Band division in the proposed method

# Experimental Results

Table 1 is the summary of the experimental results in terms of objective quality generated using the common test conditions [3]. The simulations have used HM-5.0 software with the AMP encoder speedup disabled. The simulation platform is LSF equipped with 64 bits Linux machines of different CPUs (Intel Core2 quad and Xeon) and frequencies. On average there is 0.0 %, 1.1 % and 1.1 % bitrate reduction in Y, U and V, respectively.

Table 1. BD-rate results of the proposed method compared to HM-5.0.

|  | All Intra HE |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U |  |
| Class A (8bit) | 0.0% | -0.2% | -0.6% |  |  |  |
| Class B | 0.1% | -0.8% | -0.8% |  |  |  |
| Class C | 0.0% | -0.4% | -0.5% |  |  |  |
| Class D | 0.1% | -0.4% | -0.4% |  |  |  |
| Class E | 0.1% | -1.1% | -1.1% |  |  |  |
| Overall | 0.1% | -0.6% | -0.7% |  |  |  |
|  | 0.1% | -0.6% | -0.7% |  |  |  |
| Class F | 0.3% | -0.2% | 0.2% |  |  |  |
| Enc Time[%] | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.3% | -0.6% | 0.1% | -0.7% | -0.9% |
| Class B | 0.0% | -1.3% | -1.1% | 0.0% | -1.0% | -0.9% |
| Class C | 0.0% | -0.7% | -0.5% |  |  |  |
| Class D | 0.0% | -0.6% | -0.8% |  |  |  |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.8% | -0.8% | 0.1% | -0.9% | -0.9% |
|  | 0.0% | -0.8% | -0.8% | 0.1% | -0.9% | -0.9% |
| Class F | 0.2% | -0.5% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -2.1% | -1.7% |  |  |  |
| Class C | 0.0% | -1.4% | -1.3% |  |  |  |
| Class D | 0.0% | -0.9% | -1.4% |  |  |  |
| Class E | 0.0% | -2.6% | -2.4% |  |  |  |
| Overall | 0.0% | -1.7% | -1.6% |  |  |  |
|  | 0.0% | -1.7% | -1.7% |  |  |  |
| Class F | 0.1% | -1.0% | -1.2% |  |  |  |
| Enc Time[%] | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |
|  | Low delay P HE |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -1.9% | -1.9% |  |  |  |
| Class C | 0.0% | -1.3% | -1.5% |  |  |  |
| Class D | 0.0% | -0.9% | -1.9% |  |  |  |
| Class E | -0.1% | -2.2% | -2.5% |  |  |  |
| Overall | 0.0% | -1.6% | -1.9% |  |  |  |
|  | 0.0% | -1.6% | -1.9% |  |  |  |
| Class F | 0.1% | -0.9% | -0.7% |  |  |  |
| Enc Time[%] | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |

# Conclusion

A new band offset scheme has been proposed. The number of bands is increased from 2 to 4 and the number of sub-bands is reduced from 16 to 8, so that the buffer size to store these offsets can be reduced by half. Moreover, the different BO scheme is applied to Chroma based on the statistics of reconstructed Chroma pixels. We only consider the middle pixel ranges in the design of Chroma BO and divide Chroma bands more finely. Experimental results show that the proposed method improves the coding efficiency.

# Reference

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “WD5: Working Draft 5 of High-Efficiency Video Coding,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-G1103, Nov. 2011.

- W.-S. Kim and D.-K. Kwon, “Reduced number of band offsets in SAO”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-G682, Nov., 2011.

- F. Bossen, “Common test conditions and software reference configurations,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-G1200, Nov. 2011.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD Text Changes (changes marked in yellow)

7.4.3.4	Sample adaptive offset parameter semantics

…

Table 7-1 – Specification of NumSaoClass

| - sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] | - NumSaoCategory | - Edge type (informative) |
| --- | --- | --- |
| - 0 | - 0 | - Not applied |
| - 1 | - 4 | - 1D 0-degree edge |
| - 2 | - 4 | - 1D 90-degree edge |
| - 3 | - 4 | - 1D 135-degree edge |
| - 4 | - 4 | - 1D 45-degree edge |
| - 5 | - 8 | - Central left band |
| - 6 | - 8 | - Central right band |
| - 7 | - 8 | - Side left band |
| - 8 | - 8 | - Side right band |

8.6.2.1.1	Modification process for luma and chroma samples

…

–	Otherwise, if saoTypeIdx is equal to one of the values of 5, 6, 7, and 8, the following ordered steps applies:

…

Table 8-2 – Specification of array bandTable for Luma according to bandIdx

| bandIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bandTable[0][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| bandTable[1][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandTable[2][bandIdx] | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandTable[3][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandIdx | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| bandTable[0][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandTable[1][bandIdx] | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandTable[2][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandTable[3][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |

Table 8-3 – Specification of array bandTable for Chroma according to bandIdx

| bandIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bandTable[0][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandTable[1][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandTable[2][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandTable[3][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandIdx | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| bandTable[0][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| bandTable[1][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandTable[2][bandIdx] | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandTable[3][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandIdx | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| bandTable[0][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandTable[1][bandIdx] | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandTable[2][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandTable[3][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| bandIdx | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| bandTable[0][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandTable[1][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandTable[2][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandTable[3][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |