| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0439 M23315 |
| --- | --- |

| Title: | Non-CE8: Crosscheck of Panasonic’s method of removing boundary artifacts for SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Woo-Shik Kim 12500 TI Blvd MS 8649 Dallas TX USA 75243 | Tel: Email: | 1-214-480-1562 d-kwon@ti.com wskim@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution provides cross-check results for Panasonic’s proposal in JCTVC-H0201 [1], in which artifacts around partition and category boundaries by band offset (BO) were reported and the methods to remove them were proposed. We confirmed that these artifacts are caused by BO and removed by the proposed methods. The experimental results perfectly matched with the one provided by Panasonic in terms of rate-distortion performance.

# Methods

Artifacts around partition and category boundary are caused by the current BO design. To remove such boundary artifacts, the offsets around these boundaries are scaled by weighting factor less than 1.

# Experimental results

It was confirmed first that the current BO design causes boundary artifacts and the proposed methods remove them well using the software provided by Panasonic. Its performance relative to HM-5.0 software was checked under the common test condition described in [2]. The experiment platform is LSF equipped with Intel(R) Xeon(R) CPU X5570 64 bits Linux machines of different frequencies.

Table 1 Test-1: BD-rate results of the proposal method for partition boundary

|  | All Intra HE |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% |  |  |  |
| Class C | 0.0% | 0.1% | 0.1% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% |  |  |  |
|  | 0.0% | 0.0% | 0.1% |  |  |  |
| Class F | 0.1% | 0.2% | 0.3% |  |  |  |
| Enc Time[%] | 101% |  |  |  |  |  |
| Dec Time[%] | 103% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.1% | 0.2% |  |  |  |
| Class D | 0.0% | 0.1% | 0.1% |  |  |  |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class F | 0.1% | 0.3% | 0.3% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |
|  | Low delay B HE |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | 0.4% |  |  |  |
| Class C | 0.0% | -0.2% | 0.1% |  |  |  |
| Class D | 0.0% | -0.1% | 0.0% |  |  |  |
| Class E | 0.2% | 0.0% | 0.2% |  |  |  |
| Overall | 0.1% | 0.0% | 0.2% |  |  |  |
|  | 0.1% | 0.0% | 0.2% |  |  |  |
| Class F | 0.1% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | 97% |  |  |  |  |  |
| Dec Time[%] | 94% |  |  |  |  |  |
|  | Low delay P HE |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% |  |  |  |
| Class C | 0.1% | 0.1% | 0.1% |  |  |  |
| Class D | 0.1% | 0.3% | 0.0% |  |  |  |
| Class E | 0.1% | 0.2% | 0.4% |  |  |  |
| Overall | 0.1% | 0.2% | 0.1% |  |  |  |
|  | 0.1% | 0.1% | 0.1% |  |  |  |
| Class F | 0.2% | 0.4% | 0.5% |  |  |  |
| Enc Time[%] | 100% |  |  |  |  |  |
| Dec Time[%] | 96% |  |  |  |  |  |

Table 2 Test-2: BD-rate results of proposal method for category boundary

|  | All Intra HE |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.2% | 0.1% | 0.1% |  |  |  |
| Class B | 0.1% | 0.4% | 0.7% |  |  |  |
| Class C | 0.0% | 0.4% | 0.8% |  |  |  |
| Class D | 0.1% | 0.1% | 0.2% |  |  |  |
| Class E | 0.1% | 0.4% | 0.4% |  |  |  |
| Overall | 0.1% | 0.3% | 0.5% |  |  |  |
|  | 0.1% | 0.3% | 0.5% |  |  |  |
| Class F | 0.6% | 1.1% | 1.7% |  |  |  |
| Enc Time[%] | 99% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | 0.1% | 0.0% | 0.1% | 2.7% | 3.8% |
| Class B | 0.1% | 0.5% | 0.8% | 0.1% | 0.4% | 0.6% |
| Class C | 0.1% | 0.5% | 0.7% |  |  |  |
| Class D | 0.1% | 0.2% | 0.3% |  |  |  |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | 0.3% | 0.5% | 0.1% | 1.4% | 2.0% |
|  | 0.1% | 0.3% | 0.5% | 0.1% | 1.4% | 2.0% |
| Class F | 0.9% | 1.6% | 2.3% |  |  |  |
| Enc Time[%] | 99% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |
|  | Low delay B HE |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.3% | 1.0% | 1.1% |  |  |  |
| Class C | 0.2% | 0.7% | 1.0% |  |  |  |
| Class D | 0.1% | -0.4% | 0.0% |  |  |  |
| Class E | 0.6% | 0.6% | 1.1% |  |  |  |
| Overall | 0.3% | 0.5% | 0.8% |  |  |  |
|  | 0.3% | 0.5% | 0.8% |  |  |  |
| Class F | 1.9% | 2.7% | 3.6% |  |  |  |
| Enc Time[%] | 101% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |  |
|  | Low delay P HE |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | 0.9% | 0.9% |  |  |  |
| Class C | 0.2% | 0.5% | 1.0% |  |  |  |
| Class D | 0.1% | -0.4% | 0.0% |  |  |  |
| Class E | 0.7% | 0.9% | 1.4% |  |  |  |
| Overall | 0.3% | 0.5% | 0.8% |  |  |  |
|  | 0.3% | 0.5% | 0.8% |  |  |  |
| Class F | 1.9% | 3.5% | 4.0% |  |  |  |
| Enc Time[%] | 102% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |

Table 3 Test-3: BD-rate results of the combination of proposal methods for partition boundary and category boundary

|  | All Intra HE |  |  | All Intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.2% | 0.1% | 0.1% |  |  |  |
| Class B | 0.1% | 0.4% | 0.8% |  |  |  |
| Class C | 0.1% | 0.5% | 0.9% |  |  |  |
| Class D | 0.1% | 0.1% | 0.2% |  |  |  |
| Class E | 0.1% | 0.5% | 0.5% |  |  |  |
| Overall | 0.1% | 0.3% | 0.5% |  |  |  |
|  | 0.1% | 0.3% | 0.5% |  |  |  |
| Class F | 0.7% | 1.2% | 1.9% |  |  |  |
| Enc Time[%] | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |  |
|  | Random Access HE |  |  | Random Access HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | 0.1% | 0.0% | 0.1% | 2.8% | 3.9% |
| Class B | 0.2% | 0.5% | 0.8% | 0.1% | 0.5% | 0.7% |
| Class C | 0.1% | 0.5% | 0.9% |  |  |  |
| Class D | 0.1% | 0.2% | 0.2% |  |  |  |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | 0.4% | 0.6% | 0.1% | 1.5% | 2.2% |
|  | 0.1% | 0.4% | 0.6% | 0.1% | 1.5% | 2.2% |
| Class F | 1.0% | 1.7% | 2.5% |  |  |  |
| Enc Time[%] | 99% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 105% |  |  |
|  | Low delay B HE |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.3% | 1.2% | 1.2% |  |  |  |
| Class C | 0.2% | 0.7% | 1.1% |  |  |  |
| Class D | 0.1% | 0.3% | 0.8% |  |  |  |
| Class E | 0.8% | 1.2% | 1.6% |  |  |  |
| Overall | 0.3% | 0.9% | 1.1% |  |  |  |
|  | 0.3% | 0.9% | 1.2% |  |  |  |
| Class F | 2.3% | 3.6% | 4.3% |  |  |  |
| Enc Time[%] | 101% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  |  |  |  |
|  | Low delay P HE |  |  | Low delay P HE-10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.3% | 0.8% | 0.9% |  |  |  |
| Class C | 0.3% | 0.7% | 1.1% |  |  |  |
| Class D | 0.2% | 0.2% | 0.4% |  |  |  |
| Class E | 0.6% | 1.0% | 1.5% |  |  |  |
| Overall | 0.3% | 0.7% | 1.0% |  |  |  |
|  | 0.3% | 0.7% | 0.9% |  |  |  |
| Class F | 2.3% | 3.7% | 4.7% |  |  |  |
| Enc Time[%] | 99% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  |  |  |  |

# Conclusion

The cross-checker’s results completely match with those of proponents. The proposed methods reduce boundary artifacts caused by SAO band offset well. We recommend subjective quality evaluation at the meeting.

# Reference

- T. Matsunobu, H. Sasai and T. Nishi, “Non-CE8: Method of removing boundary artifacts for SAO”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-H0201, Jan. 2012.

- F. Bossen, “Common test conditions and software reference configurations,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-G1200, Nov. 2011.