| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0441 |
| --- | --- |

| Title: | Motion Compensation Restrictions to Alleviate Memory Bandwidth Concerns for High Resolution Video |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tim Hellman Broadcom Corporation 200 Brickstone Square Andover, MA 01810 USA Wade Wan Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA | Tel: Email: Tel: Email: | +1-978-719-1348 timhellman@broadcom.com +1-949-926-6387 wwan@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

This proposal recommends adding a profile-independent level limit on the types of motion compensation prediction units to alleviate worst-case motion compensation bandwidth.  It notes that the AVC standard has a level limit on the number of motion vectors per MB pair, but the proposal claims that restricting 4x8 and 8x4 PUs to uni-prediction for HD and larger-sized pictures would be a preferred method to address the same concerns for HEVC.

# Introduction

The AVC standard specifies a profile-independent level limit on the maximum number of motion vectors.  The limit is 16 motion vectors per two consecutive macroblocks (MaxMvsPer2Mb) for Levels 3.1 and above.   This limit was defined to limit worst-case bandwidth requirements when decoding large pictures.  This submission recommends adding a level limit to HEVC to address the same concerns.  Rather than restricting the number of motion vectors, however, simply limiting 4x8 and 8x4 PUs to use only uni-directional prediction achieves a reasonable maximum bandwidth with very little impact on PSNR for HD-sized sequences, and also results in a simpler encoder implementation than a vector limit.

# Motion Compensation Bandwidth

Motion compensation memory bandwidth is determined by the PU block size, filter extent and the memory alignment (access characteristics of DDR DRAMs).  For a 32-bit DRAM interface (which is typical for embedded systems), the minimum data access size is 32 bytes (8 ‘beats’ of 4 bytes each), on an aligned 32-byte boundary.  Thus, in the worst case, a 4-pixel (byte) fetch ends up straddling two 32-byte boundaries, and requires a 64-byte memory access!  Since reference pictures are typically stored in a raster format, the DRAM alignment means that the ‘thin and tall’ prediction units (4x8) use more bandwidth than the ‘short and fat’ (8x4) PUs.  The larger filter extent of HEVC (8 tap Y, 4 tap C) versus AVC (6 tap Y, 2 tap C) means that HEVC uses more bandwidth than AVC even for the same PU size.

Decoder motion compensation is by far the largest user of memory bandwidth in embedded decoder implementations, such as set-top boxes.  It is not uncommon to allocate 1 Gbyte/second or more of memory bandwidth to an AVC HD decoder to guarantee proper decoding of worst-case compliance streams.  These streams consist of multiple pictures populated entirely with the worst-case PU size.  While memory bandwidth does increase with each generation of DDR devices, so does the expectations on decoder frame size and frame rate.  Therefore, providing a reasonable limit on motion compensation behavior for large picture sizes is extremely important to facilitate cost-effective implementations and ensure HEVC is successful in the marketplace.

The only motion compensation PU limit presently specified for HEVC is the elimination of 4x4 PUs.  Without any further limits, the worst-case bandwidth for HD pictures is more than double the corresponding worst-case for AVC.  That’s likely to be unacceptable for many applications.  Thus, an additional limit is needed.

# Proposed Limit

Limiting the number of motion vectors per consecutive macroblocks, as specified in AVC, is not an ideal way to express such a restriction.  HEVC’s variable block size makes specifying such a limit difficult, and complicates implementation in an encoder which must figure out where best to cut back on vectors.  So a simpler method would be preferable.

At the Geneva meeting, Sony presented document G392 [2].  This experiment consisted of restricting PU choices in the encoder.  Of the several cases presented, case C is the most interesting for large pictures and involved restricting 4x8 and 8x4 PUs to be only uni-directional.  No change was made to the coding syntax.  That document reported a negligible (0.2% Y, 0.2% U/V) loss for class A and class B.  Thus case C is an attractive candidate for a level limit.

Bandwidth calculations for this case (uni-directional 4x8 and 8x4 PUs) show significant improvement.  The worst-case bandwidth with this restriction is only 20% greater than for AVC, as compared to the 100% increase without the restriction.

Note that attempting to implement this constraint by changing syntax at the CU level in an effort to save bits in the coded PU sizes is not recommended as this would unnecessarily complicate the parsing syntax.

# Conclusions

This contribution recommends limiting 4x8 and 8x4 PUs to be only uni-directional for HD and larger-sized pictures.   The coding loss is negligible, and the limit is simple for an encoder to implement.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “WD5: Working Draft 5 of High-Efficiency Video Coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1103_d4, Jan. 2012.

- K. Kondo, T. Suzuki, “Non-CE3: Report on a restriction for small block”, Document of Joint Collaborative Team on Video Coding, JCTVC-G392, Nov, 2011

# Patent rights declaration(s)

Broadcom, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).