| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0442 |
| --- | --- |

| Title: | Limiting Collocated Temporal Reference to One Per Picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tim Hellman Broadcom Corporation 200 Brickstone Square Andover, MA 01810 USA Wade Wan Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA | Tel: Email: Tel: Email: | +1-978-719-1348 timhellman@broadcom.com +1-949-926-6387 wwan@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

This submission proposes to restrict pictures to have at most one collocated temporal reference.  It notes that the present working draft allows the collocated reference to change on every slice, and it claims that this puts a performance burden on decoders while providing limited and possibly no compression benefit.  The proposal provides syntax to identify the collocated picture from the reference lists, and also suggests alternative methods.  It also claims a side benefit of more flexibility in collocated reference picture selection.

# Introduction

The collocated reference picture will typically be used much more extensively in HEVC than in AVC.  It is now an integral part of motion vector construction for all inter blocks in both P and B slices, whereas AVC used it only for direct mode in B slices.  Presently, the first entry in the reference picture list is defined as the collocated picture, with a slice header bit selecting between list 0 and list 1 for B slices.  Thus, even a change between P and B slices in the same picture can cause the collocated reference to change, as can reference list reordering commands.  Frequent collocated picture changes create a performance burden for decoders, due to DRAM access limitations.

The proposed solution is simply to prohibit changes to the collocated reference within a picture.  A syntax change is needed to enforce this restriction, since the desired reference may no longer be the first one on the list for all slices.  One possible solution is illustrated here, though others are possible and equally acceptable.

# Problem Statement

For high-performance decoders, optimization of DRAM accesses is critical to efficient implementations.  Newer generations of DDR DRAM have much higher bandwidth, but to use that bandwidth efficiently memory clients must use large burst sizes and accept longer latencies.  Temporal reference data is too large to store on-chip, so it must reside in DRAM.  The decoder must read multiple LCUs worth of data in a single burst to satisfy the memory demands, and it must fetch this data well in advance of the LCU that uses it to mitigate the long memory latency.  Allowing the collocated picture to change per slice breaks this pipeline model, and can, in the worst case, be the ultimate bottleneck on decoder performance.

It is natural to wonder whether restricting slice-level changes to the collocated reference affects compression efficiency.  There has not been a lot of study, but there’s little reason to believe that changing the collocated reference within a picture would be advantageous, or even desirable.  A prior contribution [2] did not show any coding efficiency improvement with picture level adaptation.  In fact, forcing the collocated reference to always be the first entry in the list may actually be detrimental to an encoder, preventing it from making a reference list change that would otherwise be beneficial.

It seems that the present method of collocated reference selection is to provide flexibility but this flexibility comes at the expense of increased decoder cost and complexity with no evidence that real-world streams would ever take advantage of this flexibility.

# Proposed Solution

A method of signalling the collocated reference is needed to solve this problem because the existing method almost certainly produces different references when mixing P and B slices.  The reference choice must be available to every slice, to preserve error resiliency.  Adding one new syntax element to the slice header achieves these goals (existing text from [1]):

| … |  |
| --- | --- |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if (slice_type != I) |  |
| collocated_ref_idx | ue(v) |
| if( adaptive_loop_filter_enabled_flag && aps_adaptive_loop_filter_flag ) { |  |
| …. |  |

The new syntax element simply gives the reference index of the collocated picture, from list 0 in P slices and from either list 0 or list 1 (according to collocated_from_l0_flag) in B slices.  This value is semantically restricted to always point to the same reference picture for every slice in a picture.

Note that this representation results in the smallest possible binarization (1 bit) for the standard case, in which the first entry of the list is the collocated reference, thereby having negligible impact on coding efficiency.  It also provides additional flexibility to the encoder to choose an arbitrary collocated reference, although that’s not the primary motivation.

Alternative syntax changes are also possible.  For example, the collocated reference could be identified by delta POC rather than reference index, and it could be marked in the RPS rather than the slice header, as was proposed in [2].

# Conclusions

This contribution recommends prohibiting changes to the collocated reference within a picture.  It recommends adoption of this restriction, along with a single new syntax element in the slice header.  Creation of a core experiment to study alternative signalling methods may also be desirable.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “WD5: Working Draft 5 of High-Efficiency Video Coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1103_d4, Jan. 2012.

- Y. Yu, K. Panusopone, X. Fang, L. Wang, “The Improvements on Reference Picture Buffering and List Construction”, Document of Joint Collaborative Team on Video Coding, JCTVC-G717, Nov. 2011.

# Patent rights declaration(s)

Broadcom, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex A:  Draft Text Changes

Yellow highlighting used for additions.  Existing text from [1].

Section 7.3.3  Slice Header Syntax

| … |  |
| --- | --- |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if (slice_type != I) |  |
| collocated_ref_idx | ue(v) |
| if( adaptive_loop_filter_enabled_flag && aps_adaptive_loop_filter_flag ) { |  |
| …. |  |

Section 7.4.3  Slice Header Sematics

collocated_from_l0_flag equal to 1 specifies the picture that contains the co-located partition shall be derived from list 0, otherwise the picture shall be derived from list 1.

collocated_ref_idx specifies the reference index of the picture that contains the co-located partition.  When the current slice is a P slice collocated_ref_idx refers to list 0.  When the current slice is a B slice collocated_ref_idx refers to list 0 if collocated_from_l0 is 0, otherwise it refers to list 1.  Collocated_ref_idx shall always refer to a valid list entry, and the resulting picture shall be the same for all slices of a coded picture.

Section 8.4.2.1.8 Derivation process for temporal motion vector prediction

…

Depending on the values of slice_type, collocated_from_l0_flag and collocated_ref_idx, the variable colPic, specifying the picture that contains the co-located partition, is derived as follows.

- If slice_type is equal to B and collocated_from_l0_flag is equal to 0, the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList1[ collocated_ref_idx ].

- Otherwise (slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P) , the variable colPic specifies the picture that contains the co-located partition as specified by RefPicList0[ collocated_ref_idx ].