| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0443 |
| --- | --- |

| Title: | CE11: Harmonization of residual coding and NSQT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungsun Kim, Byeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea Joel Sole, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +82-10-2201-9774 jungsun.kim@lge.com +1-858-845-2429 joels@qualcomm.com |
| Source: | LG electronics, Qualcomm |  |  |

_____________________________

# Abstract

This contribution reports the result of applying the NSQT harmonization presented in JCTVC-G750 in HM5.0. The proposal uses fixed position-based contexts for 16×4/4×16 blocks. Contexts and tables are shared with 8×8 blocks. These changes harmonize the processing of blocks with the same number of coefficients and allegedly, increase the throughput. The BD-rate for RA-HE and LD-HE is 0.06% and 0.10%, respectively.

# Technical Description

This proposal provides the harmonization of JCTVC-G750 [1] with the HM5.0 within the scope of CE11. JCTVC-G750 proposed to modify the scanning order, the last significant coefficient coding, and the significance map for NSQT blocks. Because of the adoption of the straw man solution in JCTVC-G1038 [2], the harmonization is only required for the significance map.

To keep consistency with JCTVC-G750, the fixed position context tables re-use the contexts of 8×8 blocks. In HM5.0, the 8×8 contexts are obtained from a 4×4 table (called map8×8) by the following operations:

map8x8[4 * (uiPosY >> 1) + (uiPosX >> 1)]

The same table is used to accommodate sizes 16×4/4×16. For the 4×16 case, the table is accessed by

map8x8[4 * (uiPosY >> 2) + uiPosX]

and for the 16×4 case, the context is obtained by

map8x8[4 * (uiPosX >> 2) + uiPosY]

Therefore, all contexts and tables of the 8×8 blocks are re-used. Moreover the method does not depend on the context indexes themselves but on the locations of the context. Therefore it can be directly applied without considering which context tables are used for 8×8 blocks.

The significance group flag is inferred when right and bottom neighboring sub-blocks are both significant. This never occurs for the 16×4/4×16 cases because of its size. The inferred flag is used for these sizes when the right (or bottom) neighbor is significant.

- Results

The method is implemented on the top HM5.0 under the configuration of RA-HE and LD-HE [3]. The binary files for each encoder and decoder are compiled on Window7 32bit PC for Window XP 64bit PC. Experimental results are shown in Table 1. It shows 0.06%, 0.02%, 0.02% and 0.10%, 0.00%, 0.35% for luma component and chroma components in RA-HE and LD-HE case, respectively. Note that NSQT is sn inter frame tool and not part of the LC tests.

|  | Random Access HE |  |  | Low delay B HE |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.05% | 0.03% | -0.07% |  |  |  | 0.04% | -0.06% | 0.09% |
| Class B | 0.04% | 0.02% | 0.01% | 0.05% | 0.08% | 0.43% | 0.06% | 0.15% | 0.11% |
| Class C | 0.06% | -0.02% | 0.05% | 0.12% | 0.17% | 0.29% |  |  |  |
| Class D | 0.10% | -0.08% | 0.06% | 0.17% | -0.22% | 0.25% |  |  |  |
| Class E |  |  |  | 0.04% | -0.06% | 0.46% |  |  |  |
| Overall | 0.06% | -0.02% | 0.02% | 0.10% | 0.00% | 0.35% | 0.05% | 0.06% | 0.10% |
|  | 0.06% | -0.04% | 0.04% | 0.09% | 0.03% | 0.29% | 0.05% | 0.04% | 0.13% |
| Class F | -0.02% | 0.03% | 0.00% | 0.19% | 0.22% | -0.13% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 96% |  |  | 97% |  |  | 96% |  |  |

Table 1 BD rate

- Conclusions

Fixed position context tables are proposed for 16×4/4×16 blocks. No additional contexts are introduced since the blocks share the same context with 8×8 blocks. The method simplifies the context derivation and reduces the dependencies between neighboring coefficients, thus allowing for a higher throughput, while reporting small BD-rate changes.

- References

- J. Sole, X. Wang, M. Karczewicz, J. Kim, B. Jeon, “Non-CE2: Harmonization of HE residual coding and NSQT”, JCTVC-G750, 7th JCT-VC Meeting, Geneva, CH, Nov. 2011.

- J. Sole, “BoG report: Harmonization of NSQT with residual coding” , JCTVC-G1038, 7th JCT-VC Meeting, Geneva, CH, Nov. 2011.

- V. Sze, T. Nguyen, Y. Piao, J. Sole, K. Panusopone, “Description of Core Experiment (CE11): Coefficient scanning and coding”, JCTVC-G1211, 7th JCT-VC Meeting, Geneva, CH, Nov. 2011.

# Text for Working Draft

###### 9.2.3.1.1.4 Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

Inputs to this process are the color component index cIdx, the current coefficient scan position ( xC , yC ), the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the color component index cIdx, the transform block size and previsously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 2, sigCtx is derived using ctxIdxMap4x4[ ] specified in Error: Reference source not found as follows..

sigCtx  =  ctxIdxMap4x4[ ((cIdx > 0) ? 15 : 0) + (yC << 2) + xC ]	(9-1)

- Otherwise if log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 3, sigCtx is derived using ctxIdxMap8x8[ ] specified in Error: Reference source not found as follows.

sigCtx  =  ((xC + yC) = = 0) ? 10 : ctxIdxMap8x8[ ((yC >> 1 ) << 2) + (xC >> 1) ]	(9-2)

sigCtx  +=  ( cIdx > 0) ? 6: 9		(9-3)

- Otherwise if log2TrafoWidth is equal to 4 and log2TrafoHeight is equal to 2, sigCtx is derived as follows.

sigCtx  =  ((xC + yC) = = 0) ? 10 : ctxIdxMap8x8[ ((xC >> 2 ) << 2) + yC ]	(9-57)

sigCtx  +=  ( cIdx > 0) ? 6: 9		(9-58)

- Otherwise if log2TrafoWidth is equal to 2 and log2TrafoHeight is equal to 4, sigCtx is derived as follows.

sigCtx  =  ((xC + yC) = = 0) ? 10 : ctxIdxMap8x8[ ((yC >> 2 ) << 2) + xC ]	(9-59)

sigCtx  +=  ( cIdx > 0) ? 6: 9		(9-60)

- Otherwise if xC + yC is equal to 0, sigCtx is derived as follows.

sigCtx  =  ( cIdx > 0) ? 17: 20		(9-61)

Otherwise…

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).