| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0450-r1 |
| --- | --- |

| Title: | Limiting the Bin to Bit Expansion Ratio of CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Informational and Proposal |  |  |
| Author(s) or Contact(s): | Yong Yu Broadcom Corporation 200 Brickstone Square Andover, MA 01810 USA Tim Hellman Broadcom Corporation 200 Brickstone Square Andover, MA 01810 USA Wade Wan Broadcom Corporation 5300 California Ave Irvine, CA 92617 USA | Tel: Email: Tel: Email: Tel: Email: | +1-978-719-1345 yongyu@broadcom.com +1-978-719-1348 timhellman@broadcom.com +1-949-926-6387 wwan@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

This proposal presents an analysis of HEVC’s CABAC bit to bin expansion ratio using current HM5.0 common test conditions and finds no performance loss if an AVC-like constraint is applied. It proposes to keep a similar limit for the bit to bin ratio of CABAC in HEVC as was defined in the AVC/H.264 standard.

# Introduction

Several contributions have been made to address CABAC complexity concerns by limiting the expansion ratio of bits to bins. Although this bound almost never affects coding of practical, real-world video input, it is a critical requirement to allow decoder implementations to be designed in a cost-effective manner. In the AVC standard, the number of bins of a slice is required to be less than 1.33* (number of bits) + 96*(number of MBs) for 8-bit video input.  The concept of a MB (macroblock) does not exist in HEVC but an equivalent value can be derived from picture dimensions which should be sufficient.

In this proposal, we present an analysis of HEVC’s CABAC bit to bin expansion ratio using current HM5.0 common test conditions.

# Experimental Results

We collected the number of bins and bits for all the frames with different picture types (intra and inter coded pictures) and picture dimensions, from the streams in the RA-HE (Random Access - High Efficiency) category.

The experimental results are shown in the following graphs. The blue dots are the actual bin counts from the decoder. The red dots are the maximum number of bins allowed assuming an AVC-like limit.

The experimental results shown in the following graphs demonstrate that applying an AVC-like limit will not affect HEVC compression performance.

# Conclusions

Our experimental results show that an AVC-like limit on the relationship between bits and bins per picture is appropriate for HEVC and suggest adopting such a similar limit to the HEVC standard.

One possible (minor) modification from the current draft can be as follows:

cabac_zero_word is a byte-aligned sequence of two bytes equal to 0x0000.

Let NumBytesInVclNALunits be the sum of the values of NumBytesInNALunit for all VCL NAL units of a coded picture.

Let BinCountsInNALunits be the number of times that the parsing process function DecodeBin( ), specified in subclause Error: Reference source not found, is invoked to decode the contents of all VCL NAL units of a coded picture.

Let the variables RawMinCUBits and PicSizeMinCUs be derived as
RawMinCUBits = (1<< Log2MinCUSize)*( 1<<Log2MinCUSize) * BitDepthY + 2 * (1<< Log2MinCUSize-1)*( 1<<Log2MinCUSize-1) * BitDepthC, and
PicSizeMinCUs = Ceil( PicWidthInSamplesL ÷ ( 1 << Log2MinCUSize ) )* Ceil( PicHeightInSamplesL ÷ ( 1 << Log2MinCUSize ) )

BinCountsInNALunits shall not exceed ( 32 ÷ 3 ) * NumBytesInVclNALunits + ( RawMinCUBits * PicSizeInMinCUs)  32.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “WD5: Working Draft 5 of High-Efficiency Video Coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-G1103_d4, Jan. 2012.

# Patent rights declaration(s)

Broadcom, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).