| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0451 |
| --- | --- |

| Title: | CE4 Subtest 2.5: Coding of quantization matrices based on modified prediction and mapping to unsigned values |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rajan L. Joshi Joel Sole Rojals Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-658-4511 rajanj@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

A method for coding of quantizer matrices is presented. It uses raster scanning and prediction based on top and left neighbors. A biased (towards positive values) mapping is used to convert the prediction error to unsigned values. The unsigned values are coded using either exponential Golomb or Golomb Rice codes. Simulation results are provided for coding the matrices using exponential Golomb codes only, Golomb Rice codes only, or a combination. For asymmetric matrices, this method is reported to provide average bit-rate reductions of 24.3%, 24.8%, and 31.7%, respectively, for the 3 entropy coding modes described above. For symmetric matrices, in conjunction with a symmetry flag, this method is reported to provide average bit-rate reductions of 55.4%, 54.5%, and 57.2%, respectively.

# Introduction

In HM 5.0, the quantizer matrix entries are scanned using a zigzag scan. Then, the current matrix entry is predicted from the previous entry in the scan and the residual is encoded using signed exponential Golomb code with parameter 0. The zigzag scan does not work well when the quantization matrix is asymmetric. Also, for non-zero prediction residuals, the structure of the signed exponential Golomb code means that one bit is needed for signaling the sign. We propose an alternate method for overcoming these drawbacks.

# Proposed method

The use of quantization matrices is motivated by the properties of the human visual system (HVS). One model used for the design of quantization matrices is the 2-D contrast sensitivity function (CSF) of the HVS. The contrast sensitivity decreases with increasing frequency. Due to the decreased contrast sensitivity at higher frequencies, well-designed quantization matrices have the property that typically, quantization matrix entries increase both in row and column directions. Our proposed method utilizes this property in such a way that the prediction residual is almost always positive. As a result, the overhead for the sign bit is reduced.

## Proposed method

The proposed method consists of following steps:

- Scan: Raster scan.

- Prediction: The maximum of the quantizer matrix entry above and to the left (max(top,left)).

- Remapping: The prediction residual is remapped to a non-negative value using a fixed offset of 8. Instead of interleaving positive and negative value as done in HM5.0, the interleaving is biased towards positive values as follows. Let the prediction residual be denoted by X and remapped prediction residual be denoted by Y. Then,

| For X<0 | Y=offset-1-2*X | (1) |
| --- | --- | --- |
| For 0≤X≤offset | Y=X |  |
| For X>offset | Y=offset+(X−offset)∗2 |  |

Table 1 shows the remapping foroffset=4.

- Coding of prediction residual: The remapped prediction residual is coded using either exponential Golomb (parameter 0) or Golomb Rice codes. We present results for 3 different scenarios:

  - Exponential Golomb only (EG).

  - Golomb Rice only (GR): The Golomb Rice parameter is signaled in the bitstream. The parameter is chosen from the set 2, 4, 8, 16 and signaled using 2 bits.

  - Exponential Golomb and Golomb Rice (EG + GR): In this case, either exponential Golomb or Golomb Rice code can be used. For each quantization matrix, a flag is coded to indicate the choice. If Golomb Rice is chosen, the Golomb Rice parameter is also coded.

If option EG (exponential Golomb only) is chosen, the syntax is identical to existing HM5.0. The difference is in scanning, prediction and mapping to non-negative values.

Table 1 Remapping between X and Y

| Source symbol (X) | Remapped symbol (Y) |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| 2 | 2 |
| 3 | 3 |
| 4 | 4 |
| -1 | 5 |
| 5 | 6 |
| -2 | 7 |
| … | … |

# Results

Tables 2 and 3 present the bits needed for lossless coding of 6 quantization matrix sets used in CE4 subtest 2 for symmetric and non-symmetric cases, respectively.

Table 2: Bits needed for lossless coding (symmetric matrices)

| Symmetrical Quantization Matrices |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | HM5.0 | Exp-Golomb only | Golomb-Rice only | Exp-Golomb + Golomb-Rice |
| Symmetry1 | 8779 | 3887 (44.3%) | 3825 (43.6%) | 3693 (42.1%) |
| Symmetry2 | 5037 | 2531 (50.2%) | 3291 (65.3%) | 2520 (50.0%) |
| Symmetry3 | 12157 | 4797 (39.5%) | 4760 (39.2%) | 4654 (38.3%) |
| Symmetry4 | 15677 | 8571 (54.7%) | 7595 (48.4%) | 7603 (48.5%) |
| Symmetry5 | 8739 | 3851 (44.1%) | 3820 (43.7%) | 3677 (42.1%) |
| Symmetry6 | 10863 | 4327 (39.8%) | 4455 (41.0%) | 4258 (39.2%) |

Table 3: Bits needed for lossless coding (asymmetric matrices)

| Asymmetrical Quantization Matrices |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | HM5.0 | Exp-Golomb only | Golomb-Rice only | Exp-Golomb + Golomb-Rice |
| Asymmetry1 | 9443 | 6577 (69.6%) | 6325 (67.0%) | 6167 (65.3%) |
| Asymmetry2 | 5053 | 4035 (79.9%) | 5639 (111.6%) | 4006 (79.3%) |
| Asymmetry3 | 12707 | 8531 (67.1%) | 7871 (61.9%) | 7619 (60.0%) |
| Asymmetry4 | 24825 | 24759 (99.7%) | 19270 (77.6%) | 19278 (77.7%) |
| Asymmetry5 | 9357 | 6503 (69.5%) | 6265 (67.0%) | 6107 (65.3%) |
| Asymmetry6 | 11259 | 7683 (68.2%) | 7424 (65.9%) | 6995 (62.1%) |

# Conclusion

A method for coding of quantizer matrices is presented. It uses raster scanning and prediction based on top and left neighbors. A biased (towards positive values) mapping is used to convert the prediction error to unsigned values. The unsigned values are coded using either exponential Golomb or Golomb Rice codes. Simulation results are provided for coding the matrices using exponential Golomb codes only, Golomb Rice codes only, or a combination. For asymmetric matrices, this method is reported to provide average bit-rate reductions of 24.3%, 24.8%, and 31.7%, respectively, for the 3 entropy coding modes described above. For symmetric matrices, in conjunction with a symmetry flag, this method is reported to provide average bit-rate reductions of 55.4%, 54.5%, and 57.2%, respectively.

We recommend adoption of the proposed method with (EG + GR) option. If minimal changes from the current HM5.0 are desired, Exponential Golomb only (EG) is a good alternative.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Working draft text

| pic_scaling_list_enable_flag | u(1) |
| --- | --- |
| if( pic_scaling_list_enable_flag ) { |  |
| pic_scaling_list_present_flag | u(1) |
| if( pic_scaling_list_present_flag ) { |  |
| for( SizeID = 0; SizeID < 4; SizeID++ ) { |  |
| for( MatrixID = 0; MatrixID < (SizeID == 3) ? 2:6; MatrixID++ ) { |  |
| pic_scaling_list_pred_mode | u(1) |
| if( !pic_scaling_list_pred_mode ) |  |
| pic_scaling_list_pred_matrix_id_delta | ue(v) |
| else |  |
| scaling_list( ScalingList[ SizeID ][ MatrixID ][ 0 ], (1<<(SizeID+2) ) ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

## Exponential Golomb Only

#### Scaling list syntax

| scaling_list( ScalingList, blkSize ) { | Descriptor |
| --- | --- |
| scaling_list_symmetry_flag | u(1) |
| for( r=0; r < blkSize, r++) { |  |
| if (scaling_list_symmetry_flag) { |  |
| startCol = r |  |
| } else { |  |
| startCol = 0 |  |
| } |  |
| for( c=startCol; c < blkSize; c++) { |  |
| delta_coef_unsigned | ue(v) |
| conv_to_signed_with_offset( delta_coef_unsigned, 8) |  |
| If (startCol == 0) { |  |
| pred_coeff = ScalingList[(r-1)*blkSize+c]) } else { |  |
| pred_coeff = max(ScalingList[r*windowSize+c-1], ScalingList[(r-1)*blkSize+c)]) |  |
| } |  |
| ScalingList[r*windowSize+c] = pred_coeff + scaling_list_delta_coef |  |
| if (scaling_list_symmetry_flag && (r!=c)) { |  |
| ScalingList[c*windowSize+r] = ScalingList[r*windowSize+c] |  |
| } |  |
| } |  |
| } |  |
| } |  |

| Conv_to_signed_with_offset( delta_coeff_unsigned, offset ) { | Descriptor |
| --- | --- |
| if ( delta_coeff_unsigned > offset) { |  |
| diff = delta_ceoff_unsigned - offset |  |
| if ((diff % 2) == 0) { |  |
| scaling_list_delta_coef = offset + (diff/2) |  |
| } else { |  |
| scaling_list_delta_coef = -((diff+1)/2) |  |
| } |  |
| } else { |  |
| scaling_list_delta_coeff = delta_ceoff_unsigned |  |
| } |  |
| } |  |

Scaling list semantics

delta_coef_unsigned specifies the difference between the current matrix coefficient from the predicted matrix coefficient.

scaling_list_symmetry_flag specifies whether the matrix is symmetric. A value of one indicates that the mtrix is symmetric and that only (blkSize*(blkSize+1)/2) syntax elements delta_coef_unsigned need to be read from the bitstream.

Coding process

When calculating pred_coeff, if the index for ScalingList is negative, the value is assumed to be 0.