| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0452 |
| --- | --- |

| Title: | CE5 Subtest d: Modifications to last significant coefficient position coding for transform skip mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rajan L. Joshi Joel Sole Rojals Xianglin Wang Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-658-4511 rajanj@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution proposes an alternate method for coding the last significant coefficient position when the transform is skipped either in horizontal or vertical direction or both. For the direction in which the transform is skipped, the last position is coded using fixed number of bits in the bypass mode. The modification provides identical BD-rate performance while increasing the number of bins coded in the bypass mode.

# Introduction

Coding of the last significant coefficient position in HM5.0 is based on the principle that lower values for the last position are much more probable. But when the transform in a particular direction is skipped, this assumption is no longer valid. We propose a modification of the last significant coefficient position coding when a transform is skipped in at least one direction.

# Proposed method

The coding of the last significant coefficient position is changed as follows. In HM5.0, the position of the last non-zero coefficient in the scan order is coded explicitly, before coding the significance and level information. Let the last non-zero coefficient in the scan order be denoted by (lastX, lastY), where lastX denotes the column number for the last non-zero coefficient and lastY denotes the row number for the last non-zero coefficient. When a vertical scan is used, the values of lastX and lastY are swapped.

When a 2-D transform is applied to the prediction residual, lastX and lastY typically have small magnitudes because of the concentration of non-zero coefficients in the lower frequencies. But when a transform is skipped in a particular direction, the last position in that direction can have larger magnitude. Hence we code last position in the skipped direction using fixed number of bits in bypass mode. The number of bits used islog2(block_size). The coding of the last position in the direction in which transform is applied is unchanged from HM5.0.

## Square blocks

For a square block, when a vertical-skip mode is chosen, a vertical scan is applied. This results in values of lastX and lastY being swapped. In that case, lastX is coded using the proposed method and lastY is coded using the HM5.0 method. Table 1 summarizes coding of lastX and lastY. In HM5.0, the context coded parts of lastX and lastY are grouped together and so are the bypass coded parts. To follow the same principle, for square blocks, when horizontal-skip or vertical-skip mode is used, lastY is coded before lastX. This results in the grouping of bypass coded bins.

Table 1: Coding of the last position for different transform skip modes for square blocks

| Transform | LastX | LastY |
| --- | --- | --- |
| 2-D | HM 5.0 | HM 5.0 |
| Horizontal-skip | Fixed (bypass mode) | HM 5.0 |
| Vertical-skip | Fixed (bypass mode) | HM 5.0 |
| No transform | Fixed (bypass mode) | Fixed (bypass mode) |

## Non-square blocks

For non-square blocks, the scan is always diagonal. In that case, the coding of lastX and lastY for transform skip modes is summarized in Table 2. To group bypass coded bins together, for horizontal-skip mode, lastY is coded before lastX.

Table 2: Coding of the last position for different transform skip modes for non-square blocks

| Transform | LastX | LastY |
| --- | --- | --- |
| 2-D | HM 5.0 | HM 5.0 |
| Horizontal-skip | Fixed (bypass mode) | HM 5.0 |
| Vertical-skip | HM 5.0 | Fixed (bypass mode) |
| No transform | Fixed (bypass mode) | Fixed (bypass mode) |

# Results

The proposed method is implemented on top of HM5.0-tsm-scans version. Results are presented for the common test conditions [2] for all inter configurations. Since the transform skip mode is applied only to non-intra slices, the intra results are identical to HM. The encoding as well as the decoding was done on a 64-bit LINUX cluster. Table 3 shows the BD-rate results for the proposed method when compared with HM5.0. Table 4 shows the BD-rate results for the proposed method when compared with HM5.0-tsm-scans. It can be seen that the modification of last position coding provides essentially identical BD-rate performance. At the same time, the number of bins related to last position that are coded in by-pass mode is increased.

Table 3: Comparison of BD-rate performance of proposed method vs HM5.0

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.2% | -0.3% | 0.0% | -0.2% | -0.2% | 0.1% | -0.2% | -0.1% |
| Class B | -0.3% | -0.2% | -0.1% | -0.7% | -0.3% | 0.0% | -0.2% | -0.2% | -0.1% |
| Class C | -0.5% | -0.3% | -0.1% | -1.0% | -0.4% | 0.0% |  |  |  |
| Class D | -0.8% | -0.2% | -0.2% | -1.5% | 0.0% | 0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.4% | -0.2% | -0.1% | -0.9% | -0.3% | 0.0% | -0.1% | -0.2% | -0.1% |
|  | -0.4% | -0.2% | -0.2% | -0.9% | -0.3% | 0.0% | -0.1% | -0.2% | -0.1% |
| Class F | -2.0% | -1.0% | -1.1% | -2.9% | -1.7% | -1.8% |  |  |  |
| Enc Time[%] | 132% |  |  | 120% |  |  | 130% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.3% | 0.3% | 0.6% | -1.1% | -0.9% | -0.1% |  |  |  |
| Class C | -0.7% | -0.3% | 0.0% | -1.6% | -0.6% | 0.3% |  |  |  |
| Class D | -1.2% | 0.3% | 1.1% | -2.2% | 1.0% | 0.9% |  |  |  |
| Class E | -0.3% | 0.7% | 0.8% | -1.5% | 1.5% | 1.0% |  |  |  |
| Overall | -0.6% | 0.2% | 0.6% | -1.6% | 0.1% | 0.5% |  |  |  |
|  | -0.6% | 0.2% | 0.5% | -1.5% | 0.0% | 0.4% |  |  |  |
| Class F | -4.7% | -3.0% | -3.2% | -6.6% | -5.3% | -4.6% |  |  |  |
| Enc Time[%] | 124% |  |  | 117% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |  |  |  |

Table 4: Comparison of BD-rate performance of proposed method vs HM5.0-tsm-scans

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class B | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.1% | 0.0% | -0.2% | -0.2% |
| Class C | 0.0% | 0.1% | 0.0% | -0.1% | -0.2% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | -0.1% |
|  | 0.0% | -0.1% | -0.1% | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class F | 0.0% | 0.1% | -0.1% | -0.1% | -0.1% | -0.2% |  |  |  |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | -0.3% | -0.1% | -0.1% | -0.5% | -0.4% |  |  |  |
| Class C | 0.0% | -0.1% | 0.0% | -0.1% | -0.5% | -0.2% |  |  |  |
| Class D | 0.0% | -0.2% | 0.3% | 0.0% | -0.4% | -0.4% |  |  |  |
| Class E | -0.1% | 0.1% | 0.5% | -0.2% | 0.2% | -0.3% |  |  |  |
| Overall | 0.0% | -0.1% | 0.1% | -0.1% | -0.3% | -0.3% |  |  |  |
|  | 0.0% | -0.1% | 0.1% | -0.1% | -0.3% | -0.3% |  |  |  |
| Class F | -0.2% | -0.4% | -0.3% | -0.3% | -0.9% | -0.5% |  |  |  |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |  |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |  |  |  |

# Coded bin statistics

Table 5: Pecentage of bins for last significant coefficient coded in bypass mode

| Configuration | % of bins coded in bypass |  |
| --- | --- | --- |
|  | TSM | proposal |
| RA-HE | 6.36% | 12.46% |
| RA-LC | 5.78% | 15.39% |
| RA-HE10 | 9.75% | 13.88% |
| LB-HE | 7.75% | 16.43% |
| LB-LC | 7.64% | 20.74% |

# Conclusion

This contribution proposes an alternate method for coding the last significant coefficient position when the transform is skipped either in horizontal or vertical direction or both. For the direction in which the transform is skipped, the last position is coded using fixed number of bits in the bypass mode. The modification provides identical BD-rate performance while increasing the number of bins coded in the bypass mode.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Working draft text

Please refer to working draft text submitted with document JCTVC-H0211.