| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0453 |
| --- | --- |

| Title: | Non-CE4 Subtest 2: Signaling a subset of quantizer matrix coefficients for 32x32 quantization matrices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rajan L. Joshi Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-658-4511 rajanj@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

Coding of 32×32 quantization matrix requires a large number of bits. This contribution proposes signaling a syntax element for a 32×32 quantization matrix that defines a rectangular subset consisting of lower frequencies. Only the values in the quantization matrix that belong to the subset are coded, leading to saving in bit-rate. The missing entries can be predicted or set to a constant value such as the highest allowable quantization matrix value.

# Introduction

At the Geneva meeting quantization matrices were adopted into the current HEVC working draft. Since HEVC supports 16×16 and 32×32 transforms in addition to the 4×4 and 8×8 block sizes supported by AVC/H.264, quantization matrices of sizes 16×16 and 32×32 are needed. The coding of these larger size matrices requires quite a few bits. But it was shown in JCTVC-D257 [1] that only retaining 16×16 lowest frequency coefficients results in only a marginal degradation in the BD-rate performance. This appears to suggest that when a 32×32 transform is chosen, the higher frequency coefficients have very little energy. Thus, it may not be important to specify the values corresponding to higher frequencies in a 32×32 matrix. We propose sending a syntax element for a 32×32 quantization matrix that defines a rectangular subset consisting of lower frequencies. Only the values in the quantization matrix that belong to the subset are coded leading to saving in bit-rate. The missing entries can be predicted or set to the highest value.

# Proposed method

For a 32×32 quantization matrix, first a one bit flag is sent to indicate whether only a subset of the quantization matrix is being coded. If that is the case, the last row and the last column of the subset is coded using 5 bits. Then, the subset of the quantization matrix is coded using the method in HM5.0 or the method proposed in JCTVC-H0451 [2]. The quantization matrix entries for the subset are reconstructed and used to predict the quantization matrix entries outside the subset. The entries outside the subset are scanned in a raster scan. The predictor used is the maximum of the top and left quantization matrix value. Tables 3 and 4 show an example of this technique for a smaller (8×8) quantization matrix when only the lowest 5×5 rectangle is signaled. Instead of predicting the missing values, it may be possible to set all the missing values to a large constant such as 255.

This signaling scheme can be thought of as a lossy compression scheme for quantization matrices, where compression error is introduced in the quantizer matrix entries at higher frequency. From the point of view of human visual system, this approach may be a preferable to introducing them randomly at any location.

| 6 | 9 | 13 | 18 | 25 | 35 | 36 | 37 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 9 | 10 | 15 | 21 | 32 | 35 | 37 | 41 |
| 13 | 15 | 18 | 23 | 35 | 55 | 58 | 59 |
| 18 | 21 | 23 | 26 | 65 | 58 | 64 | 66 |
| 25 | 32 | 35 | 65 | 66 | 66 | 67 | 70 |
| 35 | 35 | 55 | 58 | 66 | 68 | 70 | 73 |
| 36 | 37 | 58 | 64 | 67 | 70 | 76 | 80 |
| 37 | 41 | 59 | 66 | 70 | 73 | 80 | 85 |

Table 1: Original 8×8 quantization matrix

| 6 | 9 | 13 | 18 | 25 | 25 | 25 | 25 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 9 | 10 | 15 | 21 | 32 | 32 | 32 | 32 |
| 13 | 15 | 18 | 23 | 35 | 35 | 35 | 35 |
| 18 | 21 | 23 | 26 | 65 | 65 | 65 | 65 |
| 25 | 32 | 35 | 65 | 66 | 66 | 66 | 66 |
| 25 | 32 | 35 | 65 | 66 | 66 | 66 | 66 |
| 25 | 32 | 35 | 65 | 66 | 66 | 66 | 66 |
| 25 | 32 | 35 | 65 | 66 | 66 | 66 | 66 |

Table 2: Reconstructed quantization matrix when only the lowest 5×5 entries are coded

# Conclusion

This contribution proposes signaling a syntax element for a 32×32 quantization matrix that defines a rectangular subset consisting of lower frequencies. Only the values in the quantization matrix that belong to the subset are coded, leading to saving in bit-rate. The missing entries can be predicted or set to a constant value such as the highest allowable quantization matrix value.

This signaling scheme can be thought of as a lossy compression scheme for quantization matrices, where compression error is introduced in the quantizer matrix entries at higher frequency. From the point of view of human visual system, this approach may be a preferable to introducing them randomly at any location.

# References

[1] J. Sole, R. Joshi, M. Karczewicz, Y.-M. Hong, M.-S. Cheon, and I.-K. Kim, “Low complexity 32×32 transform by partial frequency transform,” JCTVC-D257, Daegu, KR, January 2011.

[2] R. Joshi, J. Sole, and M. Karczewicz, “Coding of quantization matrices based on modified prediction and mapping to unsigned values,” JCTVC-H0451, San Jose, USA, January 2012.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).