| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, USA, 1-10 February 2012 | Document: JCTVC-H0456 |
| --- | --- |

| Title: | Non-CE 7: Experimental Results for the ROT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhan Ma Felix Fernandes Elena Alshina Alexander Alshin | Email: | zhan.ma@sta.samsung.com ffernandes@sta.samsung.com elena_a.alshina@samsung.com alexander_b.alshin@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution provides the simulation results for the Rotational Transform (ROT) enabled in HM 5.0 against the HM 5.0 anchor results. The ROT is implemented using a lifting factorization with 5-bit multipliers and is applied to 8x8, 16x16 and 32x32 luma-component, intra residuals only. For the All-Intra (AI-HE) test case, the ROT provides 0.6% performance gain with 142% encoder execution time relative to the anchor. Decoder execution times are identical to those of the anchor, within measurement-error margins, for all cases.

# Introduction

It is well known that the DCT basis functions are sub-optimal for some types of residuals. If the predicted signal deviates substantially from the original, then the residual is not smooth and the DCT does not compact energy well. In this case, an alternative transform improves performance.

One group of alternative transforms is the intra-direction dependent transforms. Among them, the DCT/DST mode-dependent combination [1] shows an excellent trade-off between memory usage and performance improvement. Such a mode-dependent transform does not require additional signaling or encoder search complexity.

Another class of additional transforms is the Rotational Transform (ROT) [2,3] which is a low-complexity secondary transform adaptively chosen at the encoder. In [4], a 5-bit precision lifting factorization is shown to reduce ROT implementation complexity significantly while retaining compression performance and perfect invertibility. In this proposal, for the low-complexity scenario, the encoder execution time is reduced relative to the anchor by eliminating redundant processing in ROT-associated RDOQ iterations.

- Algorithm description

The ROT does not replace the main transform core. It is applied as a secondary transform for only the low frequency part of transform coefficients matrix (Fig. 1).

Fig 1. ROT application on decoder side.

The secondary 8x8 Rotational Transform (ROT) [1, 2, 3] is applied after the DCT as shown in Fig. 2.

Figure 2: ROT as a secondary transform

The ROT-transformed coefficients, mo are related to the input residuals mi as follows:

where D, Rh and Rv are DCT, horizontal ROT and vertical ROT matrices, respectively.

The matrices Rv and Rh are defined as the following compound Given’s rotation matrix products:

The matrices Rx(θ) and Rz(θ) are compound rotation matrices because they rotate simultaneously about 2 pairs of axes in 8-dimensional space. By performing such rotations about carefully selected angles, the rotational transform allows partial energy exchange between the columns/rows of a transform coefficient matrix. This energy re-distribution improves energy compaction and minimizes quantization error.

To increase coding gain, we do not restrict our implementation to the single matrix pair Rh and Rv. Instead, we define four different, non-trivial, rotational transforms Rhi and Rvi corresponding to four sets of rotation angles θ1i, θ2i, θ3i, θ4i, θ5i, θ6i, for i = 1,…,4. We also define the “identity” ROT, in which trivially, all rotation matrices are identity matrices so that the ROT is effectively skipped and no transform coefficients are changed after the DCT. These five ROT transforms constitute the dictionary of alternative transforms. One of these 5 transforms is chosen at the encoder and signaled explicitly to the decoder explicitly by encoding indexROT = 0,…,4 for each transform unit (TU). The rotation angles for the 4 non-trivial transforms are obtained by training over a large class of video sequences and are shown below:

# Complexity-Reduction via Lifting Factorization

To reduce the complexity of the ROT, factorize each Given’s rotation matrix into lifting steps as shown below

where

In the above identities, observe that u = w = -tan(θ/2). However, in an actual implementation, better performance can be achieved by setting u and w to different approximations of –tan(θ/2), so that u and w are indeed distinct.

The inverse transforms with perfect invertibility are trivially obtained by replacing u, v, w with –u, -v, -w in Equations (8) to (13).

To achieve complexity reduction, the lifting multipliers u, v, w are approximated by rationals of the form k/32 so that each multiplier uses only 5-bit precision. Here are some of the advantages of this lifting implementation:

- Low-complexity implementation with perfect invertibility,

- In-place computations,

- Multipliers with 5-bit precision can be easily implemented using only adders and shifters.

## Ensuring Numerical Stability

For θ =nπ+Є, where n is an odd integer and Є is small, observe that 
                                lim tan(θ/2) → +/-∞,            (16)
                                            Є→0

and the lifting factorization becomes numerically instable. We avoid this instability by applying the following simple substitution:

Rz(θ) = Rz(θ-π/2) Rz(π/2)               (17)

This substitution eliminates the instability because for θ =nπ+Є, we have θ-π/2 = nπ/2+Є and tan(nπ/4+Є/2) is a numerically stable operation. Also note that Rz(π/2) is a trivial operation in which a pair of transform coefficients are swapped and the sign of one coefficient is flipped.

## Pseudocode

To illustrate the simplicity of our implementation, we provide the pseudocode for the forward and inverse rotational transforms. We use C-style syntax and the following notation:

short matrix[64] = 8x8 transform coefficients in row-major order

short temp[64] = intermediate storage

short Ai, Bi, Ci, Di, Fi, Gi, dum = temporary variables

short u[4][6] =

{//----vertical-|---horizontal---

{ -7,  -7,   5,  -4,  -2,  -2, },// indexROT = 0

{  4, -22,   9,  -7,  -4,   5, },// indexROT = 1

{ -3,  -5,  -3,   2, -29,   5, },// indexROT = 2

{-10,  -6,  -4,  -1, -29,   6, },// indexROT = 3

};

short v[4][6] =

{//-----vertical-|-horizontal--

{ 14,  13, -10,   8,   3,   4, },// indexROT = 0

{ -8,  30, -17,  13,   7, -10, },// indexROT = 1

{  5,   9,   6,  -5,  32, -10, },// indexROT = 2

{ 18,  11,   8,   1,  32, -12, },// indexROT = 3

};

w[][] is identical to u[][] except for w[0][4] = -1.

### Forward, Horizontal ROT

// Forward, horizontal ROT: Rh, Equation (3)

for (n=0; n<64; n+=8){

// Rx, Equation (6)

A0 = matrix[n  ] + (w[indexROT][3] * matrix[n+1])>>5

A3 = matrix[n+3] + (w[indexROT][3] * matrix[n+4])>>5

B1 = (v[indexROT][3] * A0)>>5 + matrix[n+1]

B4 = (v[indexROT][3] * A3)>>5 + matrix[n+4]

C0 = A0 + (u[indexROT][3]  *  B1)>>5

C3 = A3 + (u[indexROT][3]  *  B4)>>5

// Rz(π/2) swap and sign flip, Equation (17)

if ((indexROT == 2) || (indexROT == 3))

dum = B1

B1 = -matrix[n+2]

matrix[n+2] = dum

dum = B4

B4 = -matrix[n+5]

matrix[n+5] = dum

// Rz, Equation (7)

D1 = B1 + (w[indexROT][4]  *  matrix[n+2])>>5

D4 = B4 + (w[indexROT][4]  *  matrix[n+5])>>5

temp[n+2] = (v[indexROT][4]  *  D1)>>5 + matrix[n+2]

temp[n+5] = (v[indexROT][4]  *  D4)>>5 + matrix[n+5]

F1 = D1 + (u[indexROT][4] * temp[n+2])>>5

F4 = D4 + (u[indexROT][4] * temp[n+5])>>5

// Rx, Equation (6)

G0 = C0 + (w[indexROT][5] * F1)>>5

G3 = C3 + (w[indexROT][5] * F4)>>5

temp[n+1] = (v[indexROT][5] * G0)>>5 + F1

temp[n+4] = (v[indexROT][5] * G3)>>5 + F4

temp[n  ] = G0 + (u[indexROT][5] * temp[n+1])>>5

temp[n+3] = G3 + (u[indexROT][5] * temp[n+4])>>5

} // for: horizontal ROT

### Forward, Vertical ROT

// Forward, vertical ROT: Rv, Equation (3)

for (n=0; n<8; n++) {

// Rx, Equation (6)

A0 = temp[n   ] + (w[indexROT][0] * temp[n+8 ])>>5

A3 = temp[n+24] +(w[indexROT][0] * temp[n+32])>>5

B1 = (v[indexROT][0] * A0)>>5 + temp[n+8]

B4 = (v[indexROT][0] * A3)>>5 + temp[n+32]

C0 = A0 + (u[indexROT][0] * B1)>>5

C3 = A3 + (u[indexROT][0] * B4)>>5

// Rz(π/2) swap and sign flip, Equation (17)

if (indexROT == 1)

dum = B1

B1 = -temp[n+16]

temp[n+16] = dum

dum = B4

B4 = -temp[n+40]

temp[n+40] = dum

// Rz, Equation (7)

D1 = B1 + (w[indexROT][1] * temp[n+16])>>5

D4 = B4 + (w[indexROT][1] * temp[n+40])>>5

matrix[n+16] = (v[indexROT][1] * D1)>>5 + temp[n+16]

matrix[n+40] = (v[indexROT][1] * D4)>>5 + temp[n+40]

F1 = D1+ (u[indexROT][1] * matrix[n+16])>>5

F4 = D4+ (u[indexROT][1] * matrix[n+40])>>5

// Rx, Equation (6)

G0 = C0 + (w[indexROT][2] * F1)>>5

G3 = C3 + (w[indexROT][2] * F4)>>5

matrix[n+ 8 ] = (v[indexROT][2]*G0[n])>>5 + F1

matrix[n+32 ] = (v[indexROT][2]*G0[n])>>5 + F4

matrix[n    ] = G0[n] + (u[indexROT][2] * matrix[n+ 8])>>5

matrix[n+24 ] = G3[n] + (u[indexROT][2] * matrix[n+ 32])>>5

} // for: vertical ROT

### Inverse, Vertical ROT

// Inverse, vertical ROT: Equation (2), Rv:

for (n=0; n<8; n++){

// Rx, Equation (6)

G3 = matrix[n+24] - (u[indexROT][2] * matrix[n+32])>>5

G0 = matrix[n    ] - (u[indexROT][2] * matrix[n+8])>>5

F4 = matrix[n+32] - (v[indexROT][2] * G3)>>5

F1 = matrix[n+  8] - (v[indexROT][2] * G0)>>5

C3 = G3 - (w[indexROT][2]*F4)>>5

C0 = G0 - (w[indexROT][2]*F1)>>5

// Rz, Equation (7)

D4 = F4 - (u[indexROT][1] * matrix[n+40])>>5

D1 = F1 - (u[indexROT][1] * matrix[n+16])>>5

temp[n+40] = matrix[n+40] - (v[indexROT][1] * D4)>>5

temp[n+16] = matrix[n+16] - (v[indexROT][1] * D1)>>5

B4 = D4 - (w[indexROT][1] * temp[n+40])>>5

B1 = D1 - (w[indexROT][1] * temp[n+16])>>5

// Rz(π/2) swap and sign flip, Equation (17)

if (indexROT == 1)

dum = B1

B1 = temp[n+16]

temp[n+16] = -dum

dum = B4

B4 = temp[n+40]

temp[n+40] = -dum

// Rx, Equation (6)

A3 = C3 - (u[indexROT][0] * B4)>>5

A0 = C0 - (u[indexROT][0] * B1)>>5

temp[n+32] = B4 - (v[indexROT][0] * A3)>>5

temp[n+ 8] = B1 - (v[indexROT][0] * A0)>>5

temp[n+24] = A3 - (w[indexROT][0] * temp[n+32])>>5

temp[n   ] =  A3 - (w[indexROT][0] * temp[n+8])>>5

} // for: vertical ROT

### Inverse, Horizontal ROT

// Inverse, horizontal ROT: Rh, Equation (3)

for (n=0; n<64; n+=8){

// Rx, Equation (6)

G3 = temp[n+3] - (u[indexROT][5] * temp[n+4])>>5

G0 = temp[n  ] - (u[indexROT][5] * temp[n+1])>>5

F4 = temp[n+4] - (v[indexROT][5] * G3)>>5

F1 = temp[n+1] - (v[indexROT][5] * G0)>>5

C3 = G3 - (w[indexROT][5] * F4)>>5

C0 = G0 - (w[indexROT][5] * F1)>>5

// Rz, Equation (7)

D4 = F4 – (u[indexROT][4] * temp[n+5])>>5

D1 = F1 - (u[indexROT][4] * temp[n+2])>>5

matrix[n+5] = temp[n+5] - (v[indexROT][4] * D4)>>5

matrix[n+2] = temp[n+2] - (v[indexROT][4] * D1)>>5

B4 = D4 - (W[indexROT][4] * matrix[n+5])>>5

B1 = D1 - (W[indexROT][4] * matrix[n+2])>>5

// Rz(π/2) swap and sign flip, Equation (17)

if ((indexROT == 2) || (indexROT == 3))

dum = B1

B1 = matrix[n+2]

matrix[n+2] = -dum

dum = B4

B4 = matrix[n+5]

matrix[n+5] = -dum

// Rx, Equation (6)

A3 = C3 - (u[indexROT][3] * B4)>>5

A0 = C0 - (u[indexROT][3] * B1)>>5

matrix[n+4] = B4 - (v[indexROT][3] * A3)>>5

matrix[n+1] = B1 - (v[indexROT][3] * A0)>>5

matrix[n+3] = A3 - (w[indexROT][3] * matrix[n+4])>>5

matrix[n  ] = A0 - (w[indexROT][3] * matrix[n+1])>>5

} // for: horizontal ROT

# Eliminating Redundant ROT-Associated RDOQ Processing

Figure 3: ROT encoding scheme with RDOQ

The RDOQ based encoder design is illustrated in Figure 3. To include the ROT as secondary transform, we have to embed the ROT inside the RDOQ loop. To compact energy after primary transform, it usually requires multiple rotational transforms (corresponding to different rotational angles). Let N be the number of rotational transforms, then it would have (N+1) loops. This is indeed a significant computational complexity demand, and it is not practical for application purpose.

In this proposal, for the low-complexity scenario, we leverage the characteristics of ROT transform and break the RDOQ loop (called RDOQ loop splitting) to avoid multiple RDOQ process for the same block. Typically, ROT is applied at the upper-left block of the transform block as shown in Figure 4, where M can be 32, 16 and 8, m is 8 . Let us assume M = 16 and m = 8. It means we apply 8x8 block based ROT on the upper-left 8x8 block for each 16x16 transform block. As we can see, for each RDOQ loop, different ROT is applied to upper-left 8x8 block only. Hence only coefficients inside upper-left 8x8 block could be modified and the rest coefficients are kept as the same.

Figure 4: mxm block based rotational transform on MxM transform block

After applying the ROT, different scanning pattern is used to scan the 2-D coefficients into a 1-D vector for quantization and encoding. This kind of scanning can be popular zigzag, horizontal, vertical and other specialized patterns. In the following context, zigzag scanning is used as an example to demonstrate our proposal. Our proposal can be applied to other scanning pattern as well.  As shown in Figure 5, zigzag is used to scan coefficients after ROT to form 1-D vector. We can see that coefficients won’t be changed after a certain cut-off position. This cut-off position depends on the rotational transform block size. For example,  we are using 8x8 ROT, the cut-off position is shown in Figure 4. Since ROT is only applied on upper-left block, there aren’t any coefficient changes between RDOQ loops. Therefore, we split the large block into two sub block at cut-off position, where the first block which will be affected by ROT is defined as ROT block, and the other is defined as non-ROT block. Note that the actual implementation for this proposal uses a diagonal scan instead of the zigzag scan in the preceding illustrative example.

Figure 5: Zig-zag scanning on 16x16 block. Bold block is applied with ROT. Shadowed block represents cut-off position.

In this proposal, we encode the non-ROT block at once and store the necessary states, such as distortion, rate-distortion cost, quantized transform coefficients (levels and runs), context models, etc. We only apply the multiple encoding on ROT block where coefficients will be changed by each ROT loop. This type of RDOQ loop split reduces the computational complexity at encoder.

# Experimental Results

Experiments are conducted in the HM5.0 software according to the common test conditions. Both lifting and RDOQ loop splitting are implemented and compared against the HM5.0 anchor.

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A (8bit) | -0.7% | 0.5% | 0.4% |
| Class B | -0.8% | 0.6% | 0.4% |
| Class C | -0.5% | 0.2% | 0.2% |
| Class D | -0.5% | 0.2% | 0.3% |
| Class E | -0.5% | 0.2% | 0.2% |
| Overall | -0.6% | 0.3% | 0.3% |
|  | -0.6% | 0.3% | 0.3% |
| Class F | -0.2% | -0.2% | 0.2% |
| Enc Time[%] | 142% |  |  |
| Dec Time[%] | 101% |  |  |

Table 1: Coding efficiency and encoding/decoding time for ROT over HM5.0

# Bit precision

The maximum bit precision for intermediate storage is 16 bits. The functions which implement the lifting transforms use “short” datatype exclusively. The lifting multipliers use 5-bit precision.

# Memory requirements

Additional storage is required for lifting multipliers. Each lifting multiplier has 5-bit precision and can be packed into a byte. There are 4*6*2 + 1 = 49 lifting multipliers. Therefore 49 bytes are required.

# Quantization

No additional quantization matrices are required.

# R-D optimization

Rate-distortion iterations are performed at the encoder to select the best rotation-angle set from a dictionary of 4 non-trivial sets and one trivial zero set.

# Other statistics

No memory transfers, reordering, or other data operations are performed.

# Subjective effects

We did not observe any subjective differences.

# Operation Counts

Table 2 provides the worst-case operation counts for the inverse ROT transform. Because the forward transform is identical to the inverse transform modulo changing adders to subtractors, the operations  count for the forward transform is identical.

To compute the worst case, recall that the ROT dictionary contains 4 non-trivial sets of rotation angles and one trivial set, the zero set (no ROT). Only one set is selected for a given TU. For adder counts, all 4 non-trivial sets have the same number of adders, 288. For multiplier counts, several multiplications are by powers of 2 and would be implemented using shifts. The worst-case multiplier count occurs for the third non-trivial set of rotation angles in which 240 actual multipliers are required. However, note that these are low-complexity multiplications because each multiplication is by a 5-bit constant. In a hardware implementation, these would be replaced with low-complexity adds and shifts as explained in [3]. For this worst-case the corresponding number of shifts is 336.

We have also provided the operations count for the average-case assuming that all rotation indices occur equiprobably. These averages are obtained by first summing the number of adds, mults and shifts in the code for all 4 sets of non-trivial rotation angles. Assuming a uniform distribution on the 5 rotation indices, to get the average op-count, these sums are then divided by 5 because there is one trivial (zero) rotation-angle set and 4 non-trivial sets.

| Transform size |  | HM 2 core 2-D transform | JCTVC-E243 partial butterfly implementation of 2-D inverse DCT | ROT Inverse Transform (worst-case) | ROT Inverse Transform (average-case) |
| --- | --- | --- | --- | --- | --- |
| 4x4 | Mults | 0 | 48 | 0 | 0 |
|  | Adds | 80 | 96 | 0 | 0 |
|  | Shifts | 32 | 32 | 0 | 0 |
| 8x8 | Mults | 0 | 352 | +240 | +163 |
|  | Adds | 640 | 576 | +288 | +230 |
|  | Shifts | 352 | 128 | +336 | +298 |
| 16x16 | Mults | 1408 | 2752 | +240 | +163 |
|  | Adds | 2624 | 3712 | +288 | +230 |
|  | Shifts | 1600 | 512 | +336 | +298 |
| 32x32 | Mults | 7424 | 21888 | +240 | +163 |
|  | Adds | 13440 | 25856 | +288 | +230 |
|  | Shifts | 7296 | 2048 | +336 | +298 |

Table 2: Comparison of operation counts

# Conclusion

The simulation results show high performance gains of 0.6% (AI-HE) from the ROT. We recommend HM adoption of the ROT as an additional transform.

# References

- A. Saxena and F. Fernandes, “CE7: Mode-dependent DCT/DST without 4*4 full matrix multiplication for intra prediction,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E125, 5th JCT-VC Meeting, Geneva, CH,  Mar. 2011.

- K. McCann, W.-J. Han and I.-K. Kim, “Samsung’s Response to the Call for Proposals on Video Compression Technology,” JCTVC-A124, Dresden, Germany, April 2010.

- F. Fernandes, “Low-Complexity Rotational Transform”, JCTVC-C096, Guangzhou, October 2010.

- Z. Ma, F. Fernandes, E. Alshina, A. Alshin, “CE 7: Experimental Results for the Rotational Transform”, JCTVC-F294, Torino, July 2011.

- Z. Ma, F. Fernandes, E. Alshina, A. Alshin, “Non-CE 7: Supplementary Results for the Rotational Transform”, JCTVC-G591, Geneva, November 2011.

# Appendix: Draft Text

### Transform coefficient syntax

| transform_coeff( x0, y0, log2TrafoSize, trafoDepth, cIdx ) { | Descriptor |
| --- | --- |
| if( ( ( cIdx = = 0 && cbf_luma[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 1 && cbf_cb[ x0 ][ y0 ][ trafoDepth ] ) ¦ ¦ ( cIdx = = 2 && cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) ) ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta | se(v) ¦ ae(v) |
| IsCuQpDeltaCoded = 1 |  |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| transform_coeff( x0, y0, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x1, y0, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x0, y1, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| transform_coeff( x1, y1, log2TrafoSize − 1, trafoDepth + 1, cIdx ) |  |
| } else { |  |
| if ( PredMode = = MODE_INTRA ) { |  |
| if (cIdx = = 0) |  |
| scanIdx = ScanType[ log2TrafoSize -2][ IntraPredMode ] |  |
| Else |  |
| scanIdx = ScanTypeC[ log2TrafoSize -2][ IntraPredModeC ] |  |
| } else { |  |
| scanIdx = 0 |  |
| } |  |
| if (log2TrafoSize > 2) { |  |
| rotational_transform_idx | ue(v)¦ae(v) |
| } |  |
| if ( entropy_coding_mode_flag ) |  |
| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) |  |
| else if ( !entropy_coding_mode_flag ) |  |
| residual_coding_cavlc( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) |  |
| } |  |
| } |  |
| } |  |

## Transform unit semantics

rotational_transform_idx specifies the rotational transform index used in the rotational transform process for the transform coefficients.

## Scaling and transformation process

Inputs to this process are:

–	a luma location ( xT, yT ) specifying the top-left luma sample of the current transform unit relative to the top-left luma sample of the current picture,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding unit,

–	a variable cIdx specifying the chroma component of the current block,

–	a variable nS specifying the size of the current transform unit.

Output of this process is:

–	a modified version of the (nS)x(nS) array of residual samples r with elements rij.

The quantization parameter qP is derived as follows.

–	If cIdx is equal to 0,

qP = QP’Y			(15-1)

–	Otherwise,

qP = QP’C			(15-2)

The (nS)x(nS) array of residual samples r are derived as specified in the following ordered steps.

Depending on the values of rotational_transform_idx, the following applies.

- If rotational_transform_idx is equal to 1, the following ordered steps are specified:

- The rotational transformation process as specified in subclause xxxx is invoked with bitDepth, qP, and (nS)x(nS) array c as the inputs and the output is assigned to the (nS)x(nS) array dij of scaled transform coefficients.

- The scaling process for transform coefficients as specified in subclause Error: Reference source not found is invoked with the size of the transform unit nS, the (nS)x(nS) array d, the chroma component variable cIdx and the quantization parameter qP as the inputs and the output is a scaled transform coefficient (nS)x(nS) array e.

- The transformation process for scaled transform coefficients as specified in subclause Error: Reference source not found is invoked with the size of the transform unit nS, the scaled transform coefficient (nS)x(nS) array e and the chroma component variable cIdx as the inputs and the output is a residual samples (nS)x(nS) array r.

- Otherwise (rotational_transform_idx is equal to 0), the following ordered steps are specified:

- The inverse scanning process for transform coefficients as specified in subclause Error: Reference source not found is invoked with the size of the current transform unit nS, the list of transform coefficients transCoeffLevel[ xT ][ yT ][ trafoDepth ][ cIdx ] and the variable cIdx as the inputs and the output is a (nS)x(nS) array cij.

- The scaling process for transform coefficients as specified in subclause Error: Reference source not found is invoked with the size of the transform unit nS, the (nS)x(nS) array c, the chroma component variable cIdx and the quantization parameter qP as the inputs and the output is a scaled transform coefficient (nS)x(nS) array d.

- The transformation process for scaled transform coefficients as specified in subclause Error: Reference source not found is invoked with the size of the transform unit nS, the scaled transform coefficient (nS)x(nS) array d and the chroma component variable cIdx as the inputs and the output is a residual samples (nS)x(nS) array r.

## Rotational transform process for 8x8 transform coefficients

Input to this process is an 8x8 array of transform coefficients s with elements sij

Output of this process is transform coefficients as an 8x8 array d with elements dij.

When the value of variable idx is set to rotational_transform_idx – 1, output element dij is specified as follows.

- For i=0..7, a set of intermediate values tij is derived by

g3 = si,3 - (u[idx][2] * si,4)>>5

g0 = si,0 - (u[idx][2] * si,1)>>5

f4 = si,4 - (v[idx][2] * g3)>>5

f1 = si,1 - (v[idx][2] * g0)>>5

c3 = g3 - (w[idx][2]*f4)>>5

C0 = g0 - (w[idx][2]*f1)>>5

d4 = f4 - (u[idx][1] * si,5])>>5

d1 = f1 - (u[idx][1] * si,2])>>5

ti,5 = si,5 - (v[idx][1] * d4)>>5

ti,2 = si,2 - (v[idx][1] * d1)>>5

b4 = d4 - (w[idx][1] * ti,5)>>5

b1 = d1 - (w[idx][1] * ti,2)>>5

if (idx == 1)

e = b1

b1 = ti,2

ti,2 = -e

e = b4

b4 = ti,5

ti,5 = -e

a3 = c3 - (u[idx][0] * b4)>>5

a0 = c0 - (u[idx][0] * b1)>>5

ti,4 = b4 - (v[idx][0] * a3)>>5

ti,1 = b1 - (v[idx][0] * a0)>>5

ti,3 = a3 - (w[idx][0] * ti,4)>>5

ti,0 = a3 - (w[idx][0] * ti,1)>>5

ti,6 = si,6

ti,7 = si,7

- For j=0...7, the transformed result sij is computed from these intermediate values tij as follows:

g3 = t3,j - (u[idx][5] * t4,j)>>5

g0 = t0,j - (u[idx][5] * t1,j)>>5

f4 = t4,j - (v[idx][5] * g3)>>5

f1 = t1,j - (v[idx][5] * g0)>>5

c3 = g3 - (w[idx][5] * f4)>>5

c0 = g0 - (w[idx][5] * f1)>>5

// Rz, Equation (7)

d4 = f4 – (u[idx][4] * t5,j)>>5

d1 = f1 - (u[idx][4] * t2,j)>>5

d5,j = t5,j - (v[idx][4] * d4)>>5

d2,j = t2,j - (v[idx][4] * d1)>>5

b4 = d4 - (W[idx][4] * d5,j)>>5

b1 = d1 - (W[idx][4] * d2,j)>>5

// Rz(π/2) swap and sign flip, Equation (17)

if ((idx == 2) || (idx == 3))

e = b1

b1 = d2,j

d2,j = -e

e = b4

b4 = d5,j

d5,j = -e

// Rx, Equation (6)

a3 = c3 - (u[idx][3] * b4)>>5

a0 = c0 - (u[idx][3] * b1)>>5

d4,j = b4 - (v[idx][3] * a3)>>5

d1,j = b1 - (v[idx][3] * a0)>>5

d3,j = a3 - (w[idx][3] * d4,j)>>5

d0,j = a0 - (w[idx][3] * d1,j)>>5

d6,j = t6,j

d7,j = t7,j

The multiplier matrices u, v, w are specified in Tables 1, 2 and 3.

|  | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| idx = 0 | -7 | -7 | 5 | -4 | -2 | -2 |
| idx = 1 | 4 | -22 | 9 | -7 | -4 | 5 |
| idx = 2 | -3 | -5 | -3 | 2 | -29 | 5 |
| idx = 3 | -10 | -6 | -4 | -1 | -29 | 6 |

Table 1: u Lifting Multipliers

|  | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| idx = 0 | 14 | 13 | -10 | 8 | 3 | 4 |
| idx = 1 | -8 | 30 | -17 | 13 | 7 | -10 |
| idx = 2 | 5 | 9 | 6 | -5 | 32 | -10 |
| idx = 3 | 18 | 11 | 8 | 1 | 32 | -12 |

Table 2: v Lifting Multipliers

|  | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| idx = 0 | -7 | -7 | 5 | -4 | -1 | -2 |
| idx = 1 | 4 | -22 | 9 | -7 | -4 | 5 |
| idx = 2 | -3 | -5 | -3 | 2 | -29 | 5 |
| idx = 3 | -10 | -6 | -4 | -1 | -29 | 6 |

Table 3: w Lifting Multipliers

# Patent rights declaration(s)

Samsung Electronics Co. Ltd may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).