| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0458 |
| --- | --- |

| Title: | Non-CE1: High Throughput Coding Scheme with Rice Binarization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | T. Nguyen D. Marpe M. Siekmann T. Wiegand Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49-30-31002-230 tung.nguyen@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

A demand for a high throughput mode defined by the number of bins coded in the bypass mode of CABAC relative to the number of bins coded with adaptive context models partially exists. For that, this document proposes a simple coding scheme bases on the existing adaptive Rice binarization of CABAC. The benefit of the presented scheme is the usage of existing coding tools and thus the high throughput mode can be treated as an extension of the common conditions.

# Description

Fig. 1: Binarization scheme of transform coefficient levels

Instead of splitting the coding of transform coefficient in multiple scan passes, the proposed scheme codes all information on a transform coefficient in each step. Further, instead of using the Truncated Unary (TU) binarization scheme for the first interval as done in the current HEVC design and as illustrated in Fig. 1, the proposed scheme starts directly with the Rice binarization scheme. Thus, the TU binarization scheme is neglected as illustrated in Fig. 2. Therefore, all bins resulting from the binarization of a transform coefficient level are coded in the bypass mode of CABAC. Note that proposed scheme does not change the maximum Rice parameter r compared to the current design of HEVC and does not change the variable bound for the beginning of the 0th Exp-Golomb binarization. Consequently, 3 modifications are made compared to the current design of HEVC:

- Usage of a single scan pass instead of multiple scan passes

- Evaluation of absolute sum of neighbors instead of number of significant neighbors for all scan positions regardless of transform block size and shape

- Modified Rice parameter derivation

Fig. 2: Proposed binarization scheme for transform coefficient levels

## Rice Parameter Derivation

The Rice parameter r is derived as follows. For the last significant scan position, where significant_coeff_flag is inferred as 1, the Rice parameter r is 0. For all other scan positions, regardless of transform block size and shape, the local template as defined in the current design of HEVC for the context model selection of significant_coeff_flag is applied. All neighbors are evaluated in contrast to the current design in HEVC. Depending on the resulting absolute sum of the neighbors covered by the local template, the Rice parameter is selected. This simple derivation rule is summarized in the following formulas.

sumabsolutelevel=xi

and	rx=0123x∈0,4x∈[5,10]x∈11,23x>23

with	x=sumabslolutelevel

## Remarks

In order to overcome the issue with the non-existing run-length coding mode, the proposed scheme modify the quantization for the case with RDOQ off. In this case, the last significant coefficient is quantized to zero if the cost for the coding of the last transform coefficient level and following non-significant transform coefficient levels is higher than to quantize the last significant coefficient to 0. Furthermore, the proposed scheme is similar to the proposed cleanup JCTVC-H0228 [1] for transform coefficient coding.

# Results

Beside the coding performance results, additional results on the number of bypass bins relative to the number of bins coded with adaptive context models are reported as in [2]. Compared to the results provided in [2] and [3], the proposed scheme has similar savings in terms of bypass bins to context adaptive coded bins ratio. The code base for this experiment is the HM 5.0 software. Simulations run on a cluster with the similar hardware configurations. Simulation configurations base on common test conditions defined in [4] and in [5] for the Low Qp condition. The complete results is available in the enclosed excel sheet.

| Qp | AI_HE | RA_HE | LB_HE | RA_10 | AI_LC | RA_LC | LB_LC | Average | Saving |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 14 % | 17 % | 18 % | 13 % | 15 % | 20 % | 20 % | 17 % | 83 % |
| 5 | 18 % | 20 % | 20 % | 16 % | 18 % | 22 % | 23 % | 20 % | 80 % |
| 9 | 20 % | 22 % | 22 % | 17 % | 20 % | 26 % | 26 % | 22 % | 78 % |
| 13 | 21 % | 25 % | 25 % | 19 % | 23 % | 29 % | 29 % | 24 % | 76 % |
| 22 | 27 % | 38 % | 38 % | 25 % | 30 % | 42 % | 41 % | 34 % | 66 % |
| 27 | 34 % | 50 % | 52 % | 32 % | 35 % | 53 % | 54 % | 44 % | 56 % |
| 32 | 41 % | 60 % | 63 % | 43 % | 43 % | 62 % | 63 % | 54 % | 46 % |
| 37 | 50 % | 68 % | 72 % | 58 % | 50 % | 70 % | 73 % | 63 % | 37 % |

Common Test Conditions Qp ϵ [22,27,32,37]

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 5.1% | 6.9% | 7.4% | 5.5% | 6.0% | 5.9% |
| Class B | 6.4% | 8.7% | 8.7% | 7.7% | 9.7% | 10.2% |
| Class C | 5.3% | 7.0% | 6.7% | 5.3% | 6.2% | 6.0% |
| Class D | 4.8% | 6.8% | 6.4% | 4.9% | 6.0% | 5.7% |
| Class E | 5.3% | 8.3% | 7.4% | 5.6% | 7.1% | 6.9% |
| Overall | 5.5% | 7.6% | 7.4% | 6.0% | 7.2% | 7.2% |
|  | 5.4% | 7.5% | 7.3% | 6.0% | 7.2% | 7.2% |
| Class F | 3.8% | 4.9% | 4.8% | 3.4% | 4.2% | 3.6% |
| Enc Time[%] | 99% |  |  | 106% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 4.2% | 7.7% | 7.6% | 3.7% | 5.3% | 4.9% |
| Class B | 5.8% | 8.2% | 7.9% | 5.8% | 6.6% | 6.8% |
| Class C | 4.7% | 6.3% | 5.9% | 4.8% | 4.5% | 4.1% |
| Class D | 4.8% | 6.1% | 5.7% | 5.0% | 4.4% | 3.7% |
| Class E |  |  |  |  |  |  |
| Overall | 5.0% | 7.1% | 6.7% | 5.1% | 5.3% | 5.0% |
|  | 5.0% | 7.0% | 6.7% | 5.1% | 5.2% | 5.0% |
| Class F | 3.5% | 4.6% | 4.5% | 2.9% | 3.2% | 3.1% |
| Enc Time[%] | 99% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 7.8% | 10.2% | 9.7% | 9.9% | 8.8% | 8.4% |
| Class C | 6.8% | 8.6% | 7.8% | 9.3% | 7.9% | 7.1% |
| Class D | 7.0% | 7.5% | 7.0% | 9.0% | 7.2% | 5.1% |
| Class E | 7.2% | 8.6% | 7.8% | 9.1% | 7.1% | 5.0% |
| Overall | 7.2% | 8.8% | 8.2% | 9.4% | 7.8% | 6.6% |
|  | 7.2% | 8.8% | 8.1% | 9.4% | 7.8% | 6.7% |
| Class F | 4.4% | 6.4% | 5.5% | 5.2% | 4.4% | 5.5% |
| Enc Time[%] | 99% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

CE1 Low Qp Conditions Qp ϵ [1,5,9,13]

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 1.2% | 1.6% | 1.7% | 0.7% | 0.8% | 0.9% |
| Class B | 2.5% | 2.8% | 3.2% | 2.2% | 2.2% | 2.5% |
| Class C | 1.5% | 1.6% | 1.8% | 1.1% | 1.0% | 1.2% |
| Class D | 1.1% | 1.0% | 1.1% | 0.6% | 0.4% | 0.5% |
| Class E | 3.1% | 4.4% | 4.6% | 2.7% | 3.9% | 3.9% |
| Overall | 1.9% | 2.3% | 2.5% | 1.5% | 1.7% | 1.8% |
|  | 2.3% | 2.3% | 2.1% | 1.7% | 1.8% | 1.7% |
| Class F | 1.8% | 2.3% | 2.4% | 1.3% | 1.5% | 1.5% |
| Enc Time[%] | 90% |  |  | 92% |  |  |
| Dec Time[%] | 91% |  |  | 91% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 3.8% | 4.7% | 4.7% | 3.6% | 4.0% | 3.9% |
| Class B | 4.8% | 6.9% | 7.0% | 5.4% | 6.7% | 6.2% |
| Class C | 4.8% | 5.3% | 5.5% | 4.8% | 5.2% | 4.5% |
| Class D | 3.0% | 3.9% | 3.9% | 2.2% | 3.1% | 3.0% |
| Class E |  |  |  |  |  |  |
| Overall | 4.2% | 5.4% | 5.5% | 4.1% | 5.0% | 4.6% |
|  | 4.0% | 4.8% | 5.4% | 4.1% | 4.8% | 4.6% |
| Class F | 2.8% | 3.1% | 3.2% | 2.6% | 2.6% | 2.3% |
| Enc Time[%] | 90% |  |  | 93% |  |  |
| Dec Time[%] | 93% |  |  | 93% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 4.8% | 6.5% | 7.2% | 4.9% | 6.3% | 6.6% |
| Class C | 5.0% | 5.8% | 6.4% | 4.6% | 6.0% | 6.1% |
| Class D | 3.4% | 4.4% | 4.9% | 2.4% | 3.9% | 4.1% |
| Class E | 7.6% | 8.6% | 8.8% | 7.0% | 6.2% | 5.9% |
| Overall | 5.0% | 6.2% | 6.7% | 4.6% | 5.6% | 5.7% |
|  | 4.8% | 5.2% | 6.3% | 4.5% | 5.4% | 5.7% |
| Class F | 3.6% | 4.0% | 4.3% | 3.7% | 4.2% | 4.2% |
| Enc Time[%] | 91% |  |  | 93% |  |  |
| Dec Time[%] | 93% |  |  | 93% |  |  |

# References

- T. Nguyen, D. Marpe, and T. Wiegand, “Non-CE11: Proposed Cleanup for Transform Coefficient Coding,” JCTVC-H0228, San Jose, January 2012

- J. Lainema, K. Ugur, and A. Hallapuro, “CE1.D1: Nokia report on high throughput binarization,” JCTVC-H232, San Jose, January 2012

- J. Lainema, K. Ugur, and A. Hallapuro, “Non-CE1: CABAC bypass for coefficient data,” JCTVC-H233, San Jose, January 2012

- F. Bossen, “Common HM test conditions and software reference configurations,” JCTVC-G1200, Geneva, Nov. 2011.

- R. Joshi et. al., “Description of Core Experiment 1 (CE1): Entropy coding investigation,” JCTVC-G1201, Geneva, Nov. 2011.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e. V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).