| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0460 WG11 Number: m23336 |
| --- | --- |

| Title: | High-level Syntaxes for the Scaling List Matrices Parameters and Parametric coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Munsi Haque, Ehsan Maani, Ali Tabatabai 1730 N 1st Street San Jose, CA 95112 | Tel: Email: | +1-408-352-4099 munsi.haque@am.sony.com ehsan.maani@am.sony.com ali.tabatabai@am.sony.com |
| Source: | Sony Electronics Inc. / Sony Corp. |  |  |

_____________________________

# Abstract

This document presents the referencing of parametric models being part of  the Scaling List Matrix Coding technology group, as envisioned in APS referencing [1] and [2]. This contribution describes a group of scaling list parameters that  can be used to model a class of  scaling list matrices and thus become part of the scaling_list_param() sets.

It also shows some results of parametric model based coding of scaling matrices and states that it will be upto the encoder's choices on how accurately the scaling matrices could be re-produced at the decoder-side by sending either none to a very limited amount of prediction errors.

- Scaling Lists Matrix Parameters

This document presents the referencing of parametric models as a part of the Scaling List Matrix Coding. In the APS referencing [1] and [2], an aps_scaling_list_data_present_flag  indicates the presence of the syntax groups related to the Scaling List Matrix coding or scaling_list_param() inside aps_rbsp().

The proposal presents an efficient way of sending the Scaling List Matrices, as the encoder chooses to replaces the default scaling matrices for a certain class of video sequences. It can be done by sending a set of parameters that can predict the encoder-side scaling list matrices with either none or a minimum prediction error.

A set of limited number of  parameters (ex. 4 for a quadratic parameter model) can be used for each x and y directions to derive the predicted destination matrix. For symmetric matrix, the same parameters are used for both directions.

An example syntax structure for the scaling_list_param() set for APS is provided below with some details.

| scaling_list_param() | Descriptor |
| --- | --- |
| { |  |
| param_model_index | u(3) |
| if( param_model_index = 1) { // symmetric quadratic parameter model |  |
| parameter_coding_flag // flag for uniform or VLC coding? | u(1) |
| if (!parameter_coding_flag) // position-based uniform coding as an example |  |
| get_params() // get "a, b, c, d" parameters of the quadratic model | u(n) |
| else { // VLC (EGC, etc).coding of "a, b, c, d" parameters of the quadratic model } | ue(v) |
| } // end of param_model_index = 1 |  |
| else { // Other parameter model indices: asymmetric quadratic, HVS, etc. } |  |
| residual_error_present_flag | u(1) |
| if( residual_error_present_flag) { |  |
| residual_coding_mode | u(3) |
| if (residual_coding_mode = 0) { // AVC DPCM } |  |
| else if (residual_coding_mode = 1) { // Sony-TI or similar DPCM coding. } |  |
| else { // 2 to 7 - reserved ? } |  |
| } // end of residual_error_present_flag |  |
| } // end of scaling_list_param |  |

scaling_list_param_type  - 1 (parameter  model), 2 (parameter model index), (0, 3-7 - reserved)

parameter_coding_flag  - 0 (uniform coding), 1 (variable length coding).

param_model_type indicates what types of parameter models are present for the Scaling List Matrices (Table 1a).

param_list_index  indicates which multi param-list from a param-list table (Table 1b) is used for the Scaling List Matrices. For each parameter model type, a param-list table may contain a maximum of P multi param-lists where param_list_index, p = 0 to P-1.

get_param() gets all the parameters according to the parameter  model index descriptions for the parameter model without any VLC coding

Table 1a – parameter model types

| Parameter Model Index (param_model_index) | Parameter Model Type description | Parametric models |
| --- | --- | --- |
| 0 | reserved |  |
| 1 | Symmetric Quadratic | q(x, y) = a (x*x + y*y) + b xy + c (x + y) + d |
| 2 | Asymmetric Quadratic | q(x, y) = (a1 x + b1 y + c1) ( a2 x + b2 y + c2) |
| 3 | HVS | H(u,v) = a (b + c f (u,v)) exp( - (c f(u,v))**d ); Q(u,v) = q / H(u,v) |
| 4-7 | reserved | ... |

residual_error_present_flag - 0 (no error), 1 (Coded residual error).

residual_coding_mode - 8 different types coding modes: example - 0 (AVC-DPCM), 1 (Sony_TI), 2 to 7 = reserved.

# Scaling List Matrix Parametric Modeling and Coding

There has been many contribution on compression of scaling list matrices. In the contribution [5], a number of different compression techniques has been added (as different coding modes) and its related software.  Reference [5] provides a method of up-conversion for generation of larger matrices from smaller matrices e.g., 8x8 to 16x16 or 32x32. The same up-sampling approach has also been used in this contribution for the parametric model based coding. Here lower-sized matrices are created by using parametric models and the larger-sized matrices are derived by the same up-conversion technique presented in [5].

There are two possible ways to code the Scaling List Matrices presented in this proposal.

- Parameter Only Coding

- Parameters and Residual Prediction Error Coding.

For Live-update, a limited number of parameters can be sent directly instead of the Scaling List matrices. These parameters can be used at decoder-end to re-generate the matrices. So no prediction error needs to be coded for transmission. For larger scaling list matrices of 16x16 and 32x32, these can be computed by using the parameters of 8x8 matrices and i fractional grids to interpolate the elements of larger matrices, without coding any other parameters for them.

When Prediction Error Coding is also required by the encoder  in addition to the Parameter  coding to reproduce the scaling list matrices, the differential error (residual) can be coded using either AVC or other DPCM algorithms as proposed in contribution [5].

Below are some results shown for parametric model based coding and some comparison with the reference CE4.2.1 experiments [5] without the header bits.

Figure-1: Total bit-count for Scaling List Matrices for Symmetric Files

Figure-2: Total bit-count for Scaling List Matrices for Asymmetric Files

# Simulation results

The results are summarized in Table 1. For Option 1 test of Sony DPCM algorithms in reference [5], the generated bits for the Parametric Model based coding compared with the same reference varies from 109.13% to 129.57% for symmetric matrix files, and 106.76% to 116.29%  for asymmetric matrix files.

Table 1: Comparative Results of option 1

| (Subtest2.1, option1, Sony_DPCM) | symmetry1 | symmetry2 | symmetry3 | symmetry4 | symmetry5 | symmetry6 |
| --- | --- | --- | --- | --- | --- | --- |
| Total Bit | 550 | 372 | 686 | 668 | 414 | 568 |
| MaxError | 30 | 30 | 85 | 47 | 62 | 96 |
| AvgError | 4.11 | 5.23 | 18.23 | 6.32 | 17.35 | 34.11 |
| (Subtest2.1, option1, Sony_PARAM) | symmetry1 | symmetry2 | symmetry3 | symmetry4 | symmetry5 | symmetry6 |
| Total Bit | 641 | 482 | 782 | 729 | 525 | 662 |
| MaxError | 30 | 30 | 85 | 47 | 62 | 96 |
| AvgError | 4.11 | 5.23 | 18.23 | 6.32 | 17.35 | 34.11 |
| Ratio | 116.55% | 129.57% | 113.99% | 109.13% | 126.81% | 116.55% |
| (Subtest2.1, option1, Sony_DPCM) | asymmetry1 | asymmetry2 | asymmetry3 | asymmetry4 | asymmetry5 | asymmetry6 |
| Total Bit | 962 | 614 | 1146 | 1480 | 690 | 972 |
| MaxError | 28 | 21 | 103 | 129 | 55 | 143 |
| AvgError | 4.09 | 3.28 | 21.81 | 18.01 | 15.52 | 31.48 |
| (Subtest2.1, option1, Sony_PARAM) | asymmetry1 | asymmetry2 | asymmetry3 | asymmetry4 | asymmetry5 | asymmetry6 |
| Total Bit | 1088 | 714 | 1330 | 1580 | 792 | 1126 |
| MaxError | 28 | 21 | 103 | 129 | 55 | 143 |
| AvgError | 4.09 | 3.28 | 21.81 | 18.01 | 15.52 | 31.48 |
| Ratio | 113.10% | 116.29% | 116.06% | 106.76% | 114.78% | 115.84% |

# Conclusion

The contribution proposal has two parts. In the first part, it shows an example syntax structure for the scaling_matrix_param()  group to enable various means of conveying scaling list matrices data to the decoder-side. It allows flexibilities in terms of future upgrades and updates of the scaling matrix data handling.

In the second part, some results corresponding to the CE4 Subtest-1 with Parametric Model based Scaling List Matrix coding are presented. It shows a more compact way of coding the Scaling List matrices by using parametric models in two possible ways. In the first method, the encoder can use the parameters only models without coding any  residual error. In the second method, additionally residual errors can be transmitted for more accurate transmission  of scaling list matrices. Some comparative coding results are also provided.

We believe such coding mechanism of Scaling List matrices  result in more compact representation of these matrices  when perfect reconstruction of the scaling list matrices are not required by the encoder at the expense of some nominal computational loads at both encoder and decoder sides.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- M. Li, P. Wu, S. Wenger, J. Boyce, “APS Referencing”, JCTVC-Hxxx, January 2012.

- M. Li, P. Wu, J. Tanaka, Y. Morigami, T. Suzuki, K. Sato, “Using Multiple APSs for Quantization Matrix Parameters Signaling”, JCTVC-G1026, November 2011.

- J. Tanaka, Y. Morigami, T. Suzuki, “Non-CE4 Subtest3 : Extension of Adaptation Parameter Sets syntax for Quantization matrix”, JCTVC-G295, November 2011.

- E. Maani, M. Haque, A. Tabatabai, “Parameterization of Default Quantization Matrices”, JCTVC-G352, November 2011.

- Y. Morigami, J. Tanaka, T. Suzuki, "CE4 subtest 2.1: Enhancements of HEVC quantization matrices", JCTVC-H0230, February, 2012.