| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0463_r1 |
| --- | --- |

| Title: | Comments on specifying tile support in HEVC Profiles and Levels |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information & Proposal |  |  |
| Author(s) or Contact(s): | Wade Wan Broadcom Corporation 5300 California Ave Irvine, CA 92617 Tim Hellman Broadcom Corporation 200 Brickstone Square Suite 401 Andover, MA 01810 | Tel: Email: Tel: Email: | +1-949-926-6387 wwan@broadcom.com +1-978-719-1348 timhellman@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

Several techniques have been proposed in previous JCT-VC meetings and adopted into the general HEVC framework to enable parallelism for multi-core architectures.  As the committee attempts to reach CD (Committee Draft) status, it is unclear how these techniques map into appropriate Profile and Level definitions.  An example of these parallelism techniques is tiles.  This submission discusses several aspects related to supporting tiles in decoders to determine whether and how tiles would fit within a particular Profile/Level definition where every decoder conformant to that definition is required to support decoding of a stream encoded using tiles and some applications would not use tiles at all.

# Introduction

It is generally agreed that enabling parallelism in the HEVC design is beneficial to provide more options for different video coding implementations.  For this reason, several parallelism techniques have been proposed and were adopted into the general HEVC design.  Most of the benefits of these techniques are targeted towards assisting encoding implementations as a real-world decoder would not be able to rely on a stream to be encoded in a parallelizable fashion unless it is in a closed-system and/or depends on a Profile/Level specification that limits the streams it is expected to decode correctly to that specific parallelizable structure.  Therefore, it is important to analyze and assess the impact that tiles [1] would have on the interoperability of different decoder implementations.

# Discussion

## In-loop filtering across vertical tile boundaries

Tile dependencies across boundaries may involve all aspects of the HEVC codec: the entropy, the decoding and in-loop filtering.  It should be noted that in the current definition of independent tiles, the entropy and decoding are independent across tile boundaries but in-loop filtering does span across tile boundaries so the tiles are not completely independent.

A flag was proposed in [2] to signal whether in-loop filtering was allowed or prohibited across boundaries.  Note that while this could assist encoders, it does not change the worst-case requirement for decoders unless this flag is mandated in a Profile/Level to prohibit in-loop filtering across boundaries.  However, it is easy to envision and has been stated by some experts that there are significant visual artifacts when removing all of the in-loop filtering (deblocking, SAO and/or ALF).  Therefore, it does not seem feasible for subjective quality reasons to prohibit deblocking across tile boundaries.  However, it is unclear if there are artifacts if only the deblocking process is invoked at tile boundaries and the non-deblocking in-loop filtering is skipped.  This may be an area of investigation to reduce the complexity of tile support.

## Single-core decoders and tiles

It was claimed in [3] that single core decoder architectures do not benefit and in fact, may be penalized by having to support parallelism techniques such as tiles.  Of particular concern when supporting tiles is dependencies across vertical tile boundaries.  It was claimed that if a single-core decoder processed LCUs in the same order as the encoder, it would be cost-prohibitive as significant on-chip or DRAM storage was necessary in addition to other support logic, to handle the vertical tile boundaries.

## Dependent tiles

An alternative approach was presented in [4] to address the vertical boundary problem by changing how CABAC inheritance was specified so that it would be possible for a single-core decoder to process LCUs in raster-scan order across a picture (as it would for a picture not using tiles).  Note this processing order is different from the order used by the encoder.  It was determined towards the end of the last meeting that it was possible for the lower-left LCU to be available for an encoder processing LCUs in “tile order” that could not be available for a decoder processing LCUs in “raster scan order”.  It was suggested that this could be fixed by introducing an additional constraint on an encoder to disallow use of the lower left LCU for decoding by LCUs to the right of a vertical boundary.  This is an example of the ease in which a problem could arise when different LCU processing is used in the encoder and the decoder.  It is unclear if there are other potential issues that would complicate or prevent such a strategy.  It is also unclear whether dependent tiles are really needed as independent tiles (which are simpler to support) may be able to provide sufficient functionality.

## Constraining the maximum number of tile columns

Note that even if tile coding was constructed in a fashion to allow for a decoder to process LCUs in “raster scan order”, it does not completely solve the problem stated in [2].  It merely reduces it as there is still storage, logic and handling that must be performed at each vertical boundary to keep the decoder in the same state/context as the encoder.  The general tile concept has been accepted but with no constraints on how the technique may be used.  More specifically, there are no constraints on how many vertical tile boundaries could be used to partition a picture.  This is important because there is data/pointer management at each vertical boundary which is complicated because the location in the bitstream where each vertical tile boundary starts is not readily available at the start.  Therefore, the additional load/save processing will cause hiccups in a data pipeline as one cannot align the bits to be processed with the order of processing.  Note the bits to be processed are not the same as the encoded order with this alternative raster scan approach and it is difficult to determine the right interleave points until after you start decoding.  It should be noted that while there is a similar issue for wavefronts (another parallelizable technique), wavefront schemes do not involve more than one save/reload per LCU row (when substream interleaving is not used).  For tiles, this problem scales with the number of vertical boundaries, e.g. for a picture split into N vertical strips, there are N-1 saves/reloads per LCU row.  This could be constrained but it appears unclear how this affects potential usage modes and would be in direct opposition to encoders trying to take advantage of small tile widths which was one of the claimed benefits of tiles.  A recommended starting point is to constrain the maximum number of tile columns to investigate whether a maximum of two columns is sufficient as this has an implementation cost similar to wavefront schemes as described above.

## Tiles and slices

Tiles and slices are currently defined to be completely orthogonal to one another.  It has been suggested that this is because tiles is targeted towards enabling parallelism while slices is targeted towards enabling error resilience.  However, they will be related to one another for some environments, e.g., corruption of one slice may now not be limited to that slice in a picture when dependent tiles are used.

In addition, decoder processing is typically designed to operate in a hierarchical fashion with resources allocated to blocks and then subdivided accordingly.  The orthogonality of tiles breaks this paradigm and results in inconsistent and sporadic processing affecting efficiency.

It is recommended to restrict slices such that they do not cross tile boundaries.  More specifically, every tile boundary shall align with a slice boundary.  Every slice boundary does not have to be aligned with a tile boundary allowing multiple complete slices to be within a tile.

## Combining tiles and wavefronts

Combining wavefronts and tiles was proposed in [5] with reported improvements of 0.3% compared to tiles for two threads per tile and 0.6% better than tiles for four threads per tile.  There is already a significant amount of complexity with supporting each of these concepts by themselves and the combination of these two techniques presents a very difficult problem for implementers to support all the different possible permutations.  It is also unclear whether significant additional functionality beyond what is supportable with each of the tools themselves is provided by combining them.  Therefore, it is recommended that if both the tiles and wavefront technologies are adopted, that the HEVC standard explicitly prevent both of them to be used at the same time.

# Summary

This submission discusses several aspects related to supporting tiles in decoders to gain a better understanding of whether and how tiles would fit within a particular Profile/Level definition where every decoder conformant to that Profile/Level is required to support decoding of a stream encoded using tiles.  It is recommended that the following steps be taken or further studied before tiles is considered for inclusion in a general profile:

- Study whether deblocking is sufficient to eliminate artifacts without additional SAO and ALF processing along tile boundaries

- Prohibit dependent tiles

- Constrain the maximum number of tile columns to two

- Restrict slices such that they do not cross tile boundaries.  More specifically, every tile boundary shall align with a slice boundary.

- Prohibit the combination of tiles and wavefronts

These recommendations are suggested so that decoder architectures are not unnecessarily burdened as it is claimed single-core hardware designs which represent a significant population for many applications may have difficulty supporting all the different aspects in the current definition of tiles.  It is noted that there are also many application spaces which do not expect to utilize tiles at all.

# References

[1] A. Fuldseth, M. Horowitz, S. Xu, A. Segall, M. Zhou, “Tiles,” JCT-VC Document JCTVC-F335, Torino, IT, July, 2011.

[2]  C.-Y. Tsai, C.-W. Hsu, C.-Y. Chen, C.-M. Fu, Y.-W. Huang, S. Lei, A. Fuldseth, “AHG4: Non-cross-tiles loop filtering for independent tiles”, JCT-VC Document JCTVC-G194, Geneva, CH, November 2011

[3] W. Wan, T. Hellman, “Implication of parallelized bitstreams on single core decoder architectures”, JCT-VC Document JCTVC-G110, Geneva, CH, November 2011

[4] C.-W. Hsu, C.-Y. Tsai, Y.-W. Huang, S. Lei, “AHG4: Low latency CABAC initialization for dependent tiles”, JCT-VC Document JCTVC-G197, Geneva, CH, November 2011

[5] C.-W. Hsu, C.-Y. Tsai, Y.-W. Huang, S. Lei, “Wavefront tile parallel processing”, JCT-VC Document JCTVC-G199, Geneva, CH, November 2011

# Patent rights declaration(s)

Broadcom Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).