| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0465 |
| --- | --- |

| Title: | Simplification of mode dependent intra smoothing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li (Santa Clara University), Lingzhi Liu (Hisilicon Technologies Co. Ltd.), Nam Ling (Santa Clara University), Changcai Lai (Hisilicon Technologies Co. Ltd.), Jianhua Zheng (Hisilicon Technologies Co. Ltd.), Philipp Zhang (Hisilicon Technologies Co. Ltd.), Li Song (Santa Clara University) | Tel: Email: | +1-408-554-4794 g1li@scu.edu lingzhi.liu@huawei.com nling@scu.edu laichangcai@huawei.com zhengjianhua@huawei.com pzhang@huawei.com lsong@scu.edu |
| Source: | Santa Clara University & Hisilicon Technologies Co. Ltd.s |  |  |

_____________________________

# Abstract

A method of simplifying mode dependent intra smoothing (MDIS) is proposed. The simplification of MDIS removes intra smoothing filtering for prediction units of size 8x8 pixels. It is reported that disabling intra smoothing on 8x8 prediction units except planar mode has 0.03% and 0.05% coding loss for HE and LC cases, respectively; disabling intra smoothing for all intra modes on 8x8 prediction units has 0.04% and 0.07% coding loss for HE and LC cases.

# Introduction

MDIS (Mode Dependent Intra Smoothing) is used in current design of HEVC Test Model (HM). In the current HM-5.0 software, a 3-tap filter with coefficient [1 2 1] is used for intra smoothing according to the current block size and intra prediction mode. The filtering on/off switch is obtained from a lookup table as shown in Table-1. In the table, 0 means filtering off, and 1 means filtering on, for the corresponding block size and intra mode. [1]

{0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0}, //4x4

{1, 0, 0, 0, 1, 0, 0, 1, 0, 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0}, //8x8

{1, 0, 0, 0, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 0, 0, 1, 1, 1, 1, 1, 1, 0, 0, 1, 1, 1, 0}, //16x16

{1, 0, 0, 0, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 0}, //32x32

{0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0}, //64x64

Table-1 MDIS lookup table of HM-5.0

# Simplification of Intra LM prediction

The proposed method modifies the MDIS lookup table to remove filtering for certain cases to reduce computational complexity.

{0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0}, //4x4

{1/0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0}, //8x8

{1, 0, 0, 0, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 0, 0, 1, 1, 1, 1, 1, 1, 0, 0, 1, 1, 1, 0}, //16x16

{1, 0, 0, 0, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 0}, //32x32

{0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0}, //64x64

Table-2 Proposed modification to the MDIS lookup table

Two approaches are proposed and tested:

- Disabling intra smoothing on 8x8 units for all intra modes except intra planar mode;

- Completely disabling intra smoothing on 8x8 units.

The two approaches are jointly shown in Table-2. The only difference is the first entry in the row of 8x8 units. Filtering is on for mode 0 in case (1), and off in case (2). All other entries are 0 for size 8x8.

# Simulation results

Simulations are conducted following the common condition [2]. The results of the simplification methods are shown in the following tables.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.05% | 0.00% | 0.03% | 0.09% | 0.01% | -0.01% |
| Class B | 0.04% | 0.00% | 0.00% | 0.07% | 0.00% | -0.01% |
| Class C | -0.01% | 0.00% | 0.03% | 0.00% | -0.01% | -0.01% |
| Class D | 0.02% | 0.08% | 0.04% | 0.04% | 0.01% | -0.03% |
| Class E | 0.05% | 0.03% | 0.07% | 0.09% | -0.02% | 0.00% |
| Overall | 0.03% | 0.02% | 0.03% | 0.05% | 0.00% | -0.01% |
|  | 0.03% | 0.02% | 0.04% | 0.05% | 0.00% | -0.01% |
| Class F | -0.05% | 0.07% | -0.01% | -0.03% | -0.02% | -0.02% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 93% |  |  |

Table-3 Removing intra smoothing filtering for 8x8 intra predictions except planar mode

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.06% | -0.02% | -0.01% | 0.11% | -0.04% | -0.05% |
| Class B | 0.06% | 0.00% | -0.02% | 0.10% | -0.02% | -0.04% |
| Class C | 0.00% | 0.01% | 0.04% | 0.03% | -0.01% | -0.02% |
| Class D | 0.03% | 0.04% | 0.02% | 0.05% | 0.00% | -0.01% |
| Class E | 0.03% | 0.00% | -0.02% | 0.08% | -0.06% | -0.02% |
| Overall | 0.04% | 0.01% | 0.00% | 0.07% | -0.02% | -0.03% |
|  | 0.03% | 0.00% | 0.01% | 0.07% | -0.02% | -0.03% |
| Class F | -0.03% | -0.03% | -0.01% | -0.01% | -0.02% | -0.04% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 97% |  |  |

Table-4 Removing intra smoothing filtering for 8x8 intra predictions except planar mode

# Conclusion

As shown in the simulation results, the proposed simplification of MDIS has negligible impact on BD-rate while reducing computational complexity. It is proposed to consider adoption of removing intra smoothing for all modes on 8x8 PUs.

# Reference

[1] Benjamin Bross, et. al, “WD5: Working Draft 5 of High-Efficiency Video Coding”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1103

[2] Frank Bossen, “Common HM test conditions and software reference configurations”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G1200

# Patent rights declaration(s)

Santa Clara University and Hisilicon Technologies Co. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).