| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0466 |
| --- | --- |

| Title: | AHG21: Unified reference list for single-directional prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Viktor Wahadaniah ChongSoon Lim Block 1022 Tai Seng Avenue #06-3530 Singapore 534415 | Tel: Email: | (65) 6550 5415 viktor.wahadaniah@sg.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution proposes a number of modifications on the construction of reference picture list for single-directional prediction in P and B pictures. It is purported that the benefits of the proposed design include unification, reduced complexity and improved flexibility of the reference picture list construction and modification processes. It is suggested that the JCT-VC consider the aspects proposed in this contribution and further studies them to improve the current design of reference picture lists.

# Introduction

HEVC working draft 5 (WD5) specifies a decoded picture buffer (DPB) management scheme using reference picture sets. In this scheme, DPB management process is performed after the decoding of a slice header but prior to the decoding process for reference picture list construction and prior to the decoding of any coding unit of the slice. An active reference picture set (RPS) is signalled in the slice header either explicitly or by reference to a predefined reference picture sets signalled in an active picture parameter set (PPS).

An RPS contains a list of delta picture order count (POC) numbers identifying all reference pictures present in the DPB and usage flags indicating whether or not each reference picture is used as reference for inter prediction of the current picture/slice.

B slices utilize either single-directional or bi-directional prediction from its reference pictures. The single-predictive reference picture list RefPicListC in a B slice is created by combining the bi-predictive reference picture lists RefPicList0 and RefPicList1, either by the implicit scheme which interleaves pictures from RefPicList0 and RefPicList1 and removes any repeated pictures, or by the explicit scheme which selects and orders reference pictures from RefPicList0 and RefPicList1 in RefPicListC based on explicit signals in the video bitstream. The combination process is performed using the effective RefPicList0 and RefPicList1, i.e. the final RefPicList0 and RefPicList1 configuration after the reference picture list reordering process has been completed.

# Problem Statement

A number of issues are identified in the current design of single-predictive reference picture list as follows.

- In AVC, the initial RefPicList0 for P slices is ordered using picture/frame numbers (representing coding order), while the initial RefPicList0 for B slices is ordered using POC. In HEVC WD5, frame numbers are omitted and the initial RefPicList0 for P and B slices are both ordered in the same way based on POC. However, single-directional predictions in P slices use RefPicList0, while single-directional predictions in B slices use RefPicListC. As the same information (i.e. POC) is used for initial ordering of single-predictive reference picture lists in P and B slices, it seems conceptually cleaner to use a unified initial ordering scheme for both slice types.

- Two reference picture list combination processes (i.e. implicit and explicit) are introduced for constructing RefPicListC. These special processes can be considered as alternatives to the initial reference picture list construction process and reference picture list reordering process used for RefPicList0 and RefPicList1.

- RefPicListC is created from final configurations of RefPicList0 and RefPicList1. Consequently, single-directional and bi-directional predictions in B slices cannot use different subgroups of reference pictures.

# Proposal

The following modifications are proposed over the current design of single-predictive reference picture lists in P and B slices.

Alternative 1: Modify reference pictures lists for P slices only

In P slices, reference pictures in RefPicList0 are ordered by interleaving pictures from PocStCurr0 and PocStCurr1, followed by PocLtCurr. This ordering is similar to combined list in B slices.

Alternative 2: Modify reference pictures lists for P and B slices

- The initial RefPicListS (i.e. single-predictive reference picture list) is constructed from three intermediate lists of active/used reference pictures PocStCurr0, PocStCurr1, PocLtCurr, similar to RefPicList0 and RefPicList1. Reference pictures in RefPicListS are ordered by interleaving pictures from PocStCurr0 and PocStCurr1, followed by PocLtCurr.

- To simplify the current design, it is proposed to use the same reference picture list reordering process for RefPicList0, RefPicList1 and RefPicListS. In addition, num_ref_idx_ls_active_minus1 is introduced with similar semantics as num_ref_idx_l0_active_minus1 and num_ref_idx_l1_active_minus1.

Benefits of the proposed solutions include:

- (Alternatives 1 & 2) Unification of single-predictive reference picture lists in P and B pictures. Low-cost implementations of HEVC may choose to use random access GOP structure consisting of only P pictures. In such implementations, it can be expected that the modified single-predictive reference picture list provides improved coding efficiency over the current design.

- (Alternative 2) Improved flexibility and independence between single-predictive and bi-predictive reference picture lists in B pictures. These lists may be configured to contain different groups of reference pictures. In some use-cases, such flexibility may provide improved coding efficiency.

- (Alternative 2) Complexity reduction in both WD text and HEVC implementations as a result of closely similar schemes for constructing and reordering RefPicList0, RefPicList1 and RefPicListS. Some implementations may construct the three reference picture lists in parallel. Conceptual accuracy of single-prediction and bi-prediction can be further clarified by modifying the description of the derivation process for motion vector components and reference indices (WD5 subclause 8.4.2.1)

Both proposed solutions do not bring about any changes to slice data bitstream under the common test conditions. Between these solutions, alternative 1 is simpler and requires minimal change to working draft text, while alternative 2 provides a more comprehensive solution to the design issues highlighted above. The proponents recommends that alternative 1 be considered for adoption in the current meeting and alternative 2 be studied further to include clean-up/clarification changes to WD5 subclause 8.4.2.1 (derivation process for motion vector components and reference indices). The proponents note that clean-up editorial changes (preferably without any bitstream changes) seems desirable to clarify aspects of single-prediction vs. bi-prediction in the current WD5 text such as prediction utilization flags and reference lists/indexes.

# Conclusion

This contribution proposes a number of modifications on the construction of reference picture list for single-directional prediction in P and B pictures. It is purported that the benefits of the proposed design include unification, reduced complexity and improved flexibility of the reference picture list construction and modification processes. It is suggested that the JCT-VC consider the aspects proposed in this contribution and further studies them to improve the current design of reference picture lists.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “WD5: Working Draft 5 of High-Efficiency Video Coding,” JCTVC-G1103, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 7th meeting, Geneva, CH, November 2011.

- D. Flynn, R. Sjöberg, Y. Chen, T. K. Tan, W. Wan, Y.-K. Wang, “JCT-VC AHG report: Reference picture buffering and list construction (AHG21),” JCTVC-G021, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 7th meeting, Geneva, CH, November 2011.

- J. Boyce, R. Sjoberg, Y. K. Wang, “BoG report: Reference picture buffering and list construction,” JCTVC-G1002, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 7th meeting, Geneva, CH, November 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix: Proposed changes for HEVC WD5 (d7)

Proposed working draft text is included below using the following text conventions:

- Unchanged text that is not shown here is indicated with "…"

- Removed text is shown with strikethrough.

- Newly added text is shown in yellow highlight.

- Proponent’s comment is shown in blue italics.

# Alternative 1

…

- 8.2.3.1	Initialisation process for reference picture lists

…

The following procedure is conducted to construct the initial RefPicList0 in P slices:

cIdx = 0
while( cIdx <= num_ref_idx_ls_active_minus1 )
{
	maxNumPocStCurr = Max( NumPocStCurr0, NumPocStCurr1 )
	for( i=0, j=0, k=0; i < maxNumPocStCurr && cIdx <= num_ref_idx_ls_active_minus1; i++ )
	{
		if( j < NumPocStCurr0 )
		{
			RefPicListS[ cIdx ] = RefPicSetStCurr0[ j ]
			cIdx++
			j++
		}
		if( k < NumPocStCurr1 )
		{
			RefPicListS[ cIdx ] = RefPicSetStCurr1[ k ] 	(8-x)
			cIdx++
			k++
		}
	}
	for( i=0; i < NumPocLtCurr && cIdx <= num_ref_idx_l0_active_minus1; cIdx++, i++ )
		RefPicListS[ cIdx ] = RefPicSetLtCurr[ i ]
}

The following procedure is conducted to construct the initial RefPicList0 in B slices:

The following procedure is conducted to construct the initial RefPicList0:

…

# Alternative 2

…

- 7.3.2.2	Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| num_ref_idx_ls_default_active_minus1 | ue(v) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| … |  |
| } |  |

…

- 7.3.3	Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !lightweight_slice_flag ) { |  |
| … |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0ls_active_minus1 | ue(v) |
| if( slice_type = = B ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| } |  |
| … |  |
| } |  |

…

- 7.3.3.2	Reference picture list modification syntax

| ref_pic_list_modification( ) { | Descriptor |
| --- | --- |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| ref_pic_list_modification_flag_ls | u(1) |
| if( ref_pic_list_modification_flag_ls ) |  |
| do { |  |
| ref_pic_list_modification_idc | ue(v) |
| if( ref_pic_list_modification_idc != 3 ) |  |
| ref_pic_set_idx | ue(v) |
| } while( ref_pic_list_modification_idc != 3 ) |  |
| } |  |
| if( slice_type = = B ) { |  |
| ref_pic_list_modification_flag_l0 | u(1) |
| if( ref_pic_list_modification_flag_l0 ) |  |
| do { |  |
| ref_pic_list_modification_idc | ue(v) |
| if( ref_pic_list_modification_idc != 3 ) |  |
| ref_pic_set_idx | ue(v) |
| } while( ref_pic_list_modification_idc != 3 ) |  |
| ref_pic_list_modification_flag_l1 | u(1) |
| if( ref_pic_list_modification_flag_l1 ) |  |
| do { |  |
| ref_pic_list_modification_idc | ue(v) |
| if( ref_pic_list_modification_idc != 3 ) |  |
| ref_pic_set_idx | ue(v) |
| } while( ref_pic_list_modification_idc != 3 ) |  |
| } |  |
| } |  |

…

- 7.3.3.3	Reference picture list combination syntax

| ref_pic_list_combination( ) { | Descriptor |
| --- | --- |
| if( slice_type = = B ) { |  |
| ref_pic_list_combination_flag | u(1) |
| if( ref_pic_list_combination_flag ) { |  |
| num_ref_idx lc_active_minus1 | ue(v) |
| ref_pic_list_modification_flag_lc | u(1) |
| if( ref_pic_list_modification_flag_lc ) |  |
| for ( i =0; i <= num_ref_idx_lc_active_minus1; i++ ) { |  |
| pic_from_list_0_flag | u(1) |
| ref_idx_list_curr | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |

…

- 7.4.2.2	Picture parameter set RBSP semantics

…

num_ref_idx_ls_default_active_minus1 specifies how num_ref_idx_ls_active_minus1 is inferred for P and B slices with num_ref_idx_active_override_flag equal to 0. The value of num_ref_idx_ls_default_active_minus1 shall be in the range of 0 to 31, inclusive.

…

- 7.4.3	Slice header semantics

…

num_ref_idx_ls_active_minus1 specifies the maximum reference index for reference picture list S that shall be used to decode the slice. When the current slice is a P or B slice and num_ref_idx_ls_active_minus1 is not present, num_ref_idx_ls_active_minus1 shall be inferred to be equal to num_ref_idx_ls_default_active_minus1.The value of num_ref_idx_ls_default_active_minus1 shall be in the range of 0 to 31, inclusive.

…

- 7.4.3.2	Reference picture list modification semantics

…

ref_pic_list_modification_flag_ls equal to 1 specifies that the syntax element ref_pic_list_modification_idc is present for specifying reference picture list S. ref_pic_list_modification_flag_ls equal to 0 specifies that this syntax element is not present.

When ref_pic_list_modification_flag_ls is equal to 1, the number of times that ref_pic_list_modification_idc is not equal to 3 following ref_pic_list_modification_flag_ls shall not exceed num_ref_idx_ls_active_minus1 + 1.

…

- 7.4.3.3	Reference picture list combination semantics

ref_pic_list_combination_flag equal to 1 indicates that the reference picture list 0 and the reference picture list 1 are combined to be an additional reference picture lists combination used for the prediction units being uni-directional predicted. This flag equal to 0 indicates that the reference picture list 0 and reference picture list 1 are identical thus reference picture list 0 is used as the reference picture lists combination. The reference picture lists combination is set to be empty at the start of the loop defined in this table.

num_ref_idx lc_active_minus1+1 specifies the number of reference pictures selected from reference picture list 0 or reference picture list 1 in the reference picture lists combination.

ref_pic_list_modification_flag_lc equal to 1 specifies that the syntax elements pic_from_list_0_flag and ref_idx_list_curr are present for specifying the mapping for the entries of the reference picture lists combination to the entries of reference picture list 0 and reference picture list 1. ref_pic_list_modification_flag_lc equal to 0 specifies that these syntax elements are not present. The reference picture lists combination is initialized as specified in subclause X.X.X.X.

pic_from_list_0_flag indicates the current reference picture added into the reference picture lists combination is from reference picture list 0 or reference picture list 1. When this flag is equal to 1, the picture is from the reference picture list 0, and the CurrRefPicList is reference picture list 0; when this flag is equal to 0, the picture is from the reference picture list 1, and the CurrRefPicList is reference picture list 1.

ref_idx_list_curr indicates the reference index of the picture in the reference picture list specified by pic_from_list_0_flag to be appended at the end of the reference picture lists combination.

…

- 8.2.3	Decoding process for reference picture lists construction

…

Reference pictures are addressed through reference indices as specified in subclause X.X.X.X. A reference index is an index into a reference picture list. When decoding a P or SP slice, there is a single reference picture list RefPicList0S. When decoding a B slice, there is a secondare independent reference picture lists RefPicList0 and RefPicList1 in addition to RefPicList0S.

At the beginning of the decoding process for each slice, reference picture list RefPicList0S, and for B slices RefPicList0 and RefPicList1, are derived as specified by the following ordered steps:

- An initial reference picture list RefPicList0S and for B slices RefPicList0 and RefPicList1 are derived as specified in subclause Error: Reference source not found8.2.3.1.

- When ref_pic_list_modification_flag_l0s is equal to 1 or, when decoding a B slice, ref_pic_list_modification_flag_l0 is equal to 1 or ref_pic_list_modification_flag_l1 is equal to 1, the initial reference picture list RefPicList0S and, for B slices, RefPicList0 and RefPicList1 are modified as specified in subclause Error: Reference source not found8.2.3.2.

NOTE – The modification process for reference picture lists specified in subclause Error: Reference source not found8.2.3.2 allows the contents of RefPicList0S and for B slices RefPicList0 and RefPicList1 to be modified in a flexible fashion. In particular, it is possible for a picture that is currently marked "used for reference" to be inserted into RefPicList0S and for B slices RefPicList0 and RefPicList1 even when the picture is not in the initial reference picture list derived as specified in subclause Error: Reference source not found8.2.3.1.

The number of entries in the modified reference picture list RefPicList0S is num_ref_idx_l0s_active_minus1 + 1, and for B slices the number of entries in the modified reference picture lists RefPicList0 and RefPicList1 isare num_ref_idx_l0_active_minus1 + 1 and num_ref_idx_l1_active_minus1 + 1, respectively. A reference picture may appear at more than one index in the modified reference picture lists RefPicListS, RefPicList0 or RefPicList1.

- 8.2.3.1	Initialisation process for reference picture lists

…

The following procedure is conducted to construct the initial RefPicListS:

cIdx = 0
while( cIdx <= num_ref_idx_ls_active_minus1 )
{
	maxNumPocStCurr = Max( NumPocStCurr0, NumPocStCurr1 )
	for( i=0, j=0, k=0; i < maxNumPocStCurr && cIdx <= num_ref_idx_ls_active_minus1; i++ )
	{
		if( j < NumPocStCurr0 )
		{
			RefPicListS[ cIdx ] = RefPicSetStCurr0[ j ]
			cIdx++
			j++
		}
		if( k < NumPocStCurr1 )
		{
			RefPicListS[ cIdx ] = RefPicSetStCurr1[ k ] 	(8-x)
			cIdx++
			k++
		}
	}
	for( i=0; i < NumPocLtCurr && cIdx <= num_ref_idx_l0_active_minus1; cIdx++, i++ )
		RefPicListS[ cIdx ] = RefPicSetLtCurr[ i ]
}

…

- 8.2.3.2	Modification process for reference picture lists

After the invocation of this process, there shall be no reference pictures with greater temporal_id than the current slice included in the output RefPicListS, RefPicList0 or RefPicList1.

When ref_pic_list_modification_flag_ls is equal to 1, the following applies:

- Let refIdxLS be an index into the reference picture list RefPicListS. It is initially set equal to 0.

- The corresponding syntax elements modification_of_pic_nums_idc are processed in the order they occur in the bitstream. For each of these syntax elements, the following applies.

–	If modification_of_pic_nums_idc is equal to 0 or equal to 1, the process specified in subclause Error: Reference source not found is invoked with refIdxLS as input, and the output is assigned to refIdxLS.

–	Otherwise, if modification_of_pic_nums_idc is equal to 2, the process specified in subclause Error: Reference source not found is invoked with refIdxLS as input, and the output is assigned to refIdxLS.

–	Otherwise (modification_of_pic_nums_idc is equal to 3), the modification process for reference picture list RefPicListS is finished.

…

- 8.2.3.2.1	Modification process of reference picture lists for short-term reference pictures

- Input to this process is an index refIdxLX (with X being S, 0 or 1).

…

- 8.2.3.3	Mapping process for reference picture lists combination in B slices

[Ed.: (WJ) needs to be checked once again. Try to find better way to represent]

This initialisation process is invoked when decoding a B slice header.

Input to this process are the reference picture list RefPicListX and num_ref_idx_lX_active_minus1 with X being 0 or 1.

Outputs of this process are arrays PredLCToPredLx and RefIdxLCToRefIdxLx.

When the current slice is a B slice and ref_pic_list_modification_flag_lc is equal to 0, the following ordered steps apply:

      - Let refIdxL0 and refIdxL1 be indices into the reference picture lists RefPicListL0 and RefPicListL1. They are initially set equal to 0.

      - Let refIdxLC be an index into PredLCToPredLx and RefIdxLCToRefIdxLx. It is initially set equal to 0.

      - The following process is repeated until refIdxL0 and refIdxL1 are both greater than num_ref_idx_l0_active_minus1 and num_ref_idx_l1_active_minus1, respectively:

–	If refIdxL0 is less than or equal to num_ref_idx_l0_active_minus1,

–	If the entry RefPicListL0[ refIdxL0 ] is the first occurance of the reference picture,

PredLCToPredLx[ refIdxLC ] = Pred_L0, 	(8-1)
			RefIdxLCToRefIdxLx[ refIdxLC++ ] = refIdxL0.

–	refIdxL0++.

–	If refIdxL1 is less than or equal to num_ref_idx_l1_active_minus1 and ref_pic_list_combination_flag equal to 1,

–	If the entry RefPicListL1[ refIdxL1 ] is the first occurance of the reference picture,

PredLCToPredLx[ refIdxLC ] = Pred_L1, 	(8-2)			RefIdxLCToRefIdxLx[ refIdxLC++ ] = refIdxL1.

–	refIdxL1++.

When the current slice is a B slice and ref_pic_list_modification_flag_lc is equal to 1, the following ordered steps apply:

- Let refIdxLC be an index into the reference picture list PredLCToPredLx and RefIdxLCToRefIdxLx. It is initially set equal to 0.

- The corresponding syntax elements pic_from_list_0_flag and ref_idx_list_curr are processed in the order they occur in the bitstream. For each of these syntax elements pairs, the following applies.

–	If pic_from_list_0_flag is equal to 1,

PredLCToPredLx[ refIdxLC ] = Pred_L0, 	(8-3)

–	Otherwise,

PredLCToPredLx[ refIdxLC ] = Pred_L1	(8-4)

–	RefIdxLCToRefIdxLx[ refIdxLC++ ] = ref_idx_list_curr

When refIdxLC is greater than num_com_ref_list_active_minus1+ 1, the extra entries past position num_com_ref_list_active_minus1 are discarded from PredLCToPredLx and RefIdxLCToRefIdxLx.

When refIdxLC is less than num_com_ref_list_active_minus1 + 1, the remaining entries in PredLCToPredLx and RefIdxLCToRefIdxLx are set equal to Pred_L0 and 0, respectively.

The following process creates a mapping from RefPicListS to RefPicList0 and RefPicList1. This process is a temporary example to link the proposed aspects with the current WD text and should be removed after further study has been done to modify the derivation process for motion vector components and reference indices (WD5 subclause 8.4.2.1).

- 8.2.3.3	Mapping process for reference picture lists

Input to this process are the reference picture lists RefPicListS, RefPicList0 and RefPicList1.

Outputs of this process are arrays PredLSToPredLx and RefIdxLSToRefIdxLx, and the modified reference picture list RefPicList0.

When the current slice is a P slice, the reference list RefPicListS and the variable num_ref_idx_ls_active_minus1 are copied to the reference list RefPicList0 and the variable num_ref_idx_l0_active_minus1, respectively.

When the current slice is a B slice, the following ordered steps are repeated with refIdxLS initially set equal to 0 until refIdxLS is greater than num_ref_idx_ls_active_minus1.

–	When the reference entry RefPicListS[ refIdxLS ] is the first occurrence of a reference picture in RefPicListS, the reference picture entry is searched within RefPicList0 and RefPicList1.

–	If the reference picture is found, the following applies:

–	If the reference picture is present in both RefPicList0 and RefPicList1 at reference indexes refIdxL0 and refIdxL1, respectively,

PredLSToPredLx[ refIdxLS ] = ( refIdxL1 < refIdxL0 ? Pred_L1 : Pred_L0 )	(8-x)
RefIdxLSToRefIdxLx[ refIdxLS ] = ( refIdxL1 < refIdxL0 ? refIdxL1 : refIdxL0 )

–	Otherwise, if the reference picture is present only in RefPicList0 at reference index refIdxL0,

PredLSToPredLx[ refIdxLS ] = Pred_L0	(8-x)
RefIdxLSToRefIdxLx[ refIdxLS ] = refIdxL0

–	Otherwise (the reference picture is present only in RefPicList1 at reference index refIdxL1),

PredLSToPredLx[ refIdxLS ] = Pred_L1	(8-x)
RefIdxLSToRefIdxLx[ refIdxLS ] = refIdxL1

–	Otherwise (the entry is not found), the reference picture is appended into RefPicList0 as follows:

num_ref_idx_l0_active_minus1++
RefPicList0[ num_ref_idx_l0_active_minus1 ] = RefPicListS[ refIdxS ]
PredLSToPredLx[ refIdxLS ] = Pred_L0	(8-x)
RefIdxLSToRefIdxLx[ refIdxLS ] = num_ref_idx_l0_active_minus1

–	The variable refIdxLS is incremented by one.