| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0469 |
| --- | --- |

| Title: | Cross-check of NEC's proposal H0051 on burst transmission of I_PCM blocks by Panasonic |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Sue Mon Thet Naing Viktor Wahadaniah ChongSoon Lim Block 1022 Tai Seng Avenue #06-3530 Singapore 534415 | Tel: Email: | (65) 6550 5415 viktor.wahadaniah@sg.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This document presents a cross-check report on NEC’s proposal on burst transmission of I_PCM blocks (JCTVC-H0051). Through inspection of the source code, the cross-checker corroborated that the software changes match the algorithm description. Simulation results from the cross-checker verified that the software changes do not affect the BD-results under the HEVC common test conditions, in which PCM mode is not selected by HM-5.0. Additional simulation results using Sandstorm sequence verified that the proposed technique reduces the number of bits required for signalling consecutive I_PCM blocks. Considering the usefulness of the proposed technique especially for hardware implementations, the cross-checker recommends that JCT-VC adopt this proposal.

# Cross-check activities

This document presents a cross-check report on NEC's proposal on burst transmission of I_PCM blocks (JCTVC-H0051) by Panasonic. Algorithm description, software implementation and reference results based on HM-5.0 was received from NEC.

The source code was inspected and it was corroborated that the software implementation matches the intended operation according to the algorithm description. It was also noted that the proposal complements the recent adoption of MaxIPCMCUSize (JCTVC-G112) intended for reducing the size of hardware decoder implementations. The proposal minimizes the signalling redundancy that may arise due to the introduction of MaxIPCMCUSize.

Simulations were run on the computing platform of Intel Core i7 980 @ 3.33GHz, 12GB RAM using Linux Ubuntu 10.10 operating system, using the common test conditions as described in JCTVC-G1200. The anchor simulation is run using HM-5.0 revision 1712 while the proposal simulation is run using HM-5.0 revision 1712 with NEC's code changes.

# Experimental results

Experimental results are shown in Tables 1, 2 and 3 below. It was observed that the results obtained by Panasonic match with the reference results provided by NEC. Table 1 shows that the proposed technique does not affect common test conditions, in which I_PCM blocks are not present. Table 2 shows that the proposed technique reduces the number of bits required for signalling consecutive I_PCM blocks. Table 3 shows that the proposed technique does not increase the number of bits required for signalling I_PCM blocks individually when intermediate storage for grouping I_PCM blocks are not available during encoding.

Table 1 – Simulation results on common test conditions

|  | All intra HE |  |  | All intra LC |  |  | All intra HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| All | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time | 83% |  |  | 80% |  |  |  |  |  |
| Dec Time | 108% |  |  | 108% |  |  |  |  |  |
|  | Random access HE |  |  | Random access LC |  |  | Random access HE-10 |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| All | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time | 78% |  |  | 78% |  |  | 77% |  |  |
| Dec Time | 100% |  |  | 100% |  |  | 113% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  | Low delay B HE-10 |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| All | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time | 76% |  |  | 75% |  |  |  |  |  |
| Dec Time | 108% |  |  | 106% |  |  |  |  |  |

Table 2 – Simulation results on Sandstorm sequence for current HEVC vs. proposed method

Table 3 – Simulation results on Sandstorm sequence for current HEVC vs. proposed method with num_subsequent_pcm forced to 0 all the time

# Conclusion

This document presents a cross-check report on NEC’s proposal on burst transmission of I_PCM blocks (JCTVC-H0051). Through inspection of the source code, the cross-checker corroborated that the software changes match the algorithm description. Simulation results from the cross-checker verified that the software changes do not affect the BD-results under the HEVC common test conditions, in which PCM mode is not selected by HM-5.0. Additional simulation results using Sandstorm sequence verified that the proposed technique reduces the number of bits required for signalling consecutive I_PCM blocks. Considering the usefulness of the proposed technique especially for hardware implementations, the cross-checker recommends that JCT-VC adopt this proposal.

# References

- K. Chono, H. Aoki, “Burst transmission of I_PCM blocks,” JCTVC-H0051, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting, San Jose, USA, February 2012.

- F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-G1200, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting, Geneva, CH, November 2011.