| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0471 |
| --- | --- |

| Title: | Proposal to Improve Ultra Low Delay Coding with Minimal Decoder Impact |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Osnat Bar-Nir 10 Ha’amal St. Rosh Ha’ayin, 48092, Israel | Tel: Email: | +972 3 900 7785 Osnat.Bar-Nir@Harmonicinc.com |
| Source: | Harmonic Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a simple approach for improving ultra low delay coding in HEVC standard. Intra coded pictures in HEVC are significantly larger than inter coded pictures, hence they require significantly longer transmission delay. The extra delay increases with the ratio between the sizes of intra and inter pictures. Ultra low delay coding requires uniform bit distribution between pictures, and preferably also within the picture. A way to achieve more uniform distribution of bits between pictures is to replace the I-pictures with a progressive refresh mechanism, using rows or columns of Intra CUs.

The main hurdle in implementing ultra low delay in HEVC is the various in-loop filters that contaminate refreshed pixels that are filtered with non-refreshed pixels.

Past ultra low delay related proposals such as JCTVC-G183 and others proposed similar approach to low delay coding, but were rejected due to added burden on the decoder (JCTVC-E600). This proposal suggests less optimal solution, but with a minimal impact on the decoder.

It is proposed to provide a means for the encoder to inform the location of the refresh line to the decoder, in the PPS or in the slice header, or at least by using a dedicated SEI message and a VUI flag. The decoder would use this information to disable deblocking, SAO and ALF across the refresh line.

# Background

Ultra low delay coding was acknowledged as a "reasonable requirement" [7] for HEVC.

In a standard like HEVC with a large variation in picture size, the way to achieve a semi-uniform picture size is to use a row/column intra-refresh mechanism instead of using huge I-pictures witch increase the delay due to buffering.

Intra refresh replaces the Intra picture with an Intra row or column that progressively refresh the picture from left to right or from top to bottom, across multiple pictures.

Refresh in columns is preferable over row refresh; easier to get accurate rate control (which is critical due to minimal buffer size), it enables lower percentage of Intra blocks in the picture (due to picture dimensions), and it provides more uniform bit distribution within the picture [3].

Using progressive Intra-refresh is much less efficient than using I-pictures, since it imposes many constraints on prediction from the "non-refreshed" side (right) to the refreshed side (left).

Despite the loss of coding efficiency, this is the only method that can guaranty low-delay at a reasonable quality.

Figure 1: Column Intra Refresh

The goal of this proposal is to achieve an optimal balance between coding efficiency and decoder burden.

# The Proposal

In principle, column intra-refresh can be implemented by the encoder, without any normative changes to the decoder (as long as the decoder doesn't wait for an I-picture to start).

The encoder should place Intra coded columns at the appropriate locations, and make sure that the refreshed (left) side does not use any information from the non-refreshed (right side). This includes intra-prediction, inter-prediction, interpolation filters and in-loop filters. Each constraint is paid for in coding efficiency.

A limited set of applicable Intra-prediction directions at the refresh border, and restricted motion vectors are inherent to this coding scheme. Filters are not.

Normatively disabling all filters between refreshed and non-refresh pixels would significantly improve the coding efficiency. On the other hand it would increase complexity on the decoders.

In JCTVC-G183 [1] it was proposed to utilize tiles in order to avoid contamination of the refreshed area by using pixels from the non-refreshed area. The average BD-Rate reduction achieved by that approach was of 5.3%-5.6% [1] for a similar implementation that isolates the refreshed area from the non-refreshed area by tool already exists in HEVC.

Tile approach was rejected since it requested to use tiles but also keep the order of entropy coding as a raster scan order within a picture. Tiles were not the goal of that proposal; it was just a means to provide non-crossing predictions and filtering. This contribution proposes another mechanism to achieve the required isolation, with reduced decoder impact.

In both methods presented in [1], inter and intra predictions are restricted with the same restrictions proposed here. The gain is due to non-cross filtering between the tiles. This contribution proposes another means to achieve non-crossing filters.

This proposal distinguishes between interpolation filters and in-loop filters.

Implementing non-crossing interpolation filters in motion compensation imposes a significant burden on the decoder. Restricting inter-prediction by the encoder to full horizontal pixel position at the immediate columns of pixels to the left of the refresh line would reduce coding efficiency, but not significantly. Therefore it is suggested to leave it to the encoder.

In-loop filters impact coding efficiency more. In-loop filters effectively shift the refresh line some pixels to the left [1].  The number of pixels is the sum of the filtered pixels at each filtering stage (deblocking, SAO, ALF).

Figure 2: CU left to the refresh line has some non-valid pixels

(CU with non-valid data marked by a dot)

The practical impact of this shift, is that (0,0) and other small vectors are no longer applicable to the CU adjacent to the refresh line. As a result, on top of the Intra-refresh column, the percentage of Intra coded CU at the column on the left of the Intra-refreshed column, the one that was just refreshed in the previous frame, is relatively high, especially if the content has static and detailed background. CUs that otherwise would have been coded as skip are now are coded as Intra.

Looking closely at the simulation results [1] reveals a large variation in BD-Rate reduction between the different classes and within the classes.  The amount of improvement depends on two variables:

- Frame size: The smaller the frame is, the weight of each Intra coded CU is higher.

- Content: content with static and detailed background are less likely to find a good inter prediction for the CU left to the refresh line, hence suffer more. Good example for that are Class E videos, with average reduction of 8% in BD-Rate [1]. Another example is the variation within Class D, with 8.3% reduction in BlowingBubbles and only 4.6% reduction in RaceHorses [1].

It is proposed to provide a means for the encoder to inform the location of the refresh line to the decoder, in the PPS or in the slice header, or at least by using a dedicated SEI message and a VUI flag. The decoder shall use this information to disable deblocking, SAO and ALF across the refresh line.

# Conclusions

This contribution proposes to provide a means for the encoder to inform the location of the refresh line to the decoder, in the PPS or in the slice header, or at least by using a dedicated SEI message and a VUI flag. The decoder shall use this information to disable deblocking, SAO and ALF across the refresh line.

Simulation results [1] prove that adopting this change would significantly improve ultra low delay coding efficiency, while adding minimal burden on the decoder.

# References

[1] K. Kazui, J. Koyama, S.Shimada, A. Nakagawa, “Test Results of Low Delay Tile”, JCTVC-G183, November 2011

[2] K. Kazui, J. Koyama, S.Shimada, A. Nakagawa, ”Support of Very Low Delay Coding in Tiles”, JCTFC-F140, July 2011

[3] K. Kazui, J. Koyama, A. Nakagawa, "Proposal of requirement on very low delay coding", JCTVC-B031, July 2010

[4] K. Kazui, J. Koyama, A. Nakagawa, “Evaluation Results of JCTVC-B031”, JCTVC-C021, October 2010

[5] K. Kazui, J. Koyama, A. Nakagawa, " Benefit of the new syntax and semantics for very low delay coding in HEVC", JCTVC-D054, Jan 2011

[6] K. Kazui, J. Koyama, S.Shimada, A. Nakagawa,”Requierments of Very Low Delay Applications”, JCTVC-E061. March 2011

[7] Gary Sullivan, Jens-Rainer Ohm, " Meeting report of the fifth meeting of the Joint Collaborative Team on Video Coding (JCT-VC), Geneva, CH, 16-23 March 2011", JCTVC-E600, March 2011

# Patent rights declaration(s)

Harmonic Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).