The following changes to the working draft 5.0 are recommended to support this proposal, H471.

In the section 3, Definitions add the following:

Intra refresh boundary:  when present, a vertical boundary along the right border of one treeblock column within the picture where loop filtering is disabled.

In section 7.3.2.2, insert the elements intra_refresh_boundary_flag and intra_refresh_boundary_loc into the PPS:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| .... |  |
| intra_refresh_boundary_flag | u(1) |
| if( intra_refresh_boundary_flag = = 1 ) |  |
| intra_refresh_boundary_loc | ue(v) |
| …. |  |
| rbsp_trailing_bits( ) |  |
| } |  |

intra_refresh_boundary_flag equal to 1 specifies that the in-loop filtering operations are not performed across an intra refresh boundary established by intra_refresh_boundary_loc; otherwise, the intra refresh boundary is not present and therefore does not effect in-loop filtering operations. The in-loop filtering operations include deblocking filter, sample adaptive offset, and adaptive loop filter.

intra_refresh_boundary_loc specifies the location of the treeblock column in treeblock units whose right border marks the intra refresh boundary.  The value of intra_refresh_boundary_loc must be between 0 and PicWidthInLCUs -1. Intra_refresh_boundary_loc is not coded if intra_refresh_boundary_flag is not equal to 1.

In section 8.6.1, deblocking filter process, amend the paragraph:

The deblocking filter process shall be applied to all prediction unit edges and transform unit edges of a picture, except edges at the boundary of the picture, any edges for which the deblocking filter process is disabled by disable_deblocking_filter_flag, any edges coinside with tile boundaries when loop_filter_across_tile_flag is equal to 0, and any edges coinside with slice boundaries when loop_filter_across_slice_flag is equal to 0, and any edges that coinside with intra refresh boundaries when intra_refresh_boundary_flag is 1. For the transform units and prediction units with edges smaller than 8 samples in either vertical or horizontal direction, only the edges lying on the 8x8 sample grid are filtered.

In section 8.6.3.1.1. Boundary padding process for granularity block, amend the following paragraphs:

The padded sample array s’’ is derived as follows.

- If both loop_filter_across_slice_flag and loop_filter_across_tile_flag are equal to 1, and intra_refresh_boundary_flag is equal to 0, s’’[ x ][ y ] is set equal to s’[ x ][ y ] for x = (xC-nExtPixels)..(xC+nExtPixels-1) and y = (yC-nExtPixels)..(yC+nExtPixels-1).

- Otherwise (loop_filter_across_slice_flag is equal to 0 or loop_filter_across_tile_flag is equal to 0 or intra_refresh_boundary_flag is equal to 1), s’’[ x ][ y ] is derived depending on the slice and tile relationship between neighbouring regions specified in Error: Reference source not found and Error: Reference source not found where the function Rs(·) is specified for any two regions A and B as follows: [Ed. (WJ): large table does not seem to be the best way to specify the process. Traditional way (if-clause) may be better]