| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0472_r3 WG11 Number: m23349 |
| --- | --- |

| Title: | CE10.2: Level restrictions on tiles for line buffer reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Andrey Norkin, Rickard Sjöberg Färögatan 6 164 80 Stockholm Sweden | Tel: Email: | +46 10 714 84 02 andrey.norkin@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The document proposes to introduce level constraints on the maximum tile width as a solution to reduce the size of line-buffers. This reportedly decreases the amount of on-chip line memory that is required for in-loop filtering operations.

# Introduction

The line memory problem is widely recognized and has recently drawn a lot of attention in JCT-VC. There has been many input contributions in the area, particularly in CE8 (non-deblocking in-loop filtering) and CE12 (deblocking filtering). Examples of contributions are F053, F054, F055, F272, F215 etc…

The nature of the line buffer problem has to do with memory bandwidth in hardware decoder implementations. In order to decrease the number of read and write accesses to the memory, the in-loop filtering is generally done in decoding order. In the decoder, the CU is reconstructed, then the deblocking is applied for the internal block boundaries as well as the left and upper CU boundaries. This is followed by SAO and ALF and then the final in-loop filtered CU pixels are written to memory. However, deblocking filtering cannot be applied to block boundaries where neighboring blocks have not been reconstructed yet. Moreover, blocks not yet processed by deblocking filtering cannot be used as input to SAO which leads to a further increase in required line memory. Therefore, lines of pixels at the bottom LCU boundary, not yet processed by loop filters (or needed for them as input) are kept in the on-chip memory (line buffer). The use of line buffers are not required by the HEVC specification but it is expected that many implementations will use line buffers to reduce the frame memory bandwidth. The required minimum size of the luma line buffer is currently equal to 4 pixels for the low-complexity configuration after the proposal reducing the line buffer for ALF has been adopted [2]. This is the same size as the luma line buffer in AVC which constitutes 4 lines. If one considers that HEVC is applied to coding 4Kx2K video, the amount of luma line memory in the HEVC decoder would be 2 times the AVC decoder needs to decode 1080p.

# Proposed solution

It is proposed to introduce restrictions on the maximum tile width when levels are defined. As an example it could be specified for a certain level that the tile width shall not exceed a certain number of LCU blocks. If the resolution of the video frame exceeds this number, the frame shall be split in tiles so that the maximum tile width would not exceed the predefined number of LCUs. This would decrease the amount of line buffer memory that a decoder compliant to this level needs to have.

The usage of line buffer memory for tiles is shown in Figure 1. It can be seen that when decoding in the tile order, only yellow pixels need to be kept the line buffer. One observation that can be done here is that splitting picture in the “vertical” tiles can increase the size of the vertical “column” buffer that would need to hold the right-most columns of pixels in the rightmost LCUs in the tile (shown in green in Figure 1). In case of tile-order decoding, these columns cannot be processed before the neighboring LCU’s in the tile to the right have been decoded. However, the decoder does not need to hold these columns in the line buffer. Instead, these columns of pixels can be written to the frame memory and read back to the on-chip memory when needed. Since such a reading would happen only for one LCU in the row of LCU in the slice, the bandwidth would not be affected as much as in the line memory case. However, a restriction on the minimum tile width can also be introduced, so that the amount of reading from the “outer” memory that is done in the beginning of each LCU row is also restricted.

LCU

#0

LCU

#1

LCU

#6

LCU

#7

LCU

#14

Vert. tile boundary

Tile #1

Line buffer

Column buffer

(saved in the memory)

decoded LCU’s

LCU’s to be decoded

Tile #0

Figure 1. Line buffer requirements in case of two “vertical” tiles per picture. Yellow lines should be kept in the “line buffer” while green columns can be written to memory and then read again

Another observation is that when the picture has horizontal tile boundaries, storing/reading of the line buffer is done when the decoder moves to the next tile (in horizontal direction). Therefore, another restriction on the minimum tile height can be introduced as well.

## Proposed level restrictions

In Table 1, an example of minimum and maximum tile sizes is introduced.

Table 1 Example on restrictions on Tile size

| Level | MaxTileWidthInLCU | MinTileWidthInLCU | MinTileHeightInLCU |
| --- | --- | --- | --- |
| 5 and above | 30 | 3 | 3 |

Table 2 Example on restrictions on Tile size

| Level | MaxTileWidthInPixels | MinTileWidthInPixels | MinTileHeightInPixels |
| --- | --- | --- | --- |
| 5 | 1920 | 192 | 192 |

Another example on possible restrictions on tile size would be as shown in Table 3.

Table 3 Another example of restrictions on Tile size

| Level | MaxTileWidthInLCU | MinTileBorderInLCU |
| --- | --- | --- |
| 5 and above | 30 | 6 |

where MinTileBorderIn LCU is equal to MinTileHeightInLCU + MinTileWidthInLCU.

Table 4 Another example of restrictions on Tile size

| Level | MaxTileWidthInPixels | MinTileBorderInpixels |
| --- | --- | --- |
| 5 and above | 1920 | 384 |

where MinTileBorderIn LCU is equal to MinTileHeightInLCU + MinTileWidthInLCU.

Level 5 is here assumed to be the lowest HEVC level for which a picture size with a horizontal resolution exceeding 1920 pixels is allowed. No restrictions are proposed for levels below 5.

# Bandwidth analysis

The decoder memory bandwidth consists of the memory bandwidth for motion compensation and the memory bandwidth required for writing the reconstructed pixels to the memory. In case the line buffer is used, the decoder needs to write the reconstructed and processed pixels once to the memory. In this case 1 read/write operation is required for 1 pixel. However, if the decoder does not use the line buffer, the pixels on the lower LCU boundary are needed to be written then read again to perform the loop filtering and then saved back to the memory. If one considers a resolution of 1920x1080 and the low-complexity configuration with only the deblocking line buffer, then the memory bandwidth used for reconstructed luma pixels is 1.1037 memory access/pixel, which corresponds to the memory access increase to the reconstructed pixels of 10.37% compared to using the line buffer. If one add up the memory bandwidth used for motion compensation, then this figures would be much lower.

However, one should notice that when vertical tiles are used, the memory bandwidth is also increased compared to not using tiles (and using only line buffers). The following table shows results for the luma memory bandwidth increase due to the vertical tile boundary (which has to be written to the memory twice and read once).

Table 5  Luma bandwidth related to memory access to reconstructed pixels (two vertical tiles per picture)

| Resolution | Bandwidth, LC configuration | Bandwidth, HE configuration |
| --- | --- | --- |
| class A | 1.0044 | 1.0125 |
| class B | 1.0065 | 1.0185 |
| class C | 1.0146 | 1.0417 |
| class D | 1.0292 | 1.0833 |
| class E | 1.0097 | 1.0278 |

However, it is not proposed to impose restrictions for all resolutions. Since imposing tile width restrictions on lower resolutions can cause increase in the memory bandwidth, we propose to impose the tile level restrictions on resolutions exceeding class B (1920x1080). Therefore, we propose for certain levels to restrict the maximum tile width to 30 LCU (or to 1920 luma pixels). This sets the maximum line buffer size for an HEVC decoder equal to the required line buffer size of an AVC decoder capable of decoding 1920x1080 resolution.

# BD-rate results

The BDR results are obtained by using dependent tiles. Uniform tiles are used with with splitting the picture into two uniform tiles. Therefore, the line memory is reduced by a factor of two.

Table 6 BDR results for splitting the pictures into two vertical dependent tiles of equal width.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | -0.1% | 0.0% | -0.2% | -0.1% | 0.0% | -0.2% |
| Overall | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.1% | -0.1% | 0.1% | 0.1% | 0.3% | 0.0% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | -0.1% | -0.2% | 0.0% | 0.1% | -0.1% |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class C | -0.1% | 0.0% | -0.1% | -0.1% | -0.2% | -0.1% |
| Class D | 0.0% | -0.2% | 0.1% | 0.0% | 0.3% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | -0.1% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | -0.1% |
| Class F | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.3% | 0.0% | -0.3% | 0.1% |
| Class C | -0.1% | -0.2% | 0.0% | -0.1% | 0.1% | -0.2% |
| Class D | 0.1% | -0.5% | -0.5% | 0.1% | 1.1% | 0.1% |
| Class E | -0.1% | 0.2% | -0.8% | -0.2% | -2.2% | 0.3% |
| Overall | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% | 0.1% |
|  | -0.1% | -0.1% | -0.2% | -0.1% | -0.2% | -0.1% |
| Class F | -0.1% | -0.1% | 0.0% | 0.1% | -0.8% | -0.1% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | 0.1% | -0.1% | 0.2% | -0.5% |
| Class C | -0.1% | 0.0% | -0.4% | -0.1% | 0.1% | -0.1% |
| Class D | 0.0% | 0.3% | -0.3% | 0.0% | 0.3% | 0.5% |
| Class E | -0.2% | 0.0% | -1.3% | -0.3% | -2.2% | -0.7% |
| Overall | -0.1% | 0.0% | -0.4% | -0.1% | -0.2% | -0.2% |
|  | -0.1% | 0.0% | -0.4% | -0.1% | -0.2% | -0.2% |
| Class F | 0.0% | 0.4% | 0.2% | 0.1% | -0.5% | 0.1% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

| Random Access HE-10 |  |  |
| --- | --- | --- |
| Y | U | V |
| -0.1% | -0.4% | 0.0% |
| 0.0% | -0.1% | -0.2% |
| 0.0% | -0.2% | -0.1% |
| 0.0% | -0.2% | -0.1% |
| N/A |  |  |
| 100% |  |  |

The decoding time on a single host has been obtained. The decoding time is approximately the same as that of the anchor. One can see that splitting the picture into two vertical tiles does not bring any average BDR penalty. On the contrary, on LD configurations one can observe a -0.1% BDR decrease when splitting the frames into two vertical tiles.

The BD-rate results have also been obtained for independent tiles. The results have also been obtained for independent tiles. The results are provided in Table 7. One can see some coding efficiency decrease (not exceeding the 0.5% on average for any configurations). However, the results for the Class A (which is of the interest in this case) is from 0.1% to 0.2%.

Table 7 BDR results for splitting the pictures into two vertical independent tiles of equal width

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% |
| Class B | 0.2% | 0.1% | 0.0% | 0.2% | 0.2% | 0.2% |
| Class C | 0.2% | 0.2% | 0.3% | 0.3% | 0.3% | 0.3% |
| Class D | 0.4% | 0.5% | 0.6% | 0.4% | 0.5% | 0.6% |
| Class E | 0.3% | 0.1% | -0.1% | 0.3% | 0.2% | 0.2% |
| Overall | 0.3% | 0.2% | 0.2% | 0.3% | 0.3% | 0.3% |
|  | 0.3% | 0.2% | 0.2% | 0.3% | 0.3% | 0.3% |
| Class F | 0.4% | 0.4% | 0.6% | 0.4% | 0.6% | 0.5% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.2% | 0.1% | 0.0% | 0.2% | 0.4% | 0.1% |
| Class B | 0.2% | 0.3% | 0.2% | 0.3% | 0.4% | 0.3% |
| Class C | 0.4% | 0.4% | 0.5% | 0.3% | 0.3% | 0.3% |
| Class D | 0.9% | 0.9% | 1.3% | 0.9% | 1.1% | 0.9% |
| Class E |  |  |  |  |  |  |
| Overall | 0.4% | 0.5% | 0.5% | 0.5% | 0.6% | 0.4% |
|  | 0.4% | 0.5% | 0.5% | 0.5% | 0.6% | 0.4% |
| Class F | 0.7% | 0.9% | 0.9% | 0.7% | 0.9% | 0.9% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 102% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | 0.4% | 0.5% | 0.2% | 0.1% | 0.9% |
| Class C | 0.3% | 0.3% | 0.5% | 0.3% | 0.6% | 0.5% |
| Class D | 0.9% | 0.9% | 0.9% | 1.0% | 1.7% | 0.9% |
| Class E | 0.7% | 1.1% | -1.0% | 0.7% | -0.7% | 0.8% |
| Overall | 0.5% | 0.6% | 0.3% | 0.5% | 0.5% | 0.8% |
|  | 0.5% | 0.7% | 0.3% | 0.5% | 0.6% | 0.8% |
| Class F | 0.8% | 1.0% | 0.9% | 0.8% | -0.4% | 0.1% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 102% |  |  | 99% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | 0.3% | 0.1% | 0.2% | 0.5% | 0.1% |
| Class C | 0.3% | 0.3% | 0.1% | 0.3% | 0.5% | 0.5% |
| Class D | 0.8% | 1.3% | 0.9% | 0.8% | 1.3% | 1.2% |
| Class E | 0.6% | 1.2% | -0.9% | 0.7% | -1.0% | 0.1% |
| Overall | 0.5% | 0.6% | 0.3% | 0.5% | 0.4% | 0.5% |
|  | 0.4% | 0.7% | 0.1% | 0.5% | 0.4% | 0.4% |
| Class F | 0.9% | 1.4% | 1.4% | 0.9% | 0.4% | 1.0% |
| Enc Time[%] |  |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 95% |  |  |

| Random Access HE-10 |  |  |
| --- | --- | --- |
| Y | U | V |
| 0.2% | 0.3% | 1.0% |
| 0.3% | 0.2% | 0.1% |
| 0.3% | 0.3% | 0.5% |
| 0.3% | 0.3% | 0.5% |
| 100% |  |  |
| 99% |  |  |

# Conclusions

The proposal proposes a way of decreasing the minimum line-buffer size requirement for the decoder by introducing level limits on the maximum tile size. In this way, reduction of decoder line buffer memory can be achieved. The proposed method can also be combined with any other methods targeted at line buffer reduction, thus reducing required amount of line memory even more.

It is proposed to include limitations on tile size into HEVC levels definitions and start working on determining the required limits for tile size (related to the line buffer size).

# References

- Fuldseth (Cisco), M. Horowitz, S. Xu (eBrisk Video), A. Segall (Sharp), M. Zhou (TI), “Tiles”, JCTVC-F335, Italy, July 2011.

- C.-M. Fu, C.-Y. Chen, C.-Y. Tsai, Y.-W. Huang, S. Lei, “CE8.c.2: Single-source SAO and ALF virtual boundary processing”, JCTVC-G204, Geneva, 21-30 November, 2011.

- A. Norkin, R. Sjöberg, “Level restrictions on tiles for line buffer reduction”, JCTVC-G618, Geneva, 21-30 November, 2011.

- A. Norkin, X. Guo, B. Jeon, M. Narroschke, “Description of Core Experiment 10 (CE10): Deblocking Filtering”, JCTVC-G1210, Geneva, Nov. 2011.

# Patent rights declaration(s)

Ericsson may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).