| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0475 |
| --- | --- |

| Title: | Signalling chroma intra prediction mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungsun Kim, Byeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea | Tel: Email: | +82-10-2201-9774 jungsun.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Working Draft

### Binarization process

| Table 9-1 – Syntax elements and associated types of binarization, maxBinIdxCtx, ctxIdxTable, and ctxIdxOffset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax element |  | Type of binarization | maxBinIdxCtx | ctxIdxTable | ctxIdxOffset |
| intra_chroma_pred_mode | I | as specified in subclause 9 | 1 | Error: Reference source not found14 | 0 |
|  | P |  | 1 | Error: Reference source not found14 | 2 |
|  | B |  | 1 | Error: Reference source not found14 | 4 |

#### Binarization process for intra_chroma_pred_mode

Input to this process is a request for a binarization for the syntax element intra_luma_pred_mode and chroma_pred_from_luma_enabled_flag.

Output of this process is the binarization of the syntax element.

The binarization for intra_chroma_pred_mode is given by Table 9-35.

Table 9-35 – Binarization for intra_chroma_pred_mode

| intra_chroma_pred_mode | chroma_pred_from_luma_enabled_flag= true | chroma_pred_from_luma_enabled_flag= false |
| --- | --- | --- |
| 0 | 0 | 0 |
| 1 | 10 |  |
| 2 | 1100 | 100 |
| 3 | 1101 | 101 |
| 4 | 1110 | 110 |
| 5 | 1111 | 111 |

| Table 9-2 – Assignment of ctxIdxInc to binIdx for all ctxIdxTable and ctxIdxOffset values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Syntax element | ctxIdxTable, ctxIdxOffset |  | binIdx |  |  |  |  |
|  |  |  | 0 | 1 | 2 | 3 | >=4 |
| intra_chroma_pred_mode chroma_pred_from_luma_enabled_flag==true | Error: Reference source not found14 | 0 | 0 | 1 | na | na |  |
|  |  | 2 | 0 | 1 | na | na |  |
|  |  | 4 | 0 | 1 | na | na |  |
| intra_chroma_pred_mode chroma_pred_from_luma_enabled_flag==false | Error: Reference source not found14 | 0 | 0 | na | na |  |  |
|  |  | 2 | 0 | na | na |  |  |
|  |  | 4 | 0 | na | na |  |  |

# Patent rights declaration

LG electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).