| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0475 |
| --- | --- |

| Title: | Signalling chroma intra prediction mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungsun Kim, Byeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea | Tel: Email: | +82-10-2201-9774 jungsun.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes a revised method to code intra chroma prediction mode. In the proposed method, 2-length fixed binization and bypass coding are employed to code the chroma mode except DM and LM. It reduces context update processing with gains of 0.0%, -0.2% and -0.2% for luma and chroma components respectively under the AIHE condition and 0.0%, -0.3% and -0.4% for luma and chroma components respectively under the AILC condition.

# Introduction

In the current HM, intra chroma prediction modes are coded with context table after binarization. The binarization is as shown in Table 1. To code chroma prediction mode, two contexts are required. The first bin indicates DM has one context and the rest shares the other one.

Table 1 the binarization for intra chroma prediction modes

| Mode Idx | AIHE | AILC |
| --- | --- | --- |
| DM | 0 | 0 |
| LM | 10 | x |
| mode 0 | 110 | 10 |
| mode 1 | 1110 | 110 |
| mode 2 | 11110 | 1110 |
| mode 3 | 11111 | 1111 |

# Proposed Method

We propose the fixed length binarization and partial bypassing coding for intra chroma prediction modes. The proposed binarization is as follows.

Table 2 the proposed binarization for intra chroma prediction modes

| Mode Idx | AIHE | AILC |
| --- | --- | --- |
| DM | 0 | 0 |
| LM | 10 | x |
| mode 0 | 1100 | 100 |
| mode 1 | 1101 | 101 |
| mode 2 | 1110 | 110 |
| mode 3 | 1111 | 111 |

Since DM mode and LM mode have significant tendency, the bins that indicate DM mode and LM mode are coded with context as before. Therefore, the first and the second bins are context-coded under the AIHE condition and only the first bin is context-coded under the AILC condition. The other bins that indicate mode 0 to mode 3 are bypass coded.

# Results

The proposed method is implemented on the top HM5.0 under the configuration of AIHE and AILC [1]. The binary files for each encoder and decoder are compiled on Window7 32bit PC for Window XP 64bit PC. Experimental results are shown in Table 1. It shows 0.0%, 0.2%, 0.2% and 0.0%, -0.3%, -0.4% for luma component and chroma components in AIHE and AILC case, respectively. Detailed results are described in the accompanied excel file.

Table 3 the BD rate of the proposed method

|  | chroma mode10 IHE |  |  | chroma mode 10 ILC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.0% | -0.2% | -0.4% |
| Class B | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.4% |
| Class C | 0.0% | -0.2% | -0.3% | 0.0% | -0.3% | -0.4% |
| Class D | 0.0% | -0.1% | -0.2% | 0.1% | -0.4% | -0.6% |
| Class E | 0.0% | 0.0% | -0.1% | 0.0% | -0.3% | -0.4% |
| Overall | 0.0% | -0.1% | -0.2% | 0.0% | -0.3% | -0.4% |
|  | 0.0% | -0.1% | -0.2% | 0.0% | -0.3% | -0.4% |
| Class F | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.2% |

# Conclusions

The proposed method provides a simplified coding for code intra chroma prediction mode. Context coding is used for the bins that indicate DM mode or LM mode while bypass coding is used for the rest of the bins. The 2-length fixed binization is applied to those bins. The proposed method reduces context updating process with gains of 0.0%, -0.2% and -0.2% for AIHE condition and 0.0%, -0.3% and -0.4% for AILC condition.

# References

- F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-G1200, 7th JCT-VC Meeting, Geneva, CH, Nov. 2011.

# Patent rights declaration

LG electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).