| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0476 |
| --- | --- |

| Title: | Non-CE11 : Adjusted significant coefficient map context for Non-Square blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungsun Kim, Byeongmoon Jeon #19 Yangjae-daero 11gil, Seocho-gu Seoul 137-130 Korea | Tel: Email: | +82-10-2201-9774 jungsun.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

The contribution proposes to re-group coefficient which will have same range of context indexes for non square blocks. The coefficients locate at the position (x,y) where (x+2*y) or (2*x+y), depending on the shape of non square block, is less than threshold except the first coefficient shares context indexes in mid-range. It shows 0.0%, 0.0%, -0.1% and 0.0%, -0.1%, 0.1% for luma component and chroma components in RAHE and LDHE case, respectively.

# Introduction

In the current HM, context indexes for coefficients in large blocks including non square transform blocks are determined by their position and their neighbours. The first coefficient locates at (0,0) always has its own context index, CTX_IDX. To the others in the block, context indexes are assigned as follows.

ctx(x,y)=CTX_IDX+1+N(x,y)x+y<thresholdCTX_IDX+4+N(x,y)otherwise

Where N(x,y) indicates number of non-zero coefficients in the neighbourhood. N(x,y) can be achieved as follows.

N(x,y)=(i,j)∈Niehbourc(i,j)!=0>>1

The threshold in Eq. 1 is set as a quarter of longer side of the block. For example it is set as 4 for 16x16 and 4x16/16x4. Fig. 1 shows the example of 16x16 luma block and 16x4/4x16 luma block. The coefficient on the first pixel has context index 20. For coefficients locate in green region, the range of context index is [21, 23] and for coefficients locate in blue region, the range of context index is [24, 26].

Fig. 1 Regions in which coefficients have same context range

# Proposed Method

We propose to adjust the regions in which coefficients shares context indexes for non square blocks. In the proposed method, the shape of the region has mid-range context indexes is adjusted according to the shape of the block. Fig. 2 shows the example of 16x4/4x16 block. The green region in which the range of context index is [21, 23] shrinks as one side of the block is shortened.

Fig. 2 Proposed regions in which coefficients have same context range

Therefore, the context index can be decided as follows;

ctx(x,y)=CTX_IDX+1+N(x,y)x+2×y<threshold||chromablockCTX_IDX+4+N(x,y)otherwise1. width>height

2.width<height

ctx(x,y)=CTX_IDX+1+N(x,y)2×x+y<threshold||chromablockCTX_IDX+4+N(x,y)otherwise

# Results

The method is implemented on the top HM5.0 under the configuration of RAHE and LDHE [1]. The binary files for each encoder and decoder are compiled on Window7 32bit PC for Window XP 64bit PC. Experimental results are shown in Table 1. It shows 0.0%, 0.0%, -0.1% and 0.0%, -0.1%, 0.1% for luma component and chroma components in RAHE and LDHE case, respectively.

Table 1 BD rate of the proposed method

|  | Random Access HE |  |  | Low delay B HE |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% |  |  |  | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | -0.1% | 0.0% | -0.2% | 0.0% |  |  |  |
| Class D | 0.0% | -0.1% | -0.1% | 0.0% | -0.2% | 0.4% |  |  |  |
| Class E |  |  |  | 0.1% | -0.3% | -0.3% |  |  |  |
| Overall | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class F | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% | -0.3% |  |  |  |
| Enc Time[%] | 100% |  |  | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 100% |  |  |

The proposed method can be applied to 16x4/4x16 block and 32x8/8x32 block separately. Table 2 shows that the results when the proposed method is applied to 32x8/8x32 blocks and the method of JCTVC-G750 [2] is applied to 16x4/4x16 blocks. It shows slightly improved results of 0.0%, 0.0%, -0.0% and 0.1%, 0.1%, 0.1% for luma component and chroma components in RAHE and LDHE case, respectively. Detailed results are described in the accompanied excel files.

Table 2 BD rate of the harmonization with H0443

|  | Random Access HE |  |  | Low delay B HE |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8bit) | 0.0% | 0.0% | 0.0% |  |  |  | 0.1% | 0.1% | 0.0% |
| Class B | 0.0% | 0.0% | -0.1% | 0.1% | 0.3% | 0.3% | 0.1% | 0.1% | 0.1% |
| Class C | 0.0% | 0.0% | -0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| Class D | 0.0% | -0.1% | -0.1% | 0.1% | -0.1% | -0.1% |  |  |  |
| Class E |  |  |  | 0.1% | 0.2% | -0.1% |  |  |  |
| Overall | 0.0% | 0.0% | -0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
|  | 0.0% | 0.0% | -0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 100% |  |  |

# Conclusions

We propose to adjust regions in which coefficients have same context range according to the shape of the block. Therefore, the region has mid-range context indexes is shortened along the shorter side of the block. It reflects that coefficients in non square block do not have same properties as those in square block. It shows 0.0%, 0.0%, -0.1% and 0.0%, -0.1%, 0.1% for luma component and chroma components in RAHE and LDHE case, respectively.

# Working Draft

###### Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

Inputs to this process are the color component index cIdx, the current coefficient scan position ( xC , yC ), the transform block width log2TrafoWidth and the transform block height log2TrafoHeight.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xC, yC ), the color component index cIdx, the transform block size and previsously decoded bins of the syntax element significant_coeff_flag. For the derivation of sigCtx, the following applies.

- If log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 2, sigCtx is derived using ctxIdxMap4x4[ ] specified in Error: Reference source not found as follows..

sigCtx  =  ctxIdxMap4x4[ ((cIdx > 0) ? 15 : 0) + (yC << 2) + xC ]	(9-1)

- Otherwise if log2TrafoWidth is equal to log2TrafoHeight and log2TrafoWidth is equal to 3, sigCtx is derived using ctxIdxMap8x8[ ] specified in Error: Reference source not found as follows.

sigCtx  =  ((xC + yC) = = 0) ? 10 : ctxIdxMap8x8[ ((yC >> 1 ) << 2) + (xC >> 1) ]	(9-2)

sigCtx  +=  ( cIdx > 0) ? 6: 9		(9-3)

- Otherwise if xC + yC is equal to 0, sigCtx is derived as follows.

sigCtx  =  ( cIdx > 0) ? 17: 20		(9-4)

- Otherwise (xC + yC is greater than 0), sigCtx is derived using previously decoded bins of the syntax element significant_coeff_flag as follows.

- The variable sigCtx is initialized as follows.

sigCtx  =  0		(9-5)

- When xC is less than ( 1 << log2TrafoWidth ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC ]	(9-6)

- When xC is less than ( 1 << log2TrafoWidth ) − 1 and yC is less than ( 1 << log2TrafoHeight ) − 1, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 1 ][ yC + 1 ]	(9-7)

- When xC is less than ( 1 << log2Width ) − 2, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC + 2 ][ yC ]	(9-8)

- When all of the following conditions are true,

  - yC is less than ( 1 << log2TrafoHeight ) − 1,

  - xC % 4 is not equal to 0 or yC % 4 is not equal to 0,

  - xC % 4 is not equal to 3 or yC % 4 is not equal to 2,

the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 1 ]	(9-9)

- When yC is less than ( 1 << log2TrafoHeight ) − 2 and sigCtx is less than 4, the following applies.

sigCtx  =  sigCtx  +  significant_coeff_flag[ xC ][ yC + 2 ]	(9-10)

- The variable sigCtx is modified as follows.

  - If any of the following conditions are true, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + 24

- cIdx is equal to 0 and log2TrafoHeight is greater than log2TrafoWidth and xC<<1 + yC are greater than (1 << (max(log2TrafoWidth, log2TrafoHeight) − 2)) – 1

- cIdx is equal to 0 and log2TrafoWidth is greater than log2TrafoHeight and xC + yC<<1 are greater than (1 << (max(log2TrafoWidth, log2TrafoHeight) − 2)) – 1

- cIdx is equal to 0 and xC + yC are greater than (1 << (max(log2TrafoWidth, log2TrafoHeight) − 2)) − 1,

  - Otherwise, the following applies.

sigCtx = ( (sigCtx + 1) >> 1 ) + ( (cIdx > 0) ? 18 : 21 )	(9-11)

The context index increment ctxIdxInc is derived using the color component index cIdx and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  sigCtx		(9-12)

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  27  +  sigCtx

# References

- F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-G1200, 7th JCT-VC Meeting, Geneva, CH, Nov. 2011.

- J. Kim, B. Jeon, J. Sole, X. Wang, M. Karczewicz, “CE11: Harmonization of residual coding and NSQT”, JCTVC-H0443, 8th JCT-VC Meeting, San Jose, USA, Feb. 2012.

# Patent rights declaration

LG electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).