| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0479 |
| --- | --- |

| Title: | Cross-check of CE5.g: Integration of tools CE5.a to CE5.e |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross check report |  |  |
| Author(s) or Contact(s): | Jing Wang Xiang Yu Da-ke He | Tel: Email: | wangj@rim.com xiayu@rim.com dhe@rim.com |
| Source: | Research In Motion Ltd., Canada |  |  |

_____________________________

# Abstract

This document reports outcome of Research In Motion’s (RIM) crosscheck results of CE5.g “Integration of tools CE5.a to CE5.e”. This activity is part of Core Experiment 5 on transform skipping [1]. Tests have been performed for Inter RQT depths 3. The BD-rate numbers reported by CE5.g proponents match the numbers obtained in this crosscheck. An informal subjective viewing has been performed on some test sequences. No unusual effects were observed.

# Verification of experimental results

The table below summaries the BD-rate numbers for all five mandatory inter-settings (RA-HE, RA-LC, LB-HE, LB-LC, RA-HE10) for CE5.g based on HM5. HM5 is used as the anchor. The results match with what reported for CE5.g for tests at Inter RQT depths 3.

|  | Random Access HE |  |  | Random Access LC |  |  | Random Access HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.3% | 0.0% | -0.2% | -0.2% | 0.1% | -0.2% | 0.0% |
| Class B | -0.3% | -0.2% | -0.1% | -0.7% | -0.3% | 0.0% | -0.2% | -0.2% | -0.1% |
| Class C | -0.5% | -0.3% | -0.2% | -1.0% | -0.4% | 0.0% |  |  |  |
| Class D | -0.8% | -0.3% | -0.2% | -1.5% | 0.0% | 0.1% |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.4% | -0.3% | -0.2% | -0.9% | -0.3% | 0.0% | -0.1% | -0.2% | -0.1% |
|  | -0.4% | -0.3% | -0.2% | -0.9% | -0.3% | 0.0% | -0.1% | -0.2% | -0.1% |
| Class F | -2.1% | -1.1% | -1.1% | -2.9% | -1.7% | -1.8% |  |  |  |
| Enc Time[%] | 130% |  |  | 122% |  |  | 130% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 102% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | 0.3% | 0.4% | -1.1% | -0.9% | -0.1% |
| Class C | -0.7% | -0.3% | -0.1% | -1.6% | -0.6% | 0.3% |
| Class D | -1.2% | 0.4% | 0.4% | -2.2% | 1.0% | 0.9% |
| Class E | -0.3% | 0.3% | 0.6% | -1.5% | 1.5% | 1.0% |
| Overall | -0.6% | 0.2% | 0.3% | -1.6% | 0.1% | 0.5% |
|  | -0.6% | 0.1% | 0.2% | -1.5% | 0.0% | 0.4% |
| Class F | -4.7% | -3.0% | -3.3% | -6.6% | -5.3% | -4.6% |
| Enc Time[%] | 126% |  |  | 117% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

# Subjective quality

An informal subjective viewing has been performed on some test sequences. No unusual effects were observed.

# References

[1] M. Mrak, et al., “Description of Core Experiment 5: Transform Skipping”, JCTVC-G1205.

[2] M. Mrak, A. Gabriellini, G. Van Wallendael, R. Joshi, “CE5.g: Integration of tools CE5.a to CE5.e”, JCTVC-H0211.

# Patent rights declaration(s)

Research In Motion Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).