| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0481 |
| --- | --- |

| Title: | Multiple Sign Bits Hiding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Yu Jing Wang Dake He Gaelle Martin-Cocher Steve Campbell | Email: | xiayu@rim.com wangj@rim.com dhe@rim.com gmartincocher@rim.com scampbell@rim.com |
| Source: | Research In Motion Ltd. |  |  |

_____________________________

# Abstract

The contribution proposes to embed multiple sign bits for TUs larger than 4x4. Specifically, as nonzero coefficients and significance maps for large transform units are encoded in 4x4 coefficient sets, for each 4x4 coefficient set that satisfies certain condition the sign bit of the first nonzero coefficient is skipped from the bit stream and later inferred from the parity of the sum of all nonzero coefficients in that coefficient set. Experimental results for high efficiency settings based on HM5.0 showed an average BD-rate performance of -0.7% (AI-HE), -0.6% (RA-HE), and -0.7% (LD-HE) with an average encoder runtime of 105%, 102%, 102%, and decoder runtime of 99%, 100%, 100%, compared to HM5.0 anchors. For LC settings where RDOQ is off, the encoder may apply a minimum distortion criterion to ensure the match between a sign bit and a corresponding parity. Experimental results showed an average BD-rate performance of -1.3% (AI-LC), -1.4% (RA-LC), and -1.0% (LD-LC) with an average encoder runtime of 105%, 101%, 101%, and decoder runtime of 99%, 100%, 100%, compared to HM5.0 anchors. Subjective viewing of decoded sequences did not observe artefact that may be caused by the proposed technique.

# Introduction

In CE11 [1] subset 3, a sign data hiding technique was proposed in G271[2], where for each TU satisfying a condition that the number of coefficients between the first and last nonzero coefficients is greater than a threshold, the sign bit for the first nonzero coefficient is embedded into the parity of the sum of all nonzero coefficients. Four thresholds are signalled according to four data types. For all TUs with various sizes from 4x4 to 32x32, one sign bit is hidden for each TU.

# Proposal

In this proposal, TUs with a larger size than 4x4 may have multiple sign bits embedded. Specifically, as coefficients in a large TU are coded by 4x4 coefficient groups, this proposal applies the sign bit hiding technique to each 4x4 coefficient group. As such, a single threshold for the number of coefficients is defined for each picture.

For example, for a 16x16TU, the significance flags and significant coefficients are coded in 4x4 coefficient groups. In this proposal, the sign-bit hiding technique in [2] is applied to each coefficient group. As illustrated below, three coefficient groups may satisfy the hiding condition and results in three bits hidden in the 16x16 TU.

A 16x16 TU with 3 bits hidden in 3 4x4 coefficient groups.

# Algorithm description

## Decoding

For each coefficient group, the decoder checks the number of coefficients between the first nonzero coefficient and the last nonzero coefficient and compare it with a threshold. If the threshold is exceeded, the sign bit for the first nonzero coefficient will be inferred from the sum of all nonzero coefficients. Otherwise, the sign bit will be read from the bit-stream.

## Encoding

On the encoding side, the main task is to find a coefficient to adjust by +1 or -1 when the parity of the sum of all nonzero coefficients does not match with the sign of the first non zero coefficient. (In the current implementation, it is assumed that an even sum matches with a positive sign and an odd sum matches with a negative sign.)

The quantization procedure without RDOQ is firstly introduced to facilitate the discussion on this task.

U = (abs(piCoef) * uiQ + iAdd ) >> iQBits;   // As in HM5.0 codec              (1)

Where piCoef is the transform output, U is the quantization output, uiQ and iQBits

### RDOQ-off case

For RDOQ-off case, when the sign does not match with the parity, the following algorithm is used to search for a coefficient to be adjusted:

- For each coefficient piCoef, calculate deltaU = abs(piCoef) * uiQ – U<<iQBits.

- If deltaU>0, note down +1 as the adjusting direction and –deltaU as a local minimum cost for the current coefficient; if deltaU<0, note down -1 as the adjusting direction and deltaU as a local minimum cost for the current coefficient.

- Find the one with the minimum cost within a predefined search range (see remark-4), and adjust it by the noted direction.

Remark-1: The multiplication at Step-1 may be combined with the quantization procedure shown in (1).

Remark-2: At step-2, deltaU>0 means that abs(piCoef) is greater than its reconstruction. Thus U+1 is a better choice than U-1.

Remark-3: The above search finds the coefficient with the minimum distortion cost. As illustrated in the following figure, when deltaU>0, the distortion cost for U+1 is guaranteed to be smaller than U-1. Similar for detalU<0 case.

Illustration of the distortion cost for quantizing piCoef to U, U+1, or U-1.

Remark-4: The search range is an encoder-side choice. In the current implementation, when the current group does not contain the last nonzero for the current TU, the search range is from the first nonzero to the last coefficient in this group plus coefficients that are ahead of the first nonzero coefficient and have the  same sign as the first non zero coefficient. When the current group contains the last nonzero for the current TU, the search stops by the last nonzero coefficient.

### RDOQ-on case

For RDOQ-on case, as rate information is available from the RDOQ procedure, an RDO search is carried out to find one coefficient to adjust when the parity of the sum and the sign bit of the first nonzero coefficient does not match.

# Results

## Common-QP tests

|  |  | All Intra HE |  |  |  |  |  | Random Access HE |  |  |  |  |  | Low delay B HE |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y |  | U |  | V |  | Y |  | U | V |  |  | Y | U |  | V |  |
| Class A (8b) |  | -0.7% |  | -1.7% |  | -1.7% |  | -0.5% |  | -0.6% | -0.9% |  |  |  |  |  |  |  |
| Class B |  | -0.8% |  | -1.6% |  | -1.5% |  | -0.7% |  | -0.9% | -0.9% |  |  | -0.8% | -0.9% |  | -0.9% |  |
| Class C |  | -0.8% |  | -1.3% |  | -1.3% |  | -0.6% |  | -0.6% | -0.5% |  |  | -0.8% | -1.0% |  | -1.1% |  |
| Class D |  | -0.8% |  | -1.3% |  | -1.2% |  | -0.6% |  | -0.7% | -0.7% |  |  | -0.7% | -1.6% |  | -0.9% |  |
| Class E |  | -0.5% |  | -1.7% |  | -1.6% |  |  |  |  |  |  |  | -0.3% | -0.2% |  | -1.4% |  |
| Overall |  | -0.7% |  | -1.5% |  | -1.4% |  | -0.6% |  | -0.7% | -0.7% |  |  | -0.7% | -1.0% |  | -1.0% |  |
|  |  | -0.8% |  | -1.5% |  | -1.4% |  | -0.6% |  | -0.8% | -0.7% |  |  | -0.7% | -0.9% |  | -1.1% |  |
| Class F |  | -1.0% |  | -1.1% |  | -1.0% |  | -0.8% |  | -0.5% | -0.6% |  |  | -0.8% | -0.7% |  | -0.7% |  |
| Enc Time[%] |  | 105% |  |  |  |  |  | 102% |  |  |  |  |  | 102% |  |  |  |  |
| Dec Time[%] |  | 99% |  |  |  |  |  | 100% |  |  |  |  |  | 100% |  |  |  |  |
|  | All Intra LC |  |  |  |  |  | Random Access LC |  |  |  |  |  | Low delay B LC |  |  |  |  |  |
|  | Y |  | U |  | V |  | Y |  | U |  |  | V | Y |  |  | U |  | V |
| Class A (8b) | -1.3% |  | -1.4% |  | -1.3% |  | -1.2% |  | -0.8% |  |  | -1.3% |  |  |  |  |  |  |
| Class B | -1.3% |  | -1.3% |  | -1.3% |  | -1.4% |  | -1.1% |  |  | -1.4% | -1.2% |  |  | -0.1% |  | -0.1% |
| Class C | -1.2% |  | -1.3% |  | -1.2% |  | -1.4% |  | -1.1% |  |  | -1.1% | -1.0% |  |  | 0.1% |  | -0.3% |
| Class D | -1.4% |  | -1.2% |  | -1.2% |  | -1.7% |  | -0.8% |  |  | -1.4% | -1.2% |  |  | 0.7% |  | -0.3% |
| Class E | -1.1% |  | -0.9% |  | -0.9% |  |  |  |  |  |  |  | -0.4% |  |  | -2.0% |  | 0.2% |
| Overall | -1.3% |  | -1.2% |  | -1.2% |  | -1.4% |  | -1.0% |  |  | -1.3% | -1.0% |  |  | -0.2% |  | -0.1% |
|  | -1.3% |  | -1.2% |  | -1.2% |  | -1.4% |  | -1.0% |  |  | -1.4% | -1.0% |  |  | -0.2% |  | -0.1% |
| Class F | -1.8% |  | -1.3% |  | -1.6% |  | -2.1% |  | -1.9% |  |  | -1.9% | -2.1% |  |  | -1.9% |  | -2.2% |
| Enc Time[%] | 105% |  |  |  |  |  | 101% |  |  |  |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |  | 100% |  |  |  |  |  | 100% |  |  |  |  |  |

|  | Random Access HE-10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.7% | 2.4% | 2.7% |
| Class B | -0.3% | 0.3% | 0.1% |
| Class C |  |  |  |
| Class D |  |  |  |
| Class E |  |  |  |
| Overall | -0.5% | 1.2% | 1.3% |
|  | 0.2% | 1.4% | 1.4% |
| Class F |  |  |  |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 100% |  |  |

The results have been crossed check in [3].

## Bin-saving, EP-bin-saving and hidden ratio

As shown in JCTVC-H0481_BinCount_HiddenRate.xls, Bin-saving is calculated as Hidden_bits/Total_Bins * 100, where Hidden_bits gives the number of sign bits that are not explicitly transmitted in the bit-stream but are inferred from the parity of residuals sum, Total_Bins is collected as the total number of decoded bins.

EP-Bin-saving is computed as Hidden_bits/EP_Bins *100, where EP_Bins is the number of EP bins.

Hidden ratio is Hidden_bits/Total_bits*100, where Total_bits is the size of the bit stream in bits.

The statistics have been collected for all common QP tests.

In summary, the average Bin-saving, EP-bin-saving and hidden-ratio are as follows:

|  | AI-HE | RA-HE | LB-HE | AI-LC | RA-LC | LB-LC |
| --- | --- | --- | --- | --- | --- | --- |
| Bin-saving | 1.8 | 1.5 | 1.5 | 1.9 | 1.4 | 1.3 |
| EP-bin-saving | 7.0 | 7.3 | 9.7 | 7.6 | 7.1 | 9.4 |
| Hidden-ratio | 2.2 | 1.8 | 1.8 | 2.2 | 1.7 | 1.6 |

## Low-QP results

Low-QP results have been collected for QP=12, 17, 22, 27.

|  | All Intra HE |  |  | Random Access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8b) | -1.0% | -1.4% | -1.4% | -1.0% | -0.9% | -0.8% |  |  |  |
| Class B | -0.9% | -1.6% | -1.4% | -1.2% | -1.2% | -0.9% | -1.2% | -1.3% | -1.2% |
| Class C | -1.0% | -1.3% | -1.2% | -1.0% | -0.8% | -0.7% | -1.2% | -1.1% | -1.1% |
| Class D | -1.0% | -1.2% | -1.1% | -1.0% | -0.8% | -0.8% | -1.2% | -1.1% | -1.2% |
| Class E | -0.8% | -1.6% | -1.4% |  |  |  | -1.1% | -2.0% | -1.5% |
| Overall | -0.9% | -1.4% | -1.3% | -1.1% | -0.9% | -0.8% | -1.2% | -1.3% | -1.2% |
|  | -1.0% | -1.5% | -1.3% | -1.0% | -1.0% | -0.8% | -1.2% | -1.4% | -1.3% |
| Class F | -1.1% | -1.0% | -1.0% | -1.0% | -0.8% | -0.8% | -0.9% | -1.0% | -0.9% |
| Enc Time[%] |  |  |  |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |  |  |  |

|  | All Intra LC |  |  | Random Access LC |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A (8b) | -1.8% | -1.7% | -1.6% | -2.0% | -1.1% | -0.8% |  |  |  |
| Class B | -1.4% | -1.6% | -1.5% | -2.1% | -1.0% | -0.9% | -2.2% | -0.3% | 0.4% |
| Class C | -1.7% | -1.8% | -1.7% | -2.4% | -1.6% | -1.7% | -2.2% | -0.8% | -0.7% |
| Class D | -1.9% | -1.9% | -1.8% | -2.7% | -1.8% | -1.9% | -2.5% | -0.8% | -1.0% |
| Class E | -1.4% | -0.8% | -0.8% |  |  |  | -0.7% | 0.5% | 1.6% |
| Overall | -1.6% | -1.6% | -1.5% | -2.3% | -1.4% | -1.3% | -2.0% | -0.4% | 0.0% |
|  | -1.6% | -1.6% | -1.5% | -2.2% | -1.3% | -1.3% | -1.9% | -0.3% | 0.1% |
| Class F | -2.0% | -1.7% | -1.8% | -2.4% | -2.0% | -2.0% | -2.4% | -1.5% | -1.7% |
| Enc Time[%] |  |  |  |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |  |  |  |

|  | Random Access HE-10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.9% | 0.3% | 0.3% |
| Class B | -0.9% | 0.3% | 0.7% |
| Class C |  |  |  |
| Class D |  |  |  |
| Class E |  |  |  |
| Overall | -0.9% | 0.3% | 0.5% |
|  | -0.8% | 0.5% | 0.8% |
| Class F |  |  |  |
| Enc Time[%] |  |  |  |
| Dec Time[%] |  |  |  |

Note that the enc/dec time in the spread sheet for LowQP tests is not reliable as the anchor is obtained in CE11 report. Thus, the time is not included the above tables.

# Conclusions

This contribution proposed to embed multiple sign bits for TUs larger than 4x4 based on 4x4 coefficient groups. As for large TUs the significance flags and significant coefficients are coded in 4x4 coefficient groups in HM, it combines the sign-bit-hiding with the significant coefficient coding, with a unified syntax and homogenous coding gain. It is proposed to be adopted into HEVC working draft and reference software.

# References

[1] V. Sze et al, “Description of Core Experiment (CE11): Coefficient scanning and coding”, JCTVC- G1211.

[2] G. Clare, F. Henry, and J. Jung, “Sign Data Hiding”, JCTVC-G271.

[3] J. Sole, “Cross-check of RIM’s multiple sign bits hiding(JCTVC-H0481)”, JCTVC-H0503.

# Syntax for working draft

Please see attached document “IntegratedText-G1103_d9-H0526-H481-bb0_G644fix.doc”, where the text of this proposal and other related proposals are integrated into G1103_d9. Note that the text changes corresponding to this proposal are marked with yellow.

# Patent rights declaration(s)

Research In Motion Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).