## Scanning array initialisation process

### Up-right diagonal scanning array initialisation process

Input to this process is a block width blkWidth and a block height blkHeight.

Output of this process is the array DiagScan[ pos ][ comp ]. The array index pos specify the scan position ranging from 0 to ( blkWidthSize * blkHeightSize ) − 1. The array index comp equal to 0 specifies the horizontal component and the array index comp equal to 1 specifies the vertical component. Depending on the values of blkWidth and blkHeight, the array DiagScan is derived as follows.

–	If blkWidth is less than 8 and blkHeight is less than 8, the following applies.

i  =  0
x  =  0
y  =  0
stopLoop  =  false
while( !stopLoop ) {
	while( y  >=  0 ) {
		if ( x  <  blkWidth  &&  y  <  blkHeight ) {
			DiagScan[ i ][ 0 ]  =  x
			DiagScan[ i ][ 1 ]  =  y
			i++
		}
		y− −
		x++
	}
	y  =  x
	x  =  0
	if ( i  >=  blkWidth  *  blkHeight ) 
		stopLoop  =  true
}

–	Otherwise (blkWidth is greater than 4 or blkHeight is greater than 4), the following applies.

x_off  =  0
y_off  =  0
i_off  =  0
stopLoopSubblocks = false
while( !stopLoopSubblocks ) { 
	i  =  0
	x  =  0
	y  =  0
	stopLoop  =  false
	while( !stopLoop ) {
		while( y  >=  0 ) {
			if ( x  <  4  &&  y  <  4 ) {
				DiagScan [ i + i_off ][ 0 ]  =  x + x_off
				DiagScan [ i + i_off ][ 1 ]  =  y + y_off
				i++
			}
			y− −
			x++
		}
		y  =  x
		x  =  0
		if ( i  >=  16 ) 
			stopLoop  =  true
	}
	i_off  =  i_off + 16
	if ( i_off  >=  blkWidth * blkHeight ) {
		stopLoopSubblocks  =  true
	} else{
		do {
			y_off  =  y_off − 4
			x_off  =  x_off + 4
			if ( y_off  <  0 ) {
				y_off  =  x_off
 				x_off  =  0
			}
		} while( !( x_off  <  blkWidth  &&  y_off  <  blkHeight ) ) 
	}
}

#### Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| sign_data_hiding_flag | u(1) |
| if ( sign_data_hiding _flag){ |  |
| sign_hiding_threshold | u(4) |
| } |  |
| num_short_term_ref_pic_sets | ue(v) |
| for(idx = 0; idx < num_short_term_ref_pic_sets; idx++) |  |
| short_term_ref_pic_set( idx ) |  |
| long_term_ref_pics_present_flag | u(1) |
| entropy_coding_synchro | u(v) |
| cabac_istate_reset_flag | u(1) |
| if( entropy_coding_synchro ) |  |
| num_substreams_minus1 | ue(v) |
| num_temporal_layer_switching_point_flags | ue(v) |
| for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ) |  |
| temporal_layer_switching_point_flag[ i ] | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| [Ed. (BB): not present in HM software ] |  |
| pic_init_qp_minus26 | se(v) |
| constrained_intra_pred_flag | u(1) |
| enable_temporal_mvp_flag | u(1) |
| slice_granularity | u(2) |
| max_cu_qp_delta_depth | ue(v) |
| chroma_cb_qp_offset | se(v) |
| chroma_cr_qp_offset | se(v) |
| weighted_pred_flag | u(1) |
| weighted_bipred_idc | u(2) |
| tile_info_present_flag | u(1) |
| tile_control_present_flag | u(1) |
| if( tile_info_present_flag = = 1 ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| if( num_tile_columns_minus1 != 0 ¦ ¦ num_tile_rows_minus1 != 0 ) { |  |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width[i] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height[i] | ue(v) |
| } |  |
| } |  |
| } |  |
| if ( tile_control_present_flag ) { |  |
| if ( num_tile_columns_minus1 != 0 ¦ ¦ num_tile_rows_minus1 != 0 ) { |  |
| tile_boundary_independence_flag | u(1) |
| if ( tile_boundary_independence_flag = = 1 ) |  |
| loop_filter_across_tile_flag | u(1) |
| } |  |
| } |  |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

### 7.3.10 Residual coding syntax

| residual_coding ( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| last_significant_coeff_x_prefix | ae(v) |
| last_significant_coeff_y_prefix | ae(v) |
| if ( last_significant_coeff_x_prefix > 3 ) |  |
| last_significant_coeff_x_suffix | ae(v) |
| if ( last_significant_coeff_y_prefix > 3 ) |  |
| last_significant_coeff_y_suffix | ae(v) |
| numCoeff = 0 |  |
| do { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ numCoeff ][ 1 ] |  |
| numCoeff++ |  |
| } while( ( xC != LastSignificantCoeffX ) ¦¦ ( yC != LastSignificantCoeffY ) ) |  |
| numLastSubset = (numCoeff − 1) >> 4 |  |
| for( i = numLastSubset; i >= 0; i− − ) { |  |
| offset = i << 4 |  |
| if(scanIdx == 1 && log2TrafoWidth == 3 && log2TrafoHeight == 3) { |  |
| xCG = 0 |  |
| yCG = i |  |
| } else if(scanIdx == 2 && log2TrafoWidth == 3 && log2TrafoHeight ==3) { |  |
| xCG = i |  |
| yCG = 0 |  |
| } else { |  |
| xCG = ScanOrder[ log2TrafoWidth – 2 ][ log2TrafoHeight – 2 ][ scanIdx ][ i ][ 0 ] |  |
| yCG = ScanOrder[ log2TrafoWidth – 2 ][ log2TrafoHeight – 2 ][ scanIdx ][ i ][ 1 ] |  |
| } |  |
| rightCGFlag = (xCG = = (1<< (log2TrafoWidth – 2)) − 1) ¦¦ (scanIdx == 1 && log2TrafoWidth == 3 && log2TrafoHeight == 3)? 0 : significant_coeff_group_flag[ xCG + 1 ][ yCG ] |  |
| bottomCGFlag = (yCG = = (1 << (log2TrafoHeight – 2)) − 1) ¦¦ (scanIdx == 2 && log2TrafoWidth == 3 && log2TrafoHeight ==3)? 0 : significant_coeff_group_flag[ xCG ][ yCG + 1 ] |  |
| if( (i < numLastSubset) && (rightCGFlag + bottomCGFlag < 2) && (i > 0) ) |  |
| significant_coeff_group_flag[ xCG ][ yCG ] | ae(v) |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if ( (n + offset) < (numCoeff − 1) && significant_coeff_group_flag[ xCG ][ yCG ] ) { |  |
| numNZInCG = (i = = numLastSubset) ? 1 : 0 |  |
| if ( (n > 0) ¦ ¦ (rightCGFlag + bottomCGFlag = = 2) ¦ ¦ (i = = 0) ¦ ¦ (numNZInCG > 0) ) { |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| numNZInCG += significant_coeff_flag[ xC ][ yC ] |  |
| } |  |
| } |  |
| } |  |
| } |  |
| firstNZPosInCG = 1<<4 |  |
| lastNZPosInCG = − 1 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| if( lastNZPosInCG ==-1) |  |
| lastNZPosInCG = n |  |
| firstNZPosInCG = n |  |
| } |  |
| } |  |
| signHidden = lastNZPosInCG – firstNZPosInCG >= sign_hiding_threshold |  |
| for( n = 15; n >= 0; n− − ) { |  |
| if( coeff_abs_level_greater1_flag[ n ] ) |  |
| coeff_abs_level_greater2_flag[ n ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) |  |
| if (!sign_data_hiding ¦¦ !signHidden ¦¦ n != firstNZPosInCG) |  |
| coeff_sign_flag[ n ] | ae(v) |
| else |  |
| coeff_sign_flag[n] = 0 |  |
| } |  |
| sumAbs = 0 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| if( coeff_abs_level_greater2_flag[ n ] ) |  |
| coeff_abs_level_minus3[ n ] | ae(v) |
| xC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoWidth ][ log2TrafoHeight ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_minus3[ n ] + 3 ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if (sign_data_hiding&&signHidden) { |  |
| sumAbs += coeff_abs_level_minus3[n]+3 |  |
| if(n==firstNZPosInCG&& (sumAbs%2 == 1)) |  |
| transCoeffLevel[x0][y0][cIdx][xC][yC] = - transCoeffLevel[x0][y0][cIdx][xC][yC] |  |
| } |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = 0 |  |
| } |  |
| } |  |
| } |  |

#### Picture parameter set RBSP semantics

pic_parameter_set_id identifies the picture parameter set that is referred to in the slice header. The value of pic_parameter_set_id shall be in the range of 0 to 255, inclusive.

seq_parameter_set_id refers to the active sequence parameter set. The value of seq_parameter_set_id shall be in the range of 0 to 31, inclusive.

sign_data_hiding_flag equal to 0 specifies that sign bit hiding is disabled. sign_data_hiding_flag equal to 1 specifies that sign bit hiding is enabled

sign_hiding_threshold specifies a threshold to be compared with the number of coefficients that are between the last non-zero coefficient and the first non-zero coefficient along the scanning path within a coefficient group to determine whether the sign bit of the first nonzero coefficient is present in the bitstream or whether it will be inferred.

num_short_term_ref_pic_sets specifies the number of short-term reference picture sets that are specified in the picture parameter set. The value of num_short_term_ref_pic_sets shall be in the range of 0 to 64, inclusive.

NOTE – A decoder must allocate space for a total number of num_short_term_ref_pic_sets + 1 short-term reference picture sets since a coded video sequence may contain up to one short-term reference picture set explicitly signalled in the slice headers of a current picture. An explicitly signalled short-term reference picture set will always have an index equal to num_short_term_ref_pic_sets in the list of short-term reference picture sets.

long_term_ref_pics_present_flag equal to 0 specifies that no long-term reference picture is used for inter prediction of any coded picture referring to the picture parameter set. long_term_ref_pics_present_flag equal to 1 specifies that long-term reference pictures may be used for inter prediction of one or more coded pictures referring to the picture parameter set.

7.4.10 Residual coding semantics

significant_coeff_group_flag[ xCG ][ yCG ] specifies for the coefficient group position ( xCG, yCG ) within the current transform block whether the corresponding coefficient group at location ( xCG, yCG ) has non-zero transform coefficient level. A coefficient group at location ( xCG, yCG ) is an array of 16 transform coefficient level at locations ( xC, yC ).

- If significant_coeff_group_flag[ xCG ][ yCG ] is equal to 0, the 16 transform coefficient levels of the coefficient group at location ( xCG, yCG ) are inferred to be equal to 0;

- Otherwise (significant_coeff_group_flag[ xCG ][ yCG ] is equal to 1), the following applies.

- If significant_coeff_group_flag[ xCG ][ yCG ] is present or ( xCG << 2, yCG << 2 ) is the last significant position ( LastSignificantCoeffX, LastSignificantCoeffY ), at least one of the 16 transform coefficient levels of the coefficient group at location ( xCG, yCG ) has a non zero value.

- Otherwise (significant_coeff_group_flag[ xCG ][ yCG ] is not present and ( xCG << 2, yCG << 2 ) is not the last significant position ( LastSignificantCoeffX, LastSignificantCoeffY )), at least one of the 16 significant_coeff_flag syntax elements is present for the coefficient group at location ( xCG, yCG )

When significant_coeff_group_flag[ xCG ][ yCG ] is not present, it is inferred as follows.

- If one of the following conditions is true, significant_coeff_group_flag[ xCG ][ yCG ] is inferred to be equal to 1.

- ( xCG, yCG ) is equal to ( LastSignificantCoeffX >> 2, LastSignificantCoeffY >> 2 )

- ( xCG, yCG ) is equal to ( 0, 0 )

- (log2TrafoWidth is greater than 3 and log2TrafoHeight is greater than 3) && ( xCG < ( 1 << (log2TrafoWidth - 2) ) − 1 ) && ( yCG < ( 1 << (log2TrafoHeight - 2) ) − 1 ) &&
significant_coeff_group_flag[ xCG+1 ][ yCG ] = = 1 && significant_coeff_group_flag[ xCG ][ yCG+1 ] = = 1

- Otherwise (none of the above conditions is true), significant_coeff_group_flag[ xCG ][ yCG ] is inferred to be equal to 0.

significant_coeff_flag[ xC ][ yC ] specifies for the transform coefficient position ( xC, yC ) within the current transform block whether the corresponding transform coefficient level at location ( xC, yC ) is non-zero as follows.

- If significant_coeff_flag[ xC ][ yC ] is equal to 0, the transform coefficient level at location ( xC, yC ) is set equal to 0.

- Otherwise (significant_coeff_flag[ xC ][ yC ] is equal to 1), the transform coefficient level at location ( xC, yC ) has a non-zero value.

When significant_coeff_flag[ xC ][ yC ] is not present, it is inferred as follows.

- If ( xC, yC ) is the last significant location ( LastSignificantCoeffX, LastSignificantCoeffY ) in scan order or all of the following conditions are true, significant_coeff_flag[ xC ][ yC ] is inferred to be equal to 1

- ( xC, yC ) is equal to ( xCG << 2, yCG << 2 )

- rightCGFlag + bottomCGFlag < 2

- (xCG, yCG) is not equal to (0, 0)

- numNZInCG is equal to 0

- Otherwise( xC, yC ) is not the last significant location ( LastSignificantCoeffX, LastSignificantCoeffY ) in scan order), significant_coeff_flag[ xC ][ yC ] is inferred to be equal to 0.