| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, CA, USA, Feb. 2012 | Document: JCTVC-H0482 WG11 Number: m23360 |
| --- | --- |

| Title: | AHG21: Improvement to Intra RPS Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao, Sachin Deshpande and Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Email: | Tel: +1-360-817-8487 asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

This contribution proposes a method to reduce the bits required to code RPS by intra RPS prediction. The proposed method exploits symmetry between the first positive delta POC in an RPS and the largest,  negative delta POC. Furthermore, the proposal reports that the method can be combined with other RPS intra prediction techniques and provide further bit reduction.  Combined with the POC granularity technique in JCTVC-G643, achieves a bit-rate reduction of 48 bits (20.7%) compared to 232 bits for the existing intra RPS method when inter RPS is off.  This is for random access conditions with a GOP size of eight.  Alone, the symmetry approach achieves a bit-rate reduction of 14 bits (6%) for the same simulation.  It is asserted that the method is most beneficial for applications where slice level transmission of the RPS is used frequently

# Introduction

# RPS (Reference Picture Set) concept describes the reference pictures in the decoded picture buffer (DPB) in an absolute manner. It has been adopted into JCTVC-WD5. An RPS contains a list of delta POC information of all reference pictures that the decoder shall keep. A set of RPS template are signaled in PPS, and referred by slice header using an RPS index. RPS can also be signaled explicitly in slice header.

# Several approaches have been proposed to reduce the bits used to signal RPS set. JCTVC-G198 proposed inter RPS prediction, which has been adopted into HM5. JCTVC-G643 proposed several methods including a POC granularity method.

# Table 1 below shows the RPS set used by common test condition Random Access configuration. It is a typical Hierarchical B coding structure with GOP size 8. So there are 8 full RPS. 3 partial RPS are generated for pictures which have available reference pictures due to IDR or CRA pictures.

# Table 1.0 RPS for Random Access configuration in HM5.0 common test condition

# Table 1.0    RPS for RA configuration in HM5.0 common test condition

| - RPS # | - Type | - POC | - Temporal_id | - Ref_buf_size | - ref_pic | - #ref_pics | - Reference Pictures DeltaPOC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| - 1: | - B | - 8 | - 0 | - 4 | - 1 | - 4 | - -8 -10 -12 -16 |
| - 2: | - B | - 4 | - 0 | - 2 | - 1 | - 3 | - -4 -6 4 |
| - 3: | - B | - 2 | - 0 | - 2 | - 1 | - 4 | - -2 -4 2 6 |
| - 4: | - B | - 1 | - 0 | - 2 | - 0 | - 4 | - -1 1 3 7 |
| - 5: | - B | - 3 | - 0 | - 2 | - 0 | - 4 | - -1 -3 1 5 |
| - 6: | - B | - 6 | - 0 | - 2 | - 1 | - 4 | - -2 -4 -6 2 |
| - 7: | - B | - 5 | - 0 | - 2 | - 0 | - 4 | - -1 -5 1 3 |
| - 8: | - B | - 7 | - 0 | - 2 | - 0 | - 4 | - -1 -3 -7 1 |

Table 1.1 Partial RPS for RA configuration in HM5.0 common test condition

| - 9: | - B | - 8 | - 0 | - 4 | - 1 | - 1 | - -8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| - 10: | - B | - 4 | - 0 | - 2 | - 1 | - 2 | - -4 4 |
| - 11: | - B | - 2 | - 0 | - 2 | - 1 | - 3 | - -2 2 6 |

When Inter RPS is off, delta POC within an RPS are differentially coded. It can be understood from the syntax table and semantics in working draft below.  For example, for an RPS with reference indices [-2,-4,2,6], it differentially codes the negative RPS picture indices first, and then differentially code the positive picture indices. For the first delta POC of both negative and positive values, the previous index is set to 0.

The related RPS syntax in latest working draft JCTVC-G1103-d7 is:

| short_term_ref_pic_set( idx ) { | Descriptor |
| --- | --- |
| inter_ref_pic_set_prediction_flag | u(1) |
| if( inter_ref_pic_set_prediction_flag ) { |  |
| delta_idx_minus1 | ue(v) |
| delta_rps_sign | u(1) |
| abs_delta_rps_minus1 | ue(v) |
| for( j = 0; j <= NumDeltaPocs[ RIdx ]; j++ ) { |  |
| ref_idc0[ j ] | u(1) |
| if( !ref_idc0[ j ] ) |  |
| ref_idc1[ j ] | u(1) |
| } |  |
| } |  |
| else { |  |
| num_negative_pics | ue(v) |
| num_positive_pics | ue(v) |
| for( i = 0; i < num_negative_pics; i++ ) { |  |
| delta_poc_s0_minus1[ i ] | ue(v) |
| used_by_curr_pic_s0_flag[ i ] | u(1) |
| } |  |
| for( i = 0; i < num_positive_pics; i++ ) { |  |
| delta_poc_s1_minus1[ i ] | ue(v) |
| used_by_curr_pic_s1_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |

delta_poc_s0_minus1[ i ] plus 1 specifies an absolute difference between two picture order count values. The value of delta_poc_s0_minus1[ i ] shall be in the range of 0 to 215 – 1, inclusive.

The variable DeltaPocS0[ idx ][ i ] is derived as follows.

if (i  = =  0) 
    DeltaPocS0[ idx ][ i ] = – ( delta_poc_s0_minus1[ i ] + 1 ) 	(7-1)
else
    DeltaPocS0[ idx ][ i ] = DeltaPocS0[ idx ][ i – 1 ] – ( delta_poc_s0_minus1[ i ] + 1 )

delta_poc_s1_minus1[ i ] plus 1 specifies an absolute difference between two picture order count values. The value of delta_poc_s1_minus1[ i ] shall be in the range of 0 to 215 – 1, inclusive.

The variable DeltaPocS1[ idx ][ i ] is derived as follows.

if (i  = =  0) 
    DeltaPocS1[ idx ][ i ] = delta_poc_s1_minus1[ i ] + 1	(7-2)
else
    DeltaPocS1[ idx ][ i ] = DeltaPocS1[ idx ][ i – 1 ] + ( delta_poc_s1_minus1[ i ] + 1 )

deltaPocS0 contains the negative delta POC and deltaPocS1 contains the positive delta POC.

# Symmetric Coding the Positive RPS

For random access (Hierarchal B) coding, there may be both negative and positive delta POC in an RPS. The smallest absolute value of the negative delta POC and the smallest positive delta POC are usually the same. This is because nearest backward and forward reference pictures typically have the same distance to the current picture due to symmetric feature in Hierarchical B coding structure. Therefore, for coding the first positive delta POC value, instead of coding itself directly, we propose to send a flag to indicate if the first positive delta POC value is equal to the smallest absolute value of the negative delta POC. If this flag is true, then there is not need to read the first positive delta POC, it can be simply initialized to the smallest absolute value of negative delta POC.

In JCTVC WD5, when inter RPS is on, it specifies that DeltaPocS0[ idx ][ i0 ] and DeltaPocS1[ idx ][ i1 ] shall be sorted in decreasing order and in increasing order, respectively. However, it does not clearly specify such restriction when inter RPS is off.  We believe there should be such restriction on DeltaPocS0 and DeltaPocS1 when inter RPS is off as well. This ensures that the difference to the previous delta POC is not negative. It seems that current working draft and software assumes deltaPOCS0 and deltaPocS1 are in such an order and use ue(v) to code the differences. However, HM5.0 does not result in an error if this is not the case. For example, coding RPS of {-8, -10, -12, -16} by HM 5.0 uses 28 bits, but coding RPS of { -16, -12, -10, -8} will use 208 bits. In this case, the delta POC difference is negative, it costs a lot more bits to code a negative value which is cast into an unsigned value.

With such restriction, RPS deltaPocS0[0] would be the smallest absolute value within negative delta POC. deltaPocS1[0] would be the smallest positive delta POC if exist. Therefore, an encoder can easily determine the symmetric flag. Below are the proposed syntax changes. It is applied to the branch where inter RPS prediction is off.

| short_term_ref_pic_set( idx ) { | Descriptor |
| --- | --- |
| inter_ref_pic_set_prediction_flag | u(1) |
| if( inter_ref_pic_set_prediction_flag ) { |  |
| ……. |  |
| } |  |
| else { |  |
| num_negative_pics | ue(v) |
| num_positive_pics | ue(v) |
| if (num_positive_pics) { |  |
| symmetric_flag | u(1) |
| } |  |
| for( i = 0; i < num_negative_pics; i++ ) { |  |
| delta_poc_s0_minus1[ i ] | ue(v) |
| used_by_curr_pic_s0_flag[ i ] | u(1) |
| } |  |
| for( i = 0; i < num_positive_pics; i++ ) { |  |
| if (i!=0 ¦¦ ! symmetric_flag) { |  |
| delta_poc_s1_minus1[ i ] | ue(v) |
| } |  |
| used_by_curr_pic_s1_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |

The only change to decoder is:

symmetric_flag: specifies whether the smallest positive delta POC is the same as the smallest absolute value of negative delta POC. When not present, symmetric_flag is default to 0.

The variable DeltaPocS1[ idx ][ i ] is derived as follows.

if (i== 0) {

if (symmetric_flag)

DeltaPocS1[ idx ][ i ] = -DeltaPocS0[ idx ][ i ]

else
    DeltaPocS1[ idx ][ i ] = delta_poc_s1_minus1[ i ] + 1	(7-3)

}
else
   DeltaPocS1[ idx ][ i ] = DeltaPocS1[ idx ][ i – 1 ] + ( delta_poc_s1_minus1[ i ] + 1 )

# Combined with POC Granularity

# JCTVC-G643 proposed a method of POC granularity. POC granularity is signaled for each temporal layer, and it is used to quantize the POC differences indicated in the reference picture list description syntax structures to improve compression efficiency. Similarly, we implemented this POC granularity as a shift since for dyadic coding structure the POC granularity would be usually order of 2.

Basically, a POCScale is signaled in the bitstream. When differentially coding the delta POC, following changes are made to the semantics in section 1.

when i != 0

DeltaPocS0[ idx ][ i ] = DeltaPocS0[ idx ][ i – 1 ] – ( delta_poc_s0_minus1[ i ] + 1 )

is replaced by

DeltaPocS0[ idx ][ i ] = DeltaPocS0[ idx ][ i – 1 ] – (( delta_poc_s0_minus1[ i ] + 1 )<<deltaPOCScale)

and,

DeltaPocS1[ idx ][ i ] = DeltaPocS1[ idx ][ i – 1 ] + ( delta_poc_s1_minus1[ i ] + 1 )

is replaced by

DeltaPocS1[ idx ][ i ] = DeltaPocS1[ idx ][ i – 1 ] + (( delta_poc_s1_minus1[ i ] + 1 )<<deltaPOCScale)

At encoder, the process is reversed and the difference is shifted down.  The positive delta POC symmetric init technique can be combined with this POC granularity technique without conflict since symmetric init only applies to DeltaPocS1 and i == 0.

# Results

We tested the proposed positive delta POC symmetric init method using HM-5.0-dev-misc-ahg21 with random access configuration and with inter RPS off. The symmetric init for positive delta POC does not affect RPS coding in low delay configuration since there are no positive delta POC in low delay.

We also tested combining it with JCTVC-G643 Delta POC granularity. Results are shown in tables below. The tested configurations used are

- HM5 RA common test condition of GOP size 8.

- RA with typical Hierarchical B of GOP size16.

- Common test condition in G1036.

Table 3 Result of Symmetric Init of Positive Delta POC and

Combined result with Delta POC Granularity

Table 3.1 Result of Proposed Method for RA with GOP 8 in HM5 Common Test Condition

| RPS index | RPS Delta POC | HM5 Bits Usage (InterRPS=0) | proposed Symmetric Init | Bit Savings | Symmetric Init and POC Scale | Bit Savings* |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | { -8* -10* -12* -16* } | 28 | 28 | 0 | 22 | -6 |
| 2 | { -4* -6* 4* } | 22 | 18 | -4 | 16 | -6 |
| 3 | { -2* -4* 2* 6* } | 24 | 22 | -2 | 18 | -6 |
| 4 | { -1* 1* 3* 7* } | 22 | 22 | 0 | 18 | -4 |
| 5 | { -1* -3* 1* 5* } | 20 | 20 | 0 | 16 | -4 |
| 6 | { -2* -4* -6* 2* } | 24 | 22 | -2 | 18 | -6 |
| 7 | { -1* -5* 1* 3* } | 20 | 20 | 0 | 16 | -4 |
| 8 | { -1* -3* -7* 1* } | 22 | 22 | 0 | 18 | -4 |
| 9 | { -8* } | 12 | 12 | 0 | 12 | 0 |
| 10 | { -4* 4* } | 18 | 14 | -4 | 14 | -4 |
| 11 | { -2* 2* 6* } | 20 | 18 | -2 | 16 | -4 |
| Total |  | 232 | 218 | -14 | 184 | -48 |
| Saving Percent |  |  |  | 6% |  | 20.7% |

Note *: 3 bit is sent out side of the RPS to signal deltaPOCScale=1.

Table 3.2 Result of Proposed Method for RA with HierB GOP 16

| RPS index | RPS Delta POC | HM5 Bits Usage (InterRPS=0) | Symmetric Init | Bit Savings | Symmetric Init and POC scale | Bit Savings* |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | { -16* -18* -20* } | 24 | 24 | 0 | 20 | -4 |
| 2 | { -8* -10* 8* } | 26 | 20 | -6 | 18 | -8 |
| 3 | { -4* -6* 4* 12* } | 30 | 26 | -4 | 22 | -8 |
| 4 | { -2* -4* 2* 6* 14* } | 34 | 32 | -2 | 26 | -8 |
| 5 | { -1* -3* 1* 3* 7* 15*} | 34 | 34 | 0 | 26 | -8 |
| 6 | { -1* -3* 1* 5* 13* } | 30 | 30 | 0 | 24 | -6 |
| 7 | { -2* -4* 2* 10* } | 26 | 24 | -2 | 20 | -6 |
| 8 | { -1* -3* 1* 3* 11* } | 28 | 28 | 0 | 22 | -6 |
| 9 | { -1* -3* 1* 9* } | 22 | 22 | 0 | 18 | -4 |
| 10 | { -4* -6* -8* 4* } | 28 | 24 | -4 | 20 | -8 |
| 11 | { -2* -4* 2* 6* } | 24 | 22 | -2 | 18 | -6 |
| 12 | { -1* -3* 1* 3* 7* } | 26 | 26 | 0 | 20 | -6 |
| 13 | { -1* -3* 1* 5* } | 20 | 20 | 0 | 16 | -4 |
| 14 | { -2* -4* 2* } | 18 | 16 | -2 | 14 | -4 |
| 15 | { -1* -3* 1* 3* } | 18 | 18 | 0 | 14 | -4 |
| 16 | { -1* -3* 1* } | 14 | 14 | 0 | 12 | -2 |
| 17 | { -16* } | 14 | 14 | 0 | 14 | 0 |
| 18 | { -8* 8* } | 22 | 16 | -6 | 16 | -6 |
| 19 | { -4* 4* 12* } | 26 | 22 | -4 | 20 | -6 |
| 20 | { -2* 2* 6* 14* } | 30 | 28 | -2 | 24 | -6 |
| 21 | { -1* 1* 3* 7* 15* } | 30 | 30 | 0 | 24 | -6 |
| Total |  | 524 | 490 | -34 | 20 | -116 |
| Saving Percent |  |  |  | 6.5% |  | 22.1% |

Table 3.3 Result of Proposed method for RA Common Test Conditions in G1036

| G1036 Test case | HM-5.0-dev-misc-ahg21 (InterRPS=0) | With Symmetric Init | Bit Savings |  | Symmetric Init + POC scale | Bit Savings* |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RA_2_1_T1_32 | 232 | 218 | -14 | 6% | 184 | -48 | 20.7% |
| RA_2_2_T1_32 | 126 | 118 | -8 | 6.3% | 108 | -18 | 14.3% |
| RA_2_2_T2_32 | 142 | 134 | -8 | 5.6% | 116 | -26 | 18.3% |
| RA_2_3_T3_32 | 146 | 138 | -8 | 5.5% | 120 | -26 | 17.8% |
| RA_2_3_T4_32 | 156 | 148 | -8 | 5.1% | 124 | -32 | 20.5% |
| RA_2_4_T5_64 | 712 | 688 | -24 | 3.4% | 574 | -138 | 19.4% |
| RA_2_5_256 | 2064 | 1950 | -114 | 5.5% | 1692 | -372 | 18% |
| RA_2_7_32 | 278 | 262 | -16 | 5.8% | 214 | -64 | 23% |

These intra RPS techniques reduce bits to code RPS when inter RPS is off. It is complementary to inter RPS technique.  Intra RPS prediction technique is applicable to the first RPS, no matter if inter-RPS prediction is used.  In addition, the technique is most advantageous for scenarios that use intra-RPS exclusively – such as encoders that only signal RPS information in the slice header.

# Conclusion

We propose an intra-RPS prediction technique based on symmetry between the first positive and negative delta POC values.  The method reduces the bit-rate for RPS in random access coding configurations when RPS is signaled without inter prediction.  With this simple change, it saves about 6% RPSs bits for random access configurations. Moreover, the technique also works with other intra RPS prediction such as the delta POC granularity to further improves the intra RPS prediction.

We propose the JCT-VC adopt this change into HM.

# Patent rights declaration(s)

SHARP Labs of America may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- JCTVC-G1103, “High Efficiency Video Coding (HEVC) text specification Working Draft 5,” 7th JCT-VC meeting, Geneva, Nov. 2011.

- JCTVC-G021, “JCT-VC AHG report: Reference picture buffering and list construction (AHG21),” 7th JCT-VC Meeting, Geneva, Nov. 2011.

- JCTVC-G198, “AHG21: Inter reference picture set prediction syntax and semantics,” 7th JCT-VC Meeting, Geneva, Nov. 2011.

- JCTVC-G643, “AHG21: On reference picture list construction and reference picture marking,” 7th JCT-VC Meeting, Geneva, Nov. 2011.

- HM5.0 and HM-5.0-dev-misc-ahg21, http://hevc.kw.bbc.co.uk/svn/jctvc-hm/

- JCTVC-G1200, “Common test conditions and software reference configurations,” 7th JCT-VC Meeting, Geneva, Nov. 2011.

- JCTVC-G1036, “Common conditions for reference picture marking and list construction proposals,” 7th JCT-VC Meeting, Geneva, Nov. 2011.