#### Short-term reference picture set syntax

| short_term_ref_pic_set( idx ) { | Descriptor |
| --- | --- |
| inter_ref_pic_set_prediction_flag | u(1) |
| if( inter_ref_pic_set_prediction_flag ) { |  |
| delta_idx_minus1 |  |
| delta_rps_sign |  |
| abs_delta_rps_minus1 |  |
| for( j = 0; j <= NumDeltaPocs[ RIdx ]; j++ ) { |  |
| ref_idc0[ j ] |  |
| if( !ref_idc0[ j ] ) |  |
| ref_idc1[ j ] |  |
| } |  |
| } |  |
| else { |  |
| num_negative_pics | ue(v) |
| num_positive_pics | ue(v) |
| if (num_positive_pics) { |  |
| symmetric_flag | u(1) |
| } |  |
| for( i = 0; i < num_negative_pics; i++ ) { |  |
| delta_poc_s0_minus1[ i ] | ue(v) |
| used_by_curr_pic_s0_flag[ i ] | u(1) |
| } |  |
| for( i = 0; i < num_positive_pics; i++ ) { |  |
| if (i!=0 ¦¦ ! symmetric_flag) { |  |
| delta_poc_s1_minus1[ i ] | ue(v) |
| } |  |
| used_by_curr_pic_s1_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |

symmetric_flag: specifies whether the smallest positive delta POC is the same as the smallest absolute value of negative delta POC. When not present, symmetric_flag is default to 0.

#### 7.4.3.1 Short-term reference picture set semantics

DeltaPocS0[ idx ][ i0 ] and DeltaPocS1[ idx ][ i1 ] shall be sorted in decreasing order and in increasing order, respectively.

(The working draft should make it clear that the above restriction  applies regardless of inter_ref_pic_set_prediction_flag )

delta_poc_s1_minus1[ i ] plus 1 specifies an absolute difference between two picture order count values. The value of delta_poc_s1_minus1[ i ] shall be in the range of 0 to 215 – 1, inclusive.

The variable DeltaPocS1[ idx ][ i ] is derived as follows.

if (i== 0) {

if (symmetric_flag)

DeltaPocS1[ idx ][ i ] = -DeltaPocS0[ idx ][ i ]

else
    DeltaPocS1[ idx ][ i ] = delta_poc_s1_minus1[ i ] + 1	(7-1)

}
else
   DeltaPocS1[ idx ][ i ] = DeltaPocS1[ idx ][ i – 1 ] + ( delta_poc_s1_minus1[ i ] + 1 )