| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0485 |
| --- | --- |

| Title: | Picture size signaling and cropping operation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karsten Suehring, Benjamin Bross Fraunhofer HHI | Email: | Karsten.Suehring@hhi.fraunhofer.de Benjamin.Bross@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes a modified picture size coding scheme based on coding units and a cropping operation. Bit rate saving are reported for typically used picture sizes. The contribution also analyzes memory requirements for cropping operations inside and outside the coding loop.

# Introduction

The SPS syntax in WD5 codes picture width and height in units of luma samples (syntax elements pic_width_in_luma_samples and pic_height_in_luma_samples). On the other hand the smallest block structure that can be coded is restricted by log2_min_coding_block_size_minus3. The minimum possible value is 8x8 samples. This implies that the actual coded picture is padded to multiples of the minimum coding block size.

# Proposed syntax

We propose to signal the picture size in units of the minimum coding block size similar to the macroblock unit size in H.264/AVC. Also similar to the H.264/AVC syntax we propose a cropping rectangle to be applied to the decoded picture, either without left and top borders:

| pic_width_in_luma_samples | ue(v) |
| --- | --- |
| pic_height_in_luma_samples | ue(v) |
| pic_width_in_min_coding_blocks | ue(v) |
| pic_height_in_min_coding_blocks | ue(v) |
| frame_cropping_flag | u(1) |
| if( frame_cropping_flag ) { |  |
| frame_crop_right_offset | ue(v) |
| frame_crop_bottom_offset | ue(v) |
| } |  |

or with the complete H.264/AVC cropping syntax:

| pic_width_in_luma_samples | ue(v) |
| --- | --- |
| pic_height_in_luma_samples | ue(v) |
| pic_width_in_min_coding_blocks | ue(v) |
| pic_height_in_min_coding_blocks | ue(v) |
| frame_cropping_flag | u(1) |
| if( frame_cropping_flag ) { |  |
| frame_crop_left_offset | ue(v) |
| frame_crop_right_offset | ue(v) |
| frame_crop_top_offset | ue(v) |
| frame_crop_bottom_offset | ue(v) |
| } |  |

# Coding results for common image sizes

The following table shows the bit counts for the changed syntax elements. Savings are colored green, losses red.

Syntax without top an left:

| image size | WD5 | proposed SCU=8x8 | proposed SCU=16x16 | proposed SCU=32x32 | proposed SCU=64x64 |
| --- | --- | --- | --- | --- | --- |
| 176x144 | 32 | 23 | 19 | 31 | 33 |
| 320x240 | 34 | 25 | 21 | 27 | 23 |
| 352x288 | 36 | 27 | 23 | 19 | 35 |
| 416x240 | 36 | 27 | 23 | 27 | 33 |
| 640x480 | 38 | 29 | 25 | 21 | 29 |
| 720x576 | 40 | 31 | 27 | 31 | 31 |
| 832x480 | 40 | 31 | 27 | 23 | 29 |
| 1280x720 | 42 | 33 | 29 | 33 | 33 |
| 1920x1080 | 44 | 35 | 37 | 33 | 29 |
| 2560x1600 | 46 | 37 | 33 | 29 | 25 |

Syntax with top and left borders (being equal to zero)

| image size | WD5 | proposed SCU=8x8 | proposed SCU=16x16 | proposed SCU=32x32 | proposed SCU=64x64 |
| --- | --- | --- | --- | --- | --- |
| 176x144 | 32 | 25 | 21 | 33 | 35 |
| 320x240 | 34 | 27 | 23 | 29 | 25 |
| 352x288 | 36 | 29 | 25 | 21 | 37 |
| 416x240 | 36 | 29 | 25 | 29 | 35 |
| 640x480 | 38 | 31 | 27 | 23 | 31 |
| 720x576 | 40 | 33 | 29 | 33 | 33 |
| 832x480 | 40 | 33 | 29 | 25 | 31 |
| 1280x720 | 42 | 35 | 31 | 35 | 35 |
| 1920x1080 | 44 | 37 | 39 | 35 | 31 |
| 2560x1600 | 46 | 39 | 35 | 31 | 27 |

Except for the unusual combination of very small picture size and very large minimum coding block sizes, the proposed scheme always show gain compared to the current WD.

# Analysis of cropping operation

Due to the limit of smallest coding blocks, in some cases padding of the picture up to the next possible coded picture size will be necessary. In a JCT-VC email reflector discussion related to AHG15 it has been pointed out, that the necessary padding for a selected minimum coding block size of 64x64 will lead to increased memory requirements which could be avoided by cropping inside the coding loop.

The following table contains an analysis of the added picture size in luma pels:

|  | luma pels | SCU=8x8 |  | SCU=16x16 |  | SCU=32x32 |  | SCU=64x64 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | luma pels |  | luma pels |  | luma pels |  | luma pels |  |
| 176x144 | 25344 | 0 | 0.00% | 0 | 0.00% | 3072 | 12.12% | 9,216 | 36.36% |
| 320x240 | 76800 | 0 | 0.00% | 0 | 0.00% | 5120 | 6.67% | 5,120 | 6.67% |
| 352x288 | 101376 | 0 | 0.00% | 0 | 0.00% | 0 | 0.00% | 12,288 | 12.12% |
| 416x240 | 99840 | 0 | 0.00% | 0 | 0.00% | 6656 | 6.67% | 7,168 | 7.18% |
| 640x480 | 307200 | 0 | 0.00% | 0 | 0.00% | 0 | 0.00% | 20,480 | 6.67% |
| 720x576 | 414720 | 0 | 0.00% | 0 | 0.00% | 0 | 0.00% | 0 | 0.00% |
| 832x480 | 399360 | 0 | 0.00% | 0 | 0.00% | 0 | 0.00% | 26,624 | 6.67% |
| 1280x720 | 921600 | 0 | 0.00% | 0 | 0.00% | 20,480 | 2.22% | 61,440 | 6.67% |
| 1920x1080 | 2073600 | 0 | 0.00% | 15,360 | 0.74% | 15,360 | 0.74% | 15,360 | 0.74% |
| 2560x1600 | 4096000 | 0 | 0.00% | 0 | 0.00% | 0 | 0.00% | 0 | 0.00% |

(range 0% to 5% green, 5% to 10% yellow; above red )

As the table shows only the combination of very large minimum coding block size and very small image size shows a large relative increase in memory requirement for decoded samples. Looking at the absolute numbers, these are still not very large.

On the other hand an in-loop cropping operation will most likely require an additional copy operation for storing all samples of the cropped picture into a reference buffer that is aligned to the cropped picture size.

Based on these numbers we propose to apply the cropping outside the coding loop at the output. This includes padding operations for motion estimation to be applied at the borders of the actual coded picture size.

# Patent rights declaration(s)

Fraunhofer HHI does not have any current or pending patent rights relating to the technology described in this contribution.