| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, 1-10 February, 2012 | Document: JCTVC-H0488 |
| --- | --- |

| Title: | Cross-check of Samsung’s H0131: Simplified Multi-Level Significance Map Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Nguyen Nguyen, Tianying Ji, Dake He and Gaëlle Martin-Cocher 185 Columbia St. W. Waterloo, ON, Canada N2L 5Z5 | Tel: Email: | +1-519-8887465 nnguyen@rim.com tiji@rim.com dhe@rim.com gmartincocher@rim.com |
| Source: | Research In Motion Ltd. |  |  |

_____________________________

# Abstract

This contribution presents the results of the cross-check performed by RIM on Samsung’s proposal, JCTVC-H0131: Simplified Multi-Level Significance Map Coding [1].  The implementation provided by the proponent matches the text in their proposal.  The BD-rate, encoding and decoding times, and bin counts obtained in the cross-check also match those provided by the proponent.

# Introduction

JCTVC-H0131 [1] proposes removing the check which infers the significant_coeffgroup_flag of a coefficient group to be 1 if both its right and lower neighbours have significant_coeffgroup_flag = 1.  This increases the number of significant_coeffgroup_flags, but decreases the number significant_coeff_flags.  Under common test conditions, the net effect is an increase in the number of coded bins for significance map for AI-HE, but a decrease in other test configurations, with minimal impact on coding efficiency.  Against HM-5.0 anchor, the difference in the number of coded bins for significance map is between -0.7% and -1.0%, except for AI-HE, in which there is a difference of +0.1%.

# Results

Using the source code provided by Samsung, full encoding and decoding tests were performed.  The BD-rate, timing results and bin counts of the cross-check match the results provided by Samsung.  A review of the source code showed that the implementation matches the text description in JCTVC-H0131 [1].

# References

- J.Min, Y.Piao, and J.H. Park, “Simplified Multi-Level Significance Map Coding”, JCT-VC of ITU-T  SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 8th Meeting, San Jose, February 2012.

- N. Nguyen, T. Ji, D. He, and G. Martin-Cocher, “Multiple Level Significance Maps for Large Transform Units”, ”, JCT-VC of ITU-T  SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 7th JCT-VC Meeting, Geneva, November 2011.